3GPP RAN WG4 Meeting #115	R4-2505983

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title: 	Discussion on general aspects for AIML for NR air

Source: 	CATT

Agenda item:	7.22.2

Document for:	Discussion

Introduction

In RAN4#114bis meeting, RAN4 continued discussing the general aspects of Rel-19 AI/ML for NR air interface and the agreements were captured in [1]. Some open issues summarized in [2] need further discussions and we would like to present our views on some of the open issues in this contribution.

Discussion

Post deployment testing

In RAN4#112bis meeting, a concrete summary captured the issues that are considered for post deployment testing. In brief, the following two issues are the targets for post deployment testing:

•	Issue 1: Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance.

•	Issue 2: Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance.

To resolve these issues, two options are proposed:

•	Option 1 [Pre-activation functionality/model update testing]: Conduct the validation of a change in AI functionality before its deployment/activation in already deployed UEs (in the field or in lab conditions).

•	Option 2 [Post deployment management based on LCM]: Using performance monitoring and LCM procedures.

In RAN4#114, RAN4 discussed both options but no agreements were achieved due to different understandings. To help discuss this issue efficiently, a table that capture the pros and cons of both options was provided in [1] with some issues that should be answered by proponents. We support option 2 and the reasons are presented below.

Issue 1 can be divided into different scenarios according to the types of changes. For the changes referring to update or fine tuning, conducting the validation in lab conditions cannot ensure a consistent performance between field and post deployment testing, since the parameters are updated or fine-tuned are highly scenario-dependent. Even the updated functionality passes the post deployment testing, its performance in the field cannot be guaranteed. If the validation is conducted in the field, from our perspective, it is the same as the LCM procedures, i.e., performance monitoring.

Observation 1: For changes due to update or fine tuning of models,

- Validation in lab conditions cannot ensure a consistent performance between field and post deployment testing, because the updates or fine tuning are scenario-dependent.

- Validation in the field is the same as the performance monitoring procedure.

For the changes referring to addition/removal of models, the discussion needs to be based on RAN2 agreements on the definitions of functionality and procedures. The related RAN2 agreements are reproduced below [3]:

RAN2#127: Agreements on definitions 1 Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) 2 Applicable functionalities refers to functionalities that the UE is ready to apply for inference 3 Activated functionalities refers to functionalities already enabled for performing inference Agreements on procedures - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

As highlighted contents show, after addition/removal of models is completed and before corresponding functionalities can start to perform inference, UE needs to decide the applicability of functionalities. Therefore whether a functionality impacted by addition/removal of models can be activated or not can rely on the applicability checking procedure. At least at current stage, we do not see necessary validations of functionalities impacted by changes after UE is deployed.

Observation 2: For changes due to addition/removal of models, applicability checking procedure of functionality can handle the potential impacts on functionality performance.

For issue2, the impacts mainly come from data drift / mismatch, resulting from inconsistent scenarios between field and the training phase. In our understanding, the purpose of the post deployment testing for this issue is trying to identify the reasons why AI/ML functionality performs badly in the field. Firstly, it is already beyond the purpose of RAN4 performance testing which tests AI/ML capability and its basic performance. Secondly, even such post deployment testing is carried out and the reason is found out, what UE can do is, for example, to switch functionality or fallback to non-AI functions, which is also a part of LCM procedures to us.

Regarding Option 1, RAN4 can further discuss it during meeting after proponents clarify their motivations and the feasibility is proved.

Observation 3: For impacts due to data drift/mismatch, the post deployment testing is actually trying to identify the reasons why AI/ML functionality performs badly in the field, which is beyond the scope of RAN4 performance testing and belongs to management of AI/ML functionality.

Proposal 1: Impacts due to changes of models and data drift / mismatch be handled by at least the following procedures:

- Performance monitoring and LCM procedures

- applicability checking procedure of functionality

Proposal 2: RAN4 discuss Option 1 after proponents clarify their motivations and its feasibility is proved.

Number of requirements/tests

In last meeting, the related proposals for this issue were summarized and some open issues need more discussion [2]:

A number of contributions have considered in general terms the number of requirements to define, and the granularity of the requirements. For the purpose of discussion, the term “requirement set” is used here: A requirement set is a group of requirements. If a UE should comply with a requirement set then it should meet all requirements in the requirement set. It may be possible to declare or signal which requirement sets a UE meets. Issue 1-1-1 discusses whether a requirement set is linked to a functionality/applicable condition. A requirement set may comprise one or more individual requirements. Different requirements in a requirement set may be e.g. different channel conditions, Doppler, array conditions etc. Issue 1-1-2 considers what may determine the number of requirements in a requirement set. Issue 1-1-1: How many requirement sets to define - Proposals - Option 1: Define requirement sets for at least every feature and applicable subset (Apple proposal 1/12, Nokia proposal 1) - Option 1a: FFS how to relate subsets and applicable conditions depending on definition of applicable conditions (Apple proposal 2, 13) - Option 1b: A minimum set of configurations/IDs shall be mandatory (Nokia proposal 2) - Option 1c: Do not define requirements with a change of associated ID during testing (Nokia proposal 3) - Option 1d: Ensure consistency between applicable conditions signalling during training and inference (Apple proposal 10) - Option 2: At least a requirement set per functionality, TBC for subsets depending on conclusions on granularity (CMCC proposal 3, CATT proposal 4) - Recommended WF: Discuss how many of the following could be agreed: - There is at least one requirement set per AI functionality/feature. - There may be more than one requirement set per AI functionality/feature. If there is more than one requirement set, the requirement set applicable for the UE may depend on: - Option 1: Network signalling of associated ID. - The UE may indicate which applicable conditions it is capable of achieving. If the UE indicates capability, it shall meet the corresponding requirement set - This option ensures compatibility between network signalling and which requirements a UE meets - Option 2: UE capability - One requirement set per UE capability - Some other, intra RAN4 definition of what requirement sets can exist per AI functionality - One or more of the requirement sets will be mandatory if the UE supports the AI functionality. - The requirement set will not change during an individual test Issue 1-1-2: How many requirements to define per requirement set - Proposals - Option 1: To determine how many requirements to define, a study is needed of how AI/ML behaviour changes with different conditions (Apple proposal 3, Nokia proposal 4) - Option 2: Different test configurations/parameters are used for tests, at least for generalization (CMCC proposal 2). - Option 3: It is necessary to ensure that the testing avoids the overfitting problem; if not other solutions may be needed (Korea test laboratory proposals 4 and 5) - Option 4: Create a mixed test-set for generalization testing, based on UE capability signaling configuration report (Apple proposals 7, 8) - Option 5: Scenarios not identified in the capability report are other scenarios for test (ZTE) - Recommended WF - Discuss whether it is agreeable to study how AI/ML model behaviour varies with different conditions (on a use-case basis) to decide how many requirements per requirement set. - Discuss which different conditions should be considered. Some examples: - Different fading channel profiles - Different Doppler - Different BS array assumptions - Different deployment scenario assumptions - Different BW configurations - … Take into account that in some cases, the different conditions may be a reason to define different requirement sets, not more than one requirement per requirement set. For example, if could be that each different deployment scenario is a different requirement set. (Or it could be that one requirement set covers several deployment scenarios…)

A new concept, requirement set, was introduced, and in our understanding the number of requirement sets required depends on functionality. And in each requirement set, there will be one or more than one requirement and the number depends on several conditions, such as fading channel profile, BW configuration, etc. Apparently, the granularity of AI/ ML functionality is the key point to this issue.

In RAN1#119 meeting, the following RAN2 terminologies on beam management were clarified by RAN1 [4]:

RAN1#119 Agreement: In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs - The concept/terminology “ functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same.

According to the above agreement, the granularity of different “functionalities” may be different and this has not been decided by RAN1 yet. Therefore we suggest discussing this issue after RAN1 achieve agreements on UE capability.

Observation 4: Number of requirement set(s) depends on the granularity of AI/ML functionality which may refer to UE capability.

Proposal 3: RAN4 postpone discussing the number of requirement set(s) after RAN1 achieve agreements on UE capability.

For the number of requirements in each requirement set, the methodology is similar to the legacy method, i.e., the conditions, e.g., fading channel profile, BW configuration, can be considered when defining performance requirements. For example, the number of gNB Tx beams and BW of reference signals probably need differentiating in BM use case. But considering this issue belongs to performance part, RAN4 can come back in performance phase and we will have more concrete information about UE capability and measurement configurations at that time.

Proposal 4: RAN4 to discuss the number of requirements per requirement set in performance phase, at which time RAN4 will have concrete information about UE capability and measurement configurations.

Conclusions

This paper discussed the general issues related to AI/ML for NR air interface, and following observations and proposals are provided:

Observation 1: For changes due to update or fine tuning of models,

- Validation in lab conditions cannot ensure a consistent performance between field and post deployment testing, because the updates or fine tuning are scenario-dependent.

- Validation in the field is the same as the performance monitoring procedure.

Observation 2: For changes due to addition/removal of models, applicability checking procedure of functionality can handle the potential impacts on functionality performance.

Observation 3: For impacts due to data drift/mismatch, the post deployment testing is actually trying to identify the reasons why AI/ML functionality performs badly in the field, which is beyond the scope of RAN4 performance testing and belongs to management of AI/ML functionality.

Proposal 1: Impacts due to changes of models and data drift / mismatch be handled by at least the following procedures:

- Performance monitoring and LCM procedures

- applicability checking procedure of functionality

Proposal 2: RAN4 discuss Option 1 after proponents clarify their motivations and its feasibility is proved.

Observation 4: Number of requirement set(s) depends on the granularity of AI/ML functionality which may refer to UE capability.

Proposal 3: RAN4 postpone discussing the number of requirement set(s) after RAN1 achieve agreements on UE capability.

Proposal 4: RAN4 to discuss the number of requirements per requirement set in performance phase, at which time RAN4 will have concrete information about UE capability and measurement configurations.

References

[1] R4-2505107, WF on AI/ML air interface study, Ericsson, RAN4#114bis.

[2] R4-2504685, Topic summary for [114bis][127] NR_AIML_air_part2, Ericsson , RAN4#114bis.

[3] R2_127_ChairNotes_24-08-23_17-00_eom, RAN2#127.

[4] Chair notes RAN1#119 eom1, RAN1#119.