3GPP RAN WG4 Meeting #115	R4-2505984

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title: 	Discussion on testing issues for CSI compression two-sided models

Source: 	CATT

Agenda item:	7.22.3

Document for:	Discussion

Introduction

In RAN4#114bis meeting, RAN4 discussed testability and interoperability issues for two-sided model in Rel-19 AI/ML for NR air interface and agreements were captured in [1]. But some issues need further investigating and discussing. In this contribution, we will continue to discuss some open issues and present our views [2].

Discussion

Option 3 track 2

For Option 3 track 2, Samsung decoder was chosen as the test decoder and we trained our own encoder with this chosen decoder and the mixed dataset. Afterwards, the own encoder was tested with the frozen test decoder using mixed dataset. The performance of encoder with lower complexity was also simulated. The initial results are provided in Figure 1:

Figure 1: SGCS of Option 3 track 2

Observation 1: In Option 3 track 2, SGCS is 0.713 for encoder with 6 ResNetBlocks and 0.7 for encoder with 2 ResNetBlocks.

Option 4

For Option 4, the WF [1] provided very detailed guidance for further simulation activity. And we have the following initial simulation results which are the SGCS performances of own encoder and own decoder trained and tested with the mixed dataset.

Figure 2: SGCS of Option 4 (CATT encoder-CATT decoder with mixed dataset used for training and testing)

Observation 2: In Option 4, SGCS of CATT encoder-CATT decoder is 0.6939 for encoder with 6 ResNetBlocks and 0.6814 for encoder with 2 ResNetBlocks.

What needs to be captured or specified for Option 3

RAN4 discussed what needs to be captured or specified for Option 3 in last meeting and reached the below agreements:

RAN4#114bis: Agreements - The encoder that corresponds to the standardized test decoder is stored within 3GPP - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS) - FFS whether several encoders would be captured (e.g. due to different complexity) For further discussion: The following topics should be further discussed: - For the encoder, the TBD and FFS above; i.e., where the encoder is stored/referred to and whether several encoders would be captured. - Whether it is agreeable to assume that the training dataset for the standardized test decoder is stored within 3GPP, if available - For example in case UE or network vendors could find access to the dataset used for the test encoder useful for their developments - Whether a test dataset is needed in the specification, or some other alternative (such as a channel model etc.)

The simulation results captured in our contribution [3] submitted to RAN4#114bis meeting shows that CNN encoders with different number of ResNet blocks have a slight different SGCS performance. And a larger SGCS performance difference can be expected if UE vendors develop an encoder with a backbone that is completely different from the test decoder. Therefore it would be beneficial to capture a reference encoder for guiding UE vendors’ development. Regarding the place to capture this reference encoder, we prefer to simply store the reference encoder in 3GPP database without any reference from TS or TR, considering that the initial idea of Option 3 is to standardize a test decoder only.

On the number of reference encoders that need to be captured in 3GPP database, since there are different choices for backbones and complexity, we think it will be helpful to capture plural reference encoders.

Proposal 1: The encoder that corresponds to the standardized test decoder is simply kept in a 3GPP database (with no reference from TR or TS).

Proposal 2: Plural reference encoders with different backbones/complexity can be captured in 3GPP database.

Another issue is whether to store the training dataset for the standardized test decoder within 3GPP. Based on the simulation results submitted by companies in last meeting, RAN4 have the following observations for Option 3 track 1 and track 2:

- Option 3 track 1 Endorsed preliminary observations - The investigation on option 3 is based on using the agreed structure for the decoder and encoder - It is possible for UE vendors to train an encoder based on another company’s frozen test decoder that is functional - The performance of the encoder is very dependent on the alignment of the dataset used for training the encoder and the dataset used for training the frozen decoder, and the testing dataset - The variation in the performance depending on the training datasets and test datasets can be as much as [20%] in SGCS. Further details are available in the results spreadsheet. - Preliminary observation; further results may be added/refined and the observation updated (for example, using the mixed dataset as in track 2) - For option 3, track 1 to be useful, careful attention would be needed to the alignment of datasets - Up to now, the same encoder structure is considered as used when creating the reference decoder; the impact of different encoder structure is not yet considered - Option 3 track 2 Endorsed preliminary observations - There is reasonable SGCS convergence when companies train an encoder-decoder pair using the mixed dataset and test using the mixed testset - Around 1% variation in SGCS; see results spreadsheet for more details - Preliminary observation; further results may be added - It is possible for UE vendors to train an encoder based on a frozen test decoder that is functional - When training “own” encoder with the frozen decoder and testing, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results from results presented so far - Around [+- 2%] variation in SGCS from the average; see results spreadsheet for more details - Preliminary observation; further results may be added - Up to now, the same encoder structure is considered as used when creating the reference decoder by most companies; the impact of different encoder structure is not yet fully considered

Based on the above observations, the performance of Option 3 is very dependent on the alignment of the dataset used for training the encoder and the dataset used for training the frozen decoder. Note that the datasets used to create the mixed dataset are also aligned. Therefore we think it is also beneficial to store the corresponding dataset within 3GPP database together with the reference encoder(s).

Proposal 3: RAN4 to store the dataset used for training the frozen decoder within 3GPP database together with the reference encoder(s).

What needs to be captured or specified for Option 4

RAN4 also discussed what needs to be captured or specified for Option 4 in last meeting and reached the following agreements:

Agreements: - Dataset is specified for option 4a. - FFS on dataset is captured for option 4b. - Assume reference decoder is captured. - FFS on reference encoder for option 4a - Test decoder structure is (i) fully specified or (ii) partially specified - NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase. For further discussion: The above FFS; i.e. - Whether a dataset is captured for option 4b - Whether a reference encoder is specified for option 4a

Regarding the two open parts, i.e., whether a dataset is captured for option 4b and whether a reference encoder is specified for option 4a, since RAN4 have not carried out the last step of the feasibility study of option 4a and 4b, which is to evaluate the performance between own encoder and the decoders trained by other companies. Hence, RAN4 need to confirm the necessity of a dataset for option 4b and a reference encoder for option 4a after RAN4 complete the feasibility study of option 4a and 4b.

Proposal 4: RAN4 to evaluate the performance between own encoder and the decoders trained by other companies and then check the necessity of a dataset for option 4b and a reference encoder for option 4a.

Conclusions

This contribution discussed some testability and interoperability issues for AI/ML enabled CSI compression and some observations and proposals are provided:

Observation 1: In Option 3 track 2, SGCS is 0.713 for encoder with 6 ResNetBlocks and 0.7 for encoder with 2 ResNetBlocks.

Observation 2: In Option 4, SGCS of CATT encoder-CATT decoder is 0.6939 for encoder with 6 ResNetBlocks and 0.6814 for encoder with 2 ResNetBlocks.

Proposal 1: The encoder that corresponds to the standardized test decoder is simply kept in a 3GPP database (with no reference from TR or TS).

Proposal 2: Plural reference encoders with different backbones/complexity can be captured in 3GPP database.

Proposal 3: RAN4 to store the dataset used for training the frozen decoder within 3GPP database together with the reference encoder(s).

Proposal 4: RAN4 to evaluate the performance between own encoder and the decoders trained by other companies and then check the necessity of a dataset for option 4b and a reference encoder for option 4a.

References

[1] R4-2505107, WF on AI/ML air interface study, Ericsson, RAN4#114bis.

[2] R4-250685, Topic summary for [114bis][127] NR_AIML_air_part2, Ericsson, RAN4#114bis.

[3] R4-2503410, Discussion on testing issues for CSI compression two-sided models, CATT, RAN4#114bis.