3GPP RAN WG4 Meeting #115	R4-2505986

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title: 	Discussion on impacts of AIML measurement event prediction on RRM requirements

Source: 	CATT

Agenda item:	7.23.3.2

Document for:	Discussion

Introduction

In RAN4#114bis meeting, RAN4 initially discussed the RRM impacts of AI/ML measurement event prediction and the agreements were captured in the WF [1]. In this contribution, we will further discuss the open issues in [2] and their potential impacts and present our views.

Discussion

Performance metric for event prediction use case

Measurement event prediction use case

According to RAN2 discussions, there are two types of measurement event prediction: direct and indirect prediction, differences of which are copied below [3]:

RAN2#127bis - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window.

Furthermore, RAN2 has following agreements in RAN2#128 meeting, which further categorizes indirect measurement event prediction into four cases [4]:

Agreements on event prediction 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model 2 The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed. 3 For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional.

Based on above RAN2 agreements, we will define measurement event prediction from direct and indirect prediction.

Direct measurement event prediction

For direct measurement event prediction, RAN2 has agreed that the input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Therefore, from RAN4 perspective, the corresponding achievements on model input discussion can be reused as well as test configurations.

Regarding the model output, since what model outputs is the probability of event occurrence within a time window, a candidate test metric is successful prediction rate, e.g., 90%.

Proposal 1: RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction, based on RAN2 agreement.

Indirect measurement event prediction

For indirect measurement event prediction, related issues were discussed and the following agreements are reached:

Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs

Firstly, RAN4 need to define delay requirement for predicted event triggered reporting, the non-AI conditional handover requirements can be used as a starting point, and RAN4 can further discuss the details in WI phase.

Secondly, regarding whether to define accuracy requirement, this is related to the second part in the above agreement and depends on the contents of reporting. If the report contains the final result of indirect measurement event prediction, i.e., probability of event occurrence within a time window, we do not think it is needed to define accuracy requirement. However, if the report includes the predicted RSRPs corresponding to the predicted event occurrence time, we think the accuracy requirement for RRM measurement prediction can be directly reused.

Proposal 2: RAN4 to define delay requirement for indirect case and discuss the details in WI phase.

Proposal 3: RAN4 to define accuracy requirement for indirect case and reuse the accuracy requirement for RRM measurement prediction, when the report includes the predicted RSRPs. No accuracy requirements need to be defined for each final result report.

On the metric for the indirect event prediction, there are two cases based on the contents of reports according to the discussion above:

Case 1: Report includes final results only, i.e., probability of event occurrence within a time window.

Case 2: Report includes final results and corresponding RSRPs.

For the reported RSRPs, as we discussed above, the accuracy requirement for RRM measurement prediction can be directly reused. For the final result which is the probability of event occurrence within a time window, the necessity of defining a performance requirement for each report needs justifying. But using successful prediction rate as a metric for monitoring the model performance over time duration may be more appropriate.

Proposal 4: Successful prediction rate can be used as a test metric for AI/ML model performance monitoring during a time period for both direct and indirect measurement event prediction.

Impact on core requirements for event prediction use case

Besides, measurement event prediction also impacts the conditional handover delay. In RAN2#129 meeting [5], RAN2 provides objectives for some measurement event predictions:

Objective in Case A (FR2): Predict A3 events to enhance HO execution via earlier HO Prep upon A3 event prediction (as shown in Option 2 or Option 3), i.e., HO performance enhancement. Objective in Case B (FR1): Predict A3 events in the presence of skipped measurements, i.e., overhead reduction while maintaining similar HO performance.

According to both objectives, some components in current conditional handover delay requirements can be updated or removed, e.g., Tprocessing and/or Tmeasure. Details can be further discussed in WI phase.

Proposal 5: With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure. FFS details in WI phase.

Conclusions

This contribution discussed the impacts of AI/ML measurement event prediction on RRM requirements, and some observations and proposals are provided:

Proposal 1: RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction, based on RAN2 agreement.

Proposal 2: RAN4 to define delay requirement for indirect case and discuss the details in WI phase.

Proposal 3: RAN4 to define accuracy requirement for indirect case and reuse the accuracy requirement for RRM measurement prediction, when the report includes the predicted RSRPs. No accuracy requirements need to be defined for each final result report.

Proposal 4: Successful prediction rate can be used as a test metric for AI/ML model performance monitoring during a time period for both direct and indirect measurement event prediction.

Proposal 5: With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure. FFS details in WI phase.

References

[1] R4-2504901, WF on AI/ML Mobility, Nokia, RAN4#114bis.

[2] R4-2503620, Topic summary for [114bis][210] FS_NR_AIML_Mob, Nokia, RAN4#114bis.

[3] R2-2409502, Report of 3GPP TSG RAN WG2 meeting #127bis, RAN2#127bis.

[4] Report of 3GPP TSG RAN WG2 meeting #128, RAN2#128.

[5] Report of 3GPP TSG RAN WG2 meeting #129, RAN2#129.