3GPP RAN WG4 Meeting #115	R4-2505987

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title: 	Discussion on testability and interoperability issues for AIML mobility

Source: 	CATT

Agenda item:	7.23.4

Document for:	Discussion

Introduction

In RAN4#114bis meeting, RAN4 continued to discuss the AI/ML mobility SI and the agreements were captured in [1]. In this contribution, we will further discuss the impacts of testability and interoperability of AI/ML mobility.

Discussion

Testing goals

In RAN4#113 meeting, RAN4 agreed a high-level principle for testing goal:

RAN4#113: - Agreement - As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

However, the testing goal needs to be further discussed for each scenario based on the above principle. Actually RAN2 has clarified study goals for all use cases and in simulation assumption the evaluation metrics are also given:

TR 38.744 The use cases in this study focus on RRC_CONNECTED mode and cover RRM measurement prediction, measurement event prediction and RLF/HOF prediction for PCell change procedure in standalone NR scenario. The study of the use cases is driven mainly by two study goals. The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements. The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption). … Measurement reduction rate for intra-frequency scenario is defined in the temporal domain (called MRRT) by assuming same length of measurement time instances and spatial domain respectively(called MRRS): MRRT = skipped measurement time instances / total measurement time instances MRRS = skipped beams to be measured/ total beams to be measured

Apparently, the motivation of this feature is to save measurement efforts or to improve HO performance. Therefore for scenario 2 and 3, the aspect that matters is the measurement efforts that UE reduces. However, given that the MRRT or MRRS may be a fixed value when measurement configuration signalling is indicated to UE, RAN4 can test whether the prediction results satisfy the existing measurement accuracy requirements under certain side conditions, e.g., MRRT or MRRS.

Proposal 1: For scenario 2 and 3, RAN4 to verify whether measurement results satisfy the existing accuracy requirements under certain side conditions, e.g., MRRT or MRRS.

For scenario 4, RAN4 needs to verify the HO performance as it is highlighted above. The issue is the metrics for HO performance evaluation is not defined in RAN4. The only requirement that is related to handover is the handover delay requirement. Thus, if RAN2’s purpose is to shorten the handover delay/interruption, then current RAN4 handover delay requirement can be used as a starting point. But if RAN2 try to reduce other aspects, such as Ping-pong HO, HOF/RLF, then RAN4 needs to further study how to evaluate HO performance in RAN4 test and the feasibility of defining metrics based on ping-pong HO, etc.

Proposal 2: For scenario 4, RAN4 can use HO delay requirement as a starting point when HO delay is shortened. For other HO enhanced aspects, RAN4 further discuss whether and how to define HO performance metric.

Testing in FR1

In RAN#112bis meeting, it was agreed to adopt conducted testing method for AI/ML mobility test in FR1 and consider at least the following aspects for this SI:

	-	Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction (for scenario 3)

	-	Testing cell-level L3-RSRP measurements

	-	Consideration of serving and neighbor cells

Firstly, we would like to discuss how to make DUT obtain cell-level L3-RSRPs at model input in FR1 conducted test. In general, there are two methods: (1) configure only one SSB resource for each serving/neighbor cell and DUT performs RSRP measurements, and the results are treated as cell-level RSRPs which then can be used as model input for further predictions. (2) configure multiple SSB resources for each serving/neighbor cell and the RSRP results are regarded as beam-level RSRPs, and then DUT calculates cell-level RSRPs for all cells based on these beam-level RSRPs. A figure illustrating these two methods is provided in Figure 1. Considering the test will be carried out in a conducted manner and RAN4 agrees to prioritize scenario 2, 3 and 4 which focus on saving measurement efforts in temporal or frequency domain, method (1), in our opinion, is sufficient to test DUT’s capability of AI/ML-enabled RRM measurement prediction.

Regarding channel model, since the prioritized scenarios in RAN4 do not involve predictions in spatial domain, AWGN or TDL channel can serve the purpose, which do not require upgrade on existing test equipment.

Figure 1. Setting up cell-level RSRP in FR1 conducted test

Proposal 3: RAN4 configures one SSB per cell and the measured RSRP results are regarded as cell-level RSRPs.

Proposal 4: AWGN or TDL channel can be used in FR1 conducted test.

For number of cells configured in FR1 test, RAN4 discussed this issue in last meeting and reached the following agreement:

Agreement: - RAN4 to consider two cells: serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction. - FFS: RAN4 to study if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction.

There is one open issue that needs further discussion. For inter-cell RRM measurement prediction/event prediction, two approaches are considered by RAN2, i.e., single-cell approach and cluster approach. If single-cell method is adopted, configuring 2 cells in tests are enough, and more cells may be needed if cluster approach is adopted. Therefore we suggest RAN4 to decide the exact number of cells for inter-cell RRM measurement prediction/event prediction in WI phase.

Proposal 5: 2 cells are enough if single-cell approach is adopted and more cells are needed if cluster approach is used. RAN4 can decide the exact number in WI phase.

Testability of inter-frequency scenario

Inter-frequency prediction is a slightly special case since UE predicts the RSRP of carrier 1 based on the measurement result of carrier 2. Some conditions need to be met if a good performance of this mechanism is expected. One of the key aspects is these two cells needs to be co-located. How to reflect their colocation in test is a main problem. Since test up in FR2 is preferred to be deprioritized, the following discussion focuses on FR1 only.

First, both carriers belong to the same frequency range, which means they experience a similar propagation condition over the air, e.g., they probably have a similar pathloss. If reference signals carried by these two carriers are transmitted with the same power, the RSRP at the DUT may be similar as well. Hence, a straightforward method is to reflect the co-location relationship by a minor RSRP difference between reference signals carried by co-located carriers.

Proposal 6: Colocation can be reflected by a minor RSRP difference between two carriers.

Conclusions

This contribution discussed the testability and interoperability issues in AI/ML mobility and the following proposals are provided:

Proposal 1: For scenario 2 and 3, RAN4 to verify whether measurement results satisfy the existing accuracy requirements under certain side conditions, e.g., MRRT or MRRS.

Proposal 2: For scenario 4, RAN4 can use HO delay requirement as a starting point when HO delay is shortened. For other HO enhanced aspects, RAN4 further discuss whether and how to define HO performance metric.

Proposal 3: RAN4 configures one SSB per cell and the measured RSRP results are regarded as cell-level RSRPs.

Proposal 4: AWGN or TDL channel can be used in FR1 conducted test.

Proposal 5: 2 cells are enough if single-cell approach is adopted and more cells are needed if cluster approach is used. RAN4 can decide the exact number in WI phase.

Proposal 6: Colocation can be reflected by a minor RSRP difference between two carriers.

References

[1] R4-2504901, WF on AI/ML Mobility, Nokia, RAN4#114bis.