3GPP RAN WG4 Meeting #115	R4-2505988

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title: 	Discussion on UE to UE CLI handling

Source: 	CATT

Agenda item:	7.25.4.1

Document for:	Discussion

Introduction

In RAN4#114bis meeting, RAN4 made fruitful progress of the UE to UE CLI requirements in R19 SBFD WI. Some agreements were achieved and captured in the way forward [1]. In this contribution, we will further discuss the UE to UE CLI handling.

Discussion

Requirement applicability

Regarding the applicability of L1 CLI requirements, proposals were brought up in previous meeting and the majority view is reproduced below:

RAN4#114bis: - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resource is configured within active BWP

Considering this description is aligned with Rel-16 CLI requirements and RAN1 definition for L1 CLI measurements, we suggest agreeing the above option:

Proposal 1: 	The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

      - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP.

      - when CLI-RSSI measurement resource is configured within active BWP.

Scheduling restriction

L1-SRS-RSRP:

In RAN4#114bis, RAN4 agreed the following scheduling restriction for L1-SRS-RSRP CLI measurement:

- Agreement - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for SRS resource and Y symbols before, where Y is re-using same number as R16 requirements. - For the symbols after the SRS resource, not define scheduling restriction on symbols after symbols for SRS resource in TS 38.133, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211. - For DL, - if UE is able to support FDM-ed DL reception and SRS measurement, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, otherwise (if UE does not support the capability), scheduling restriction applies. - The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements. - FFS whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband.

One open issue as highlighted is whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband. Similar to the agreement for UL, we do not think extra restricted symbol(s) is needed for this case with the assumption that the UL/DL switching time is covered by RAN1 specification TS 38.211.

Proposal 2: For DL scheduling restriction of L1-SRS-RSRP measurement, RAN4 not to define scheduling restriction on symbols after SRS resource assuming the UL/DL switching time is covered by RAN1 specification TS 38.211.

L1-CLI-RSSI:

For L1-CLI RSSI measurement method #3, discussion is also required to determine whether extra restricted symbols are needed after RSSI resources:

- Agreements - Method #3: - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for RSSI resource. - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 - For DL, - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource. - Note: the common understanding in RAN4 if the RSSI in UL suband is too strong compare to the wanted signals in DL suband(s), UE may fail to decode the PDCCH or PDSCH. - FFS whether restricted symbol(s) is needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband.

Similar to our discussion for L1-SRS-RSRP measurement above, we do not think extra restricted symbol(s) is needed for this case with the assumption that the UL/DL switching time is covered by RAN1 specification TS 38.211.

Proposal 3: For DL scheduling restriction of L1-CLI-RSSI measurement method #3, RAN4 not to define scheduling restriction on symbols after RSSI resource assuming the UL/DL switching time is covered by RAN1 specification TS 38.211.

Measurement restriction

Collision between L1-SRS-RSRP and L1-CLI-RSSI collision

In RAN4#114bis meeting, further discussion is needed when L1-SRS-RSRP and L1-CLI-RSSI measurements collide:

RAN4#114bis Agreements: - Measurement requirements do not apply for L1-SRS-RSRP when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options - Option 1: Prioritize L1-SRS-RSRP measurement - Option 2: Prioritize L1-CLI-RSSI measurement - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI

Considering that the purpose of CLI measurements is to evaluate the interference level to victim UE in DL subband(s), i.e., network cares the channel quality of DL subbands of victim UE. Therefore, when L1-SRS-RSRP and L1-CLI-RSSI measurement resources collide, UE shall prioritize L1-CLI-RSSI measurement.

Proposal 4: UE shall prioritize L1-CLI-RSSI measurement when L1-SRS-RSRP and L1-CLI-RSSI measurement resources collide.

Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements

In last meeting, RAN4 achieved the following agreement for the collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements:

- Agreements - Collision between L1-CLI-RSSI measurements with different methods - Capture in RAN4 specs that UE is not expected to perform CLI measurement using Method #1 and Methods#3 in the same OFDM symbol. - Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements - For Method #1& Method 3: - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR2, FFS for FR1.

For FR1, with the assumption that omnidirectional Rx antenna is used by UE to perform DL reception, we think UE can perform both L1-CLI-RSSI and L1/L3 RRM measurements simultaneously when the corresponding resources overlap in time domain. Therefore it is not needed to define measurement restrictions.

Proposal 5: For FR1, UE performs both L1-CLI-RSSI and L1/L3 RRM measurements simultaneously. No measurement restrictions are required.

Conclusions

This contribution discussed the impacts of L1 UE-to-UE CLI on RRM requirements and some proposals are provided:

Proposal 1: 	The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

      - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP.

      - when CLI-RSSI measurement resource is configured within active BWP.

Proposal 2: For DL scheduling restriction of L1-SRS-RSRP measurement, RAN4 not to define scheduling restriction on symbols after SRS resource assuming the UL/DL switching time is covered by RAN1 specification TS 38.211.

Proposal 3: For DL scheduling restriction of L1-CLI-RSSI measurement method #3, RAN4 not to define scheduling restriction on symbols after RSSI resource assuming the UL/DL switching time is covered by RAN1 specification TS 38.211.

Proposal 4: UE shall prioritize L1-CLI-RSSI measurement when L1-SRS-RSRP and L1-CLI-RSSI measurement resources collide.

Proposal 5: For FR1, UE performs both L1-CLI-RSSI and L1/L3 RRM measurements simultaneously. No measurement restrictions are required.

References

[1] R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN4#114bis.