3GPP TSG-RAN WG4 Meeting #115				                                                R4-2505994

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title:	Discussion on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Source:	CATT

Agenda Item:	7.18.3.2

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction by optimizing CSSF outside gap in CA/DC was discussed and the conclusions were captured in the approved WF [1].In this paper, we will continue the discussion on open issues of FR2-1 SSB based measurement delay reduction by optimizing CSSF outside gap in CA/DC [2] and give our views.

2 Discussion

In the way forward [1], an open issue is that whether to expand solution 1 to FR1 band:

Issue 1-2-2: Expand solution 1 to FR1 case [Way forward]: - FFS: - Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band. - Option 1a (Nokia): - Expand the solution 1 to at least FR1 intra-band contiguous CA. - Details: - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately. - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA. - Option 2 (Huawei): Not to expand solution 1 to FR1 band. - Option 3 (moderator): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

According to the WID [3], the scope of CSSF optimization focuses on FR2-1 band. Expanding the solution 1 to FR1 band will be out of scope.

Proposal 1: RAN4 do not expand the solution 1 to FR1 band.

Another issue is about the type of signaling that is used to indicate the to-be-measured serving SCC in a FR2 band. The candidates are shown below:

“Network indication of a specific SCC per-band for measurement” is Option 1 (Apple, HW, ZTE, CATT): dynamic indication based on MAC CE Option 2 (QC, OPPO): semi-static indication based on RRC signaling Option 3 (ZTE, Nokia, Ericsson, MTK): leave to RAN2 to decide.

In our understanding, both MAC CE and RRC signallings are feasible. However, dynamical indication using MAC CE can provide more flexibility to network and save overheads in air interface. In real deployment, network may indicate a UE to measure a specific SCC in a FR2 band by considering the system-level performance, such as load-balancing. If network can indicate this specific SCC via MAC CE, then UE can store the measurement and reporting configurations of other SCCs. And in another time period, network may indicate the same UE to measure another configured SCC instead of previous SCC in this FR2 band. However, this flexibility cannot be achieved by using RRC which will re-configure the measurement and reporting configurations, and apparently this procedure takes longer time than using MAC CE and therefore degrades the delay reduction gain of this feature. Based on above analysis, we prefer to use MAC CE to dynamic indicate the to-be-measured SCC per band.

Proposal 2: Network indication of a specific SCC per-band for measurement is dynamic indication based on MAC CE.

For 3-searcher based solution 3, the following issues need further discussion:

Issue 1-3-2: detailed design for solution 3 (by-default option 3 vs. need network indication for specific SCC) [Way forward]: - FFS: - Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF. - Note: 3-searchers will be used by UE if UE report this capability. - FFS: Other restriction (triggered by Huawei): - Option 1: NW can configure per-FR measurement gaps to UE when UE supports per-FR measurement gaps and new 3-searcehr capability simultaneously. But the SMTC occasions of outside gap measurement object in one FR should not be overlapped by per-FR measurement gaps in another FR. (HW, MTK, VIVO, Apple) - Option 2: When UE supports per-FR measurement gaps and 3-searcher capability simultaneously, no new requirements. (MTK, ZTE, Apple) - Option 3: UE is able to support per-FR measurement gaps and new 3-searceher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR. (HW, MTK, Apple, VIVO, Ericssion, ZTE, CATT) - Option 3a: The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. (HW, OPPO)

For the 1st FFS, we are okay with the by default solution 3 and we are also open to further discuss whether to introduce a network signalling to indicate a specific SCC to use for the 3rd searcher of CSSF. For the 2nd FFS, the key point is to avoid the number of searchers increases and exceeds the number in the searcher assumption in some scenarios, e.g., when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR. During the last meeting, an applicability rule was proposed that UE will follow legacy 2-searcehr requirements and the new 3-searcher requirements are not applicable. With this applicability rule, the situation mentioned above can be avoided. Therefore we support to add this applicability rule to the solution 3.

Proposal 3: 3-searcher based solution includes the following applicability rule:

The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR.

Since the solutions for L3 measurement delay reduction by optimizing CSSF outside gap are almost stable and we have only 2 meetings left before the core part ends, we would like to discuss the UE capabilities in this contribution. In last meeting, some agreements about UE capabilities design were reached:

Issue 1-4-1: UE capability for CSSF enhancement Agreement: - UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR. - UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

Based on the above agreement, an initial summary of UE capabilities that need defining for this objective is provided below:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| xx. NR_RRM_ph5 | xx-1 | L3 measurement delay reduction by measuring one serving CC per band | Support of L3 measurement delay reduction by measuring one serving CC per band |  | Yes | N/A | UE does not support L3 measurement delay reduction by measuring one serving CC per band | Per UE | No | N/A | FR2 only |  | Optional |
| xx. NR_RRM_ph5 | xx-2 | L3 measurement delay reduction by supporting 3 searchers | 1. Scenarios where UE supports L3 measurement delay reduction by supporting 3 searchers |  | Yes | N/A | UE does not support L3 measurement delay reduction by supporting 3 searchers | Per UE | No | N/A | N/A | Component 1 candidate values: {FR1 only CA and FR1 only NR-DC, FR1 only EN-DC, FR1+F2 CA, FR1+FR2 NR-DC} | Optional |

Proposal 4: RAN4 to discuss UE capabilities for this objective based on the following table:

| Features | Index | Feature group | Components | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| xx. NR_RRM_ph5 | xx-1 | L3 measurement delay reduction by measuring one serving CC per band | Support of L3 measurement delay reduction by measuring one serving CC per band | Yes | UE does not support L3 measurement delay reduction by measuring one serving CC per band | Per UE | No | FR2 only |  | Optional |
| xx. NR_RRM_ph5 | xx-2 | L3 measurement delay reduction by supporting 3 searchers | 1. Scenarios where UE supports L3 measurement delay reduction by supporting 3 searchers | Yes | UE does not support L3 measurement delay reduction by supporting 3 searchers | Per UE | No | N/A | Component 1 candidate values: {FR1 only CA and FR1 only NR-DC, FR1 only EN-DC, FR1+F2 CA, FR1+FR2 NR-DC} | Optional |

3 Conclusion

In this paper, we have some discussions on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, and the following proposals are given:

Proposal 1: RAN4 do not expand the solution 1 to FR1 band.

Proposal 2: Network indication of a specific SCC per-band for measurement is dynamic indication based on MAC CE.

Proposal 3: 3-searcher based solution includes the following applicability rule:

The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR.

Proposal 4: RAN4 to discuss UE capabilities for this objective based on the following table:

| Features | Index | Feature group | Components | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| xx. NR_RRM_ph5 | xx-1 | L3 measurement delay reduction by measuring one serving CC per band | Support of L3 measurement delay reduction by measuring one serving CC per band | Yes | UE does not support L3 measurement delay reduction by measuring one serving CC per band | Per UE | No | FR2 only |  | Optional |
| xx. NR_RRM_ph5 | xx-2 | L3 measurement delay reduction by supporting 3 searchers | 1. Scenarios where UE supports L3 measurement delay reduction by supporting 3 searchers | Yes | UE does not support L3 measurement delay reduction by supporting 3 searchers | Per UE | No | N/A | Component 1 candidate values: {FR1 only CA and FR1 only NR-DC, FR1 only EN-DC, FR1+F2 CA, FR1+FR2 NR-DC} | Optional |

4 References

- R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#114bis.

- R4-2500531, Topic summary for [114bis][208] NR_RRM_Ph5_Part1, Apple, RAN4#114bis.

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, RAN#105.