3GPP TSG-RAN WG4 Meeting #115				                                                R4-2505997

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title:	Discussion on performance requirements for FR2-1 L3 measurement delay reduction

Source:	CATT

Agenda Item:	7.18.5

Document for:	Discussion

1 Introduction

RAN4 started discussing the performance requirements of FR2-1 L3 measurement delay reduction in April meeting. In this contribution, we will continue to present our views on the relevant open issues.

2 Discussion

In last meeting, tables including initial considerations for test cases of Rx BSF optimization and CSSF optimization are provided in the WF [1] and the details of configurations and scopes need further discussion.

RAN4#114bis meeting: Issue 2-1-1: TC list for Rx BSF optimization Agreement in online session: Further discuss based on the following table: Index; Scenario; Need or not; Detailed configurations and scopes; Company. 1; Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1 or 2 TCs; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); CMCC. 3; Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); CATT. 2; Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1 or 2 TCs; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); Huawei. 4; Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); Samsung. 5; Scenario 5: Handover; 1 or 2 TCs; TBA(e.g., Intra-frequency or inter-frequency, unknown); OPPO. 6; Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection; 1 or 2 TCs; TBA(e.g., re-establishment or RRC redirection, Intra-frequency or inter-frequency, [or known] or unknown); Intel. Issue 2-2-1: TC list for CSSF optimization - TC for solution 1: Agreement in online session: - Define the test cases for solution 1 of CSSF enhancement following the table below: TC index; Scenario; Need or not; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; Nokia. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; Ericsson. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; ZTE. - TC for solution 3: [Way forward]: - FFS: - Define the test cases for solution 3 of CSSF enhancement following the table below: TC index; Aspects; CA/DC mode; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA.

For the details of test cases, the legacy test cases can be used as starting points for both core and performance requirements of Rx BSF optimization and CSSF optimization. In order to control the number of test cases, we choose some legacy test cases as baseline and they are listed in table 1, 2 and 3 to facilitate RAN4 discussions [2].

In table 1, we list the existing test cases that can be used as starting points for both FR2-1 L3 measurement delay reduction by optimizing Rx BSF and FR2-1 L3 measurement delay reduction by optimizing CSSF. And in table 2 and 3, we list test cases that can be used as starting points for FR2-1 L3 measurement delay reduction by optimizing Rx BSF and FR2-1 L3 measurement delay reduction by optimizing CSSF respectively. Down-selection or addition of new test cases can be made during discussion.

Table 1-Test cases needed for both FR2-1 L3 measurement delay reduction by optimizing Rx BSF and

FR2-1 L3 measurement delay reduction by optimizing CSSF

| Test case No. | Title of the existing test cases | Requirements to be verified |
| --- | --- | --- |
| 1-1 | A.7.6.1.1 SA event triggered reporting test without gap under non-DRX [Intra-frequency without MG] | 9.2.5.1, 9.2.5.2 |
| 1-2 | A.7.6.1.2 SA event triggered reporting test without gap under DRX [Intra-frequency without MG] | 9.2.5.1, 9.2.5.2 |
| 1-3 | A.7.6.1.3 SA event triggered reporting test with per-UE gaps under non-DRX [Intra-frequency with MG] | 9.2.6 |
| 1-4 | A.7.6.1.4 SA event triggered reporting test with per-UE gaps under DRX [Intra-frequency with MG] | 9.2.6 |
| 1-5 | A.7.6.14.1 Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2 [Intra-frequency with Pre-MG] | 9.2.6.2, 9.2.6.3 |
| 1-6 | A.7.6.14.2 Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2 [Intra-frequency with Pre-MG] | 9.2.6.2, 9.2.6.3 |
| 1-7 | A.7.6.16.1 SA event triggered reporting test with per-UE NCSG under non-DRX [Intra-frequency with NCSG] | 9.2.7.1, 9.2.7.2 |
| 1-8 | A.7.6.2.10 SA event triggered reporting test without gap under non-DRX [Inter-frequency without MG] | 9.3.9 |
| 1-9 | A.7.6.2.11 SA event triggered reporting test without gap under DRX [Inter-frequency without MG] | 9.3.9 |
| 1-10 | A.7.6.2.1 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2) [Inter-frequency with MG] | 9.3.4 |
| 1-11 | A.7.6.2.4 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR2) [Inter-frequency with MG] | 9.3.4 |
| 1-12 | A.7.6.16.2 SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2) [Inter-frequency with NCSG] | 9.3.10 |
| 1-13 | A.7.6.15.1 SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements [Inter-frequency with MG] | 9.3.4 |

Table 2-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing Rx BSF

| Test case No. | Title of the existing test cases | Requirements to be verified |
| --- | --- | --- |
| 2-1 | A.7.3.1.1 Inter-frequency handover from FR1 to FR2; unknown target cell [handover] | 6.1.1.5. |
| 2-2 | A.7.3.1.2 Intra-frequency handover from FR2 to FR2; unknown target cell [handover] | 6.1.1.4. |
| 2-3 | A.7.3.1.3 Inter-frequency handover from FR2 to FR2; unknown target cell [handover] | 6.1.1.4. |
| 2-4 | A.7.3.2.1.1 Intra-frequency RRC Re-establishment in FR2 [RRC Re-establishment] | 6.2.1. |
| 2-5 | A.7.3.2.1.2 Inter-frequency RRC Re-establishment in FR2 [RRC Re-establishment] | 6.2.1. |
| 2-6 | A.7.3.2.3.1 Redirection from NR in FR2 to NR in FR2 [RRC Connection Release with Redirection] | 6.2.3.2.1. |

Table 3-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing CSSF

| Test case No. | Test case title | Requirement to be verified |
| --- | --- | --- |
| 3-1 | A.5.6.1.1 EN-DC event triggered reporting test without gap under non-DRX [intra-frequency without gap] | 9.2.5.1 and 9.2.5.2 |
| 3-2 | A.5.6.1.2 EN-DC event triggered reporting test without gap under DRX [intra-frequency without gap] | 9.2.5.1 and 9.2.5.2 |
| 3-3 | A.5.6.1.3 EN-DC event triggered reporting test with per-UE gaps under non-DRX [intra-frequency with gap] | 9.2.6 |
| 3-4 | A.5.6.1.4 EN-DC event triggered reporting test with per-UE gaps under DRX [intra-frequency with gap] | 9.2.6 |
| 3-5 | A.5.6.2.1 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used [inter-frequency with gap] | 9.3.4 |
| 3-6 | A.5.6.2.2 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used [inter-frequency with gap] | 9.3.4 |
| 3-7 | A.5.6.2.3 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used [inter-frequency with gap] | 9.3.4 |
| 3-8 | A.5.6.2.4 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used [inter-frequency with gap] | 9.3.4 |

Proposal 1: Use the following existing test cases as starting points to define test cases for FR2-1 L3 measurement delay reduction by optimizing Rx BSF and CSSF:

Table 1-Test cases needed for both FR2-1 L3 measurement delay reduction by optimizing Rx BSF and

FR2-1 L3 measurement delay reduction by optimizing CSSF

| Test case No. | Title of the existing test cases | Requirements to be verified |
| --- | --- | --- |
| 1-1 | A.7.6.1.1 SA event triggered reporting test without gap under non-DRX [Intra-frequency without MG] | 9.2.5.1, 9.2.5.2 |
| 1-2 | A.7.6.1.2 SA event triggered reporting test without gap under DRX [Intra-frequency without MG] | 9.2.5.1, 9.2.5.2 |
| 1-3 | A.7.6.1.3 SA event triggered reporting test with per-UE gaps under non-DRX [Intra-frequency with MG] | 9.2.6 |
| 1-4 | A.7.6.1.4 SA event triggered reporting test with per-UE gaps under DRX [Intra-frequency with MG] | 9.2.6 |
| 1-5 | A.7.6.14.1 Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2 [Intra-frequency with Pre-MG] | 9.2.6.2, 9.2.6.3 |
| 1-6 | A.7.6.14.2 Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2 [Intra-frequency with Pre-MG] | 9.2.6.2, 9.2.6.3 |
| 1-7 | A.7.6.16.1 SA event triggered reporting test with per-UE NCSG under non-DRX [Intra-frequency with NCSG] | 9.2.7.1, 9.2.7.2 |
| 1-8 | A.7.6.2.10 SA event triggered reporting test without gap under non-DRX [Inter-frequency without MG] | 9.3.9 |
| 1-9 | A.7.6.2.11 SA event triggered reporting test without gap under DRX [Inter-frequency without MG] | 9.3.9 |
| 1-10 | A.7.6.2.1 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2) [Inter-frequency with MG] | 9.3.4 |
| 1-11 | A.7.6.2.4 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR2) [Inter-frequency with MG] | 9.3.4 |
| 1-12 | A.7.6.16.2 SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2) [Inter-frequency with NCSG] | 9.3.10 |
| 1-13 | A.7.6.15.1 SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements [Inter-frequency with MG] | 9.3.4 |

Proposal 2: Use the following existing test cases as starting points to define test cases for FR2-1 L3 measurement delay reduction by optimizing Rx BSF:

Table 2-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing Rx BSF

| Test case No. | Title of the existing test cases | Requirements to be verified |
| --- | --- | --- |
| 2-1 | A.7.3.1.1 Inter-frequency handover from FR1 to FR2; unknown target cell [handover] | 6.1.1.5. |
| 2-2 | A.7.3.1.2 Intra-frequency handover from FR2 to FR2; unknown target cell [handover] | 6.1.1.4. |
| 2-3 | A.7.3.1.3 Inter-frequency handover from FR2 to FR2; unknown target cell [handover] | 6.1.1.4. |
| 2-4 | A.7.3.2.1.1 Intra-frequency RRC Re-establishment in FR2 [RRC Re-establishment] | 6.2.1. |
| 2-5 | A.7.3.2.1.2 Inter-frequency RRC Re-establishment in FR2 [RRC Re-establishment] | 6.2.1. |
| 2-6 | A.7.3.2.3.1 Redirection from NR in FR2 to NR in FR2 [RRC Connection Release with Redirection] | 6.2.3.2.1. |

Proposal 3: Use the following existing test cases as starting points to define test cases for FR2-1 L3 measurement delay reduction by optimizing CSSF:

Table 3-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing CSSF

| Test case No. | Test case title | Requirement to be verified |
| --- | --- | --- |
| 3-1 | A.5.6.1.1 EN-DC event triggered reporting test without gap under non-DRX [intra-frequency without gap] | 9.2.5.1 and 9.2.5.2 |
| 3-2 | A.5.6.1.2 EN-DC event triggered reporting test without gap under DRX [intra-frequency without gap] | 9.2.5.1 and 9.2.5.2 |
| 3-3 | A.5.6.1.3 EN-DC event triggered reporting test with per-UE gaps under non-DRX [intra-frequency with gap] | 9.2.6 |
| 3-4 | A.5.6.1.4 EN-DC event triggered reporting test with per-UE gaps under DRX [intra-frequency with gap] | 9.2.6 |
| 3-5 | A.5.6.2.1 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used [inter-frequency with gap] | 9.3.4 |
| 3-6 | A.5.6.2.2 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used [inter-frequency with gap] | 9.3.4 |
| 3-7 | A.5.6.2.3 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used [inter-frequency with gap] | 9.3.4 |
| 3-8 | A.5.6.2.4 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used [inter-frequency with gap] | 9.3.4 |

For Rx BSF optimization, another aspect is whether to test quitting FBS or not in the tests. From our perspective, as a part of solution, the quitting should be tested in the tests. But considering the details of quitting mechanism is not completed yet, RAN4 can further discuss the relevant configurations after it is concluded.

Proposal 4: RAN4 to test quitting FBS in the tests. Details of configuration can be further discussed after the quitting mechanism is concluded.

Regarding test cases for performance requirements, we also think the existing test cases for SS-RSRP, SS-RSRQ and SS-SINR measurements can be used as a starting point. Details of configuration can be discussed after the test cases for delay requirement verification are stabilized.

Proposal 5: Use the existing test cases for SS-RSRP, SS-RSRQ and SS-SINR measurements as starting point. Details of configuration can be discussed after the test cases for delay requirement verification are stabilized.

3 Conclusion

In this paper, we discussed performance requirements for FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, and the following proposals are given:

Proposal 1: Use the following existing test cases as starting points to define test cases for FR2-1 L3 measurement delay reduction by optimizing Rx BSF and CSSF:

Table 1-Test cases needed for both FR2-1 L3 measurement delay reduction by optimizing Rx BSF and

FR2-1 L3 measurement delay reduction by optimizing CSSF

| Test case No. | Title of the existing test cases | Requirements to be verified |
| --- | --- | --- |
| 1-1 | A.7.6.1.1 SA event triggered reporting test without gap under non-DRX [Intra-frequency without MG] | 9.2.5.1, 9.2.5.2 |
| 1-2 | A.7.6.1.2 SA event triggered reporting test without gap under DRX [Intra-frequency without MG] | 9.2.5.1, 9.2.5.2 |
| 1-3 | A.7.6.1.3 SA event triggered reporting test with per-UE gaps under non-DRX [Intra-frequency with MG] | 9.2.6 |
| 1-4 | A.7.6.1.4 SA event triggered reporting test with per-UE gaps under DRX [Intra-frequency with MG] | 9.2.6 |
| 1-5 | A.7.6.14.1 Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2 [Intra-frequency with Pre-MG] | 9.2.6.2, 9.2.6.3 |
| 1-6 | A.7.6.14.2 Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2 [Intra-frequency with Pre-MG] | 9.2.6.2, 9.2.6.3 |
| 1-7 | A.7.6.16.1 SA event triggered reporting test with per-UE NCSG under non-DRX [Intra-frequency with NCSG] | 9.2.7.1, 9.2.7.2 |
| 1-8 | A.7.6.2.10 SA event triggered reporting test without gap under non-DRX [Inter-frequency without MG] | 9.3.9 |
| 1-9 | A.7.6.2.11 SA event triggered reporting test without gap under DRX [Inter-frequency without MG] | 9.3.9 |
| 1-10 | A.7.6.2.1 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2) [Inter-frequency with MG] | 9.3.4 |
| 1-11 | A.7.6.2.4 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR2) [Inter-frequency with MG] | 9.3.4 |
| 1-12 | A.7.6.16.2 SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2) [Inter-frequency with NCSG] | 9.3.10 |
| 1-13 | A.7.6.15.1 SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements [Inter-frequency with MG] | 9.3.4 |

Proposal 2: Use the following existing test cases as starting points to define test cases for FR2-1 L3 measurement delay reduction by optimizing Rx BSF:

Table 2-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing Rx BSF

| Test case No. | Title of the existing test cases | Requirements to be verified |
| --- | --- | --- |
| 2-1 | A.7.3.1.1 Inter-frequency handover from FR1 to FR2; unknown target cell [handover] | 6.1.1.5. |
| 2-2 | A.7.3.1.2 Intra-frequency handover from FR2 to FR2; unknown target cell [handover] | 6.1.1.4. |
| 2-3 | A.7.3.1.3 Inter-frequency handover from FR2 to FR2; unknown target cell [handover] | 6.1.1.4. |
| 2-4 | A.7.3.2.1.1 Intra-frequency RRC Re-establishment in FR2 [RRC Re-establishment] | 6.2.1. |
| 2-5 | A.7.3.2.1.2 Inter-frequency RRC Re-establishment in FR2 [RRC Re-establishment] | 6.2.1. |
| 2-6 | A.7.3.2.3.1 Redirection from NR in FR2 to NR in FR2 [RRC Connection Release with Redirection] | 6.2.3.2.1. |

Proposal 3: Use the following existing test cases as starting points to define test cases for FR2-1 L3 measurement delay reduction by optimizing CSSF:

Table 3-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing CSSF

| Test case No. | Test case title | Requirement to be verified |
| --- | --- | --- |
| 3-1 | A.5.6.1.1 EN-DC event triggered reporting test without gap under non-DRX [intra-frequency without gap] | 9.2.5.1 and 9.2.5.2 |
| 3-2 | A.5.6.1.2 EN-DC event triggered reporting test without gap under DRX [intra-frequency without gap] | 9.2.5.1 and 9.2.5.2 |
| 3-3 | A.5.6.1.3 EN-DC event triggered reporting test with per-UE gaps under non-DRX [intra-frequency with gap] | 9.2.6 |
| 3-4 | A.5.6.1.4 EN-DC event triggered reporting test with per-UE gaps under DRX [intra-frequency with gap] | 9.2.6 |
| 3-5 | A.5.6.2.1 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used [inter-frequency with gap] | 9.3.4 |
| 3-6 | A.5.6.2.2 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used [inter-frequency with gap] | 9.3.4 |
| 3-7 | A.5.6.2.3 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used [inter-frequency with gap] | 9.3.4 |
| 3-8 | A.5.6.2.4 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used [inter-frequency with gap] | 9.3.4 |

Proposal 4: RAN4 to test quitting FBS in the tests. Details of configuration can be further discussed after the quitting mechanism is concluded.

Proposal 5: Use the existing test cases for SS-RSRP, SS-RSRQ and SS-SINR measurements as starting point. Details of configuration can be discussed after the test cases for delay requirement verification are stabilized.

4 References

- R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#114bis.

- 3GPP TS 38.133, NR; Requirements for support of radio resource management, V18.8.0.