3GPP RAN WG4 Meeting #115	R4-2506000

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title: 	Discussion on the impacts of low band CA via switching on RRM requirements

Source: 	CATT

Agenda item:	7.7.4

Document for:	Discussion

Introduction

In RAN4#114bis meeting, RAN4 discussed the impacts of low-low band carrier aggregation via switching on RRM requirements. The achieved agreements are captured in [1]. In this contribution, we will further present our views on the open issues summarized in [2].

Discussion

Switching pattern applicability for RRM requirement in LBCA via switching

RAN4 discussed the applicability of switching pattern in RAN4#114bis and the following agreements were reached that were informed to RAN1 via LS [3]:

Issue 1-1-3: switching pattern applicability for RRM requirement in R19 LB CA via switching Agreement: - For the SCell which has been activated (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirements for deactivated SDL SCell measurement. Note: the above FFS parts will be discussed and decided in RAN4.

According to the above agreement, the switching pattern for SDL SCell applies for the SCell that has been activated. For the SCell which is in activation procedure or deactivated, this switching pattern designed by RAN1 does not apply, and RAN4 will define corresponding rules to perform activation procedure and measurements in this low-band CA via switching work item.

SCell activation procedure

For SCell activation, a basic principle is assuming that UE utilizes the RS occasions to perform SDL SCell activation, which means UE will switch to SDL SCell to receive RS occasions and when UE finishes the receptions it will switch back to PCell for DL reception and UL transmission. Apparently, SCell activation procedure has impacts only on the symbols where RS occasions are transmitted. Since NW knows the temporal location of RS occasions of SCell, it can avoid scheduling this UE for DL reception and UL transmission in PCell. Therefore we think scheduling restriction needs to be defined to avoid collisions between PCell data transmission and SCell activation. Regarding the type of RS occasions, at least SSB needs to be supported.

Proposal 1: RAN4 to define scheduling restriction for PCell when UE performs DL receptions of RS occasions in SCell.

Proposal 2: At least support SSB for SDL SCell activation.

Deactivation SCell measurement

For deactivated SCell measurement, there are two main opinions which are copied below for reference:

- Discuss following points: - Whether to define the requirement for deactivated SDL SCell L3 measurement or not? - Option 1: No (QC) - Option 2: Yes for SSB based operation (OPPO, CATT, Apple, LGE, HW, Ericsson, vivo, MTK, Nokia)

Companies that support Option 1 think that this feature is mainly for load balance instead of mobility, considering that the SCell and PCell are co-located. However, in our understanding, UE still needs to perform deactivated SCell measurement and report the results, because there is coverage difference between PCell and SCell, and NW needs to know whether UE has entered the coverage of SCell(s) or not and the quality of SCell(s) before it decides to activate the SCell(s) for UE.

Proposal 3: Define requirement for deactivated SDL SCell L3 measurement.

On how to define requirement for deactivated SDL SCell L3 measurement, there are two alternatives in the WF [1]:

If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirement for deactivated SDL SCell measurement.

Alt 1: UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement.

Alt 2: UE follows a new measurement requirement for deactivated SDL SCell measurement.

In our opinion, RAN4 should reuse the legacy deactivated SCell measurement requirements as much as possible, since there is no strong motivation to define a new measurement requirement. But considering that when UE performs deactivated SCell measurement following the measCycleSCell, it cannot perform data reception/transmission in PCell, RAN4 need to discuss the impacts of collisions between data reception/transmission in PCell and deactivated SCell measurement on deactivated SCell measurement requirement.

Proposal 4: RAN4 to reuse the legacy deactivated SCell measurement requirements as baseline.

Proposal 5: RAN4 to discuss the impacts of collisions between data reception/transmission in PCell and deactivated SCell measurement on deactivated SCell measurement requirement.

Interruption requirement

In RAN4#114bis, RAN4 discussed this issue without any progress. Based on the discussion, we think companies still have different understanding about this switching pattern, such as the temporal location which could lead to completely different conclusions. Considering RF group is discussing the location of timing mask which helps RRM further discuss the impacts of switching pattern on interruption requirement, we suggest RRM group wait for RF agreements and then discuss interruption requirement.

Proposal 6: RRM to discuss the interruption requirement after RF group reaches agreements on the time location of timing mask.

Conclusions

This contribution discussed the impacts of LBCA on RRM requirements and some proposals are provided:

Proposal 1: RAN4 to define scheduling restriction for PCell when UE performs DL receptions of RS occasions in SCell.

Proposal 2: At least support SSB for SDL SCell activation.

Proposal 3: Define requirement for deactivated SDL SCell L3 measurement.

Proposal 4: RAN4 to reuse the legacy deactivated SCell measurement requirements as baseline.

Proposal 5: RAN4 to discuss the impacts of collisions between data reception/transmission in PCell and deactivated SCell measurement on deactivated SCell measurement requirement.

Proposal 6: RRM to discuss the interruption requirement after RF group reaches agreements on the time location of timing mask.

References

[1] R4-2504895, WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4#114bis.

[2] R4-2503614, Topic summary for [114bis][204] NR_LBCA_Sw, Apple, RAN4#114bis.

[3] R4-2504969, LS of RAN4 agreements on LB CA via switching, Apple, RAN4#114bis.