3GPP TSG-RAN WG4 Meeting #115                                                                                           R4-2506006

Malta, MT, 19th – 23rd May, 2025

Source: 	CATT

Title: 	Discussion on SBFD general issues

Agenda item:	7.25.2

Document for:	Discussion

- Introduction

This contribution provides further discussion on the open issues for SBFD parameters according to the WF [1] in last RAN4 meeting. Channel BW, subband configuration and guard band size are discussed.

- Discussion

  - DU/UL subband declaration

Issue 1-1-1: Declaration and flexibility for SBFD operation

Agreement:

- For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

- For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

  - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

    - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

For FR1 MR SBFD capable BSs, 33dBm is a relatively reasonable value for power level threshold. If the BS output power is higher than this threshold, all configurations shall be tested. When the BS output power is lower than this threshold, the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

Proposal 1: About declaration and flexibility for SBFD operation, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm.

  - DL/UL subband configurations

According to WF [1] of  RAN4#114bis, some issues about DL/UL subband configurations need to be further studied.

2.2.1 Restriction on the number of UL subband sizes supported

Issue 1-2-1: Restriction on the number of UL subband sizes supported

Agreement:

- For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared

  - Maximum 3 subband configurations allowed for FR1 WA BS

    - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS.

    - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations.

  - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable.

    - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs.

      - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

For FR1 MR SBFD capable BSs, 33dBm is a relatively reasonable value for power level threshold. If the BS output power is higher than this threshold, the maximum number of specific UL subband sizes is same as that of FR1 WA BS, i.e.3. When the BS output power is lower than this threshold, the restriction on the number of UL subband sizes supported is not applicable.

Proposal 2: About restriction on the number of UL subband sizes supported, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm.

2.2.2 Restriction on the number of DL subband sizes supported

Issue 1-2-2: Restriction on the number of DL subband sizes supported

Agreement:

- For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

  - This restriction shall be applied to FR1 WA BS

  - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable.

    - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs.

      - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

For FR1 MR SBFD capable BSs, 33dBm is a relatively reasonable value for power level threshold. If the BS output power is higher than this threshold, the restriction on the number of DL subband sizes supported is same as that of FR1 WA BS. When the BS output power is lower than this threshold, the restriction on the number of DL subband sizes supported is not applicable.

Proposal 3: About restriction on the number of DL subband sizes supported, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm.

2.2.3 Restriction on guardband size for SBFD

Issue 1-2-3: Restriction on guardband size for SBFD

Agreement:

- Restriction on guardband size for SBFD:

  - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guardband.

    - The declaration of guardband size should ensure the benefits from SBFD operation.

      - In the RAN4 specification, the clause “It is expected that guardband size should be minimized to ensure the benefits from SBFD operation.” could be captured, and the detailed wording can be further refined.

From a standard point of view, in order to guarantee the benefits of introducing SBFD, there should be a limit on the maximum number of RBs for the guardband. This limit can be a declaration in the RAN4 specification, and the declaration can be “The benefits from SBFD operation are achievable with the declared guardband size.” .

Proposal 4: The declaration of guardband size should ensure the benefits from SBFD operation, and it can be “The benefits from SBFD operation are achievable with the declared guardband size.” in the RAN4 specification.

The position of guardband has not been discussed before, for each SBFD subband configuration, there may be the following three options for guardband position. From a standard point of view, all these positions should be part of the declaration, i.e., declare a single fixed guardband position for each configuration, or only one position common for all configurations.

Option1： All guardband is confined in DL subband

Option2： All guardband is confined in UL subband

Option3： Half guardband is confined in DL subband, and the other half is confined in UL subband

Proposal 5: For each SBFD subband configuration, there may be the following three options for guardband position. All these positions should be part of the declaration, i.e., declare a single fixed guardband position for each configuration, or only one position common for all configurations.

Option1： All guardband is confined in DL subband

Option2： All guardband is confined in UL subband

Option3： Half guardband is confined in DL subband, and the other half is confined in UL subband

2.2.4 Restriction on DL subband size

Issue 1-2-4: Restriction on DL subband size

Agreement:

- If the SSB symbol is not overlapped with SBFD symbol,

  - There is no restriction on DL subband size.

- If the SSB symbol is overlapped with SBFD symbol,

  - FFS the impact from RAN4 perspective.

If the SSB symbol is overlapped with SBFD symbol, DL subband should include the whole DL initial BWP.

Proposal 6: DL subband should include the whole DL initial BWP if the SSB symbol is overlapped with SBFD symbol.

- Conclusion

This contribution provides our further analysis for SBFD general issues. We have the following proposals.

Proposal 1: About declaration and flexibility for SBFD operation, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm.

Proposal 2: About restriction on the number of UL subband sizes supported, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm.

Proposal 3: About restriction on the number of DL subband sizes supported, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm.

Proposal 4: The declaration of guardband size should ensure the benefits from SBFD operation, and it can be “The benefits from SBFD operation are achievable with the declared guardband size.” in the RAN4 specification.

Proposal 5: For each SBFD subband configuration, there may be the following three options for guardband position. All these positions should be part of the declaration, i.e., declare a single fixed guardband position for each configuration, or only one position common for all configurations.

Option1： All guardband is confined in DL subband

Option2： All guardband is confined in UL subband

Option3： Half guardband is confined in DL subband, and the other half is confined in UL subband

Proposal 6: DL subband should include the whole DL initial BWP if the SSB symbol is overlapped with SBFD symbol.

- References

[1] R4-2504751, “Way Forward for [114bis][306] NR_duplex_evo_General”, Samsung, RAN4#114bis