3GPP TSG-RAN WG4 Meeting # 115                                                              R4-2506007

Malta, MT, 19th – 23rd May, 2025

Title: 	Discussion on modification of existing Tx requirements for SBFD operation

Source: 	CATT

Agenda item:	7.25.3.2

Document for:	Discussion

- Background

WF [1] of RAN4#114bis concluded that further studies are needed for some TX RF requirements. This contribution provides our further analysis of the requirements.

- Impact on existing BS Tx RF Requirement

  - Deployment scenarios for FR1 SBFD macro BS

Issue 1-1: Deployment scenarios for FR1 SBFD macro BS

- Way forward

  - Encourage companies to provide more inputs on whether SBFD-SBFD co-ex case should be in Rel-19 scope.

  - If it is in the scope of Rel-19, companies are also encouraged to provide input on whether/how to derive the additional requirements. FFS aspects that may need to be taken into account include:

    - Expected emissions and blocking levels, which could be estimated by means of considering, e.g.:

      - Option 1: Potential expected coupling loss and total receiver desensitization (considering self-interference, co-channel interference and adjacent operator interference) considering relevant site deployment scenarios

      - Option 2: Co-existence simulation and potentially analysis of co-located scenarios

    - Expected spectrum allocation and sub-band sizes

  - Expected feasible filtering performances considering the above two aspects

In [R4-2501026], we agreed with 1dB degradation of victim BS sensitivity and also derived the best UL/DL spectrum configuration for two SBFD-capable BSs.

When two SBFD-capable BSs with adjacent operating bands configured as {DU}, and two DL sub-bands are adjacent, frequency offset between aggressor channel to victim channel is max, and this configuration can optimize the rejection at victim BS UL sub-band by RF BPF used after PA, as shown in figure 1. The best configuration is not a general requirement, but can be used for some scenario.

Figure 1 Interference rejection diagram

If channel bandwidth is 100MHz, the DL sub-band widths of two BSs are both 80MHz and UL sub-band width of two BSs are both 20MHz, when the two DL sub-bands are adjacent, victim BS UL sub-band is 80MHz offset to the edge of aggressor BS passband, the BPF after PA can provide 70dB suppression.

Proposal 1: We agree with 1dB degradation of victim BS sensitivity.

Proposal 2: For two co-located SBFD-capable BSs with adjacent operating bands, the best configuration is both BSs configured as {DU}, and two DL sub-bands are also adjacent, and this configuration does not require the introduction of additional ACLR and/or OBUE requirements for FR1 WA BS.

The UL and DL spectrum configurations between two co-located base stations need to be coordinated, so the best configuration can be achieved by same operator, but not by different operators. There is no need to define requirements for best configurations and scenarios.

Proposal 3: There is no need to define requirements for specific configuration and scenario in Rel -19.

If needed, the SBFD-SBFD adjacent operator scenario can be considered in a future WI, which will address necessary emissions and blocking requirements of this scenario.

Proposal 4: If needed, the SBFD-SBFD adjacent operator scenario can be considered in a future WI.

  - EVM

Issue 1-2: Joint measurement for normal DL symbols/slots and SBFD symbols/slots

- Way forward

  - Further discuss on the following aspects in the performance WI

    - Whether the following two slots configured with flexible symbols could be used for EVM measurement.

      - Flexible symbols per slot is configured with both SBFD symbols and guard period symbols or

      - Flexible symbols per slot is configured with SBFD symbols, guard period and also uplink symbols.

    - Separately measure the averaged EVM for SBFD and non-SBFD, and the averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots.

    - Use the following text proposal as starting point.

EVM is a Tx requirement, when calculating average EVM for SBFD slots, only the data from SBFD DLs within the SBFD slots need to be taken into account, and data from SBFD ULs needn’t to be included.

Proposal 5: When calculating average EVM for SBFD slots, only the data from SBFD DLs within the SBFD slots need to be taken into account.

- Summary

In this contribution, we have the following proposals.

Proposal 1: We agree with 1dB degradation of victim BS sensitivity.

Proposal 2: For two co-located SBFD-capable BSs with adjacent operating bands, the best configuration is both BSs configured as {DU}, and two DL sub-bands are also adjacent, and this configuration does not require the introduction of additional ACLR and/or OBUE requirements for FR1 WA BS.

Proposal 3: There is no need to define requirements for specific configuration and scenario in Rel -19.

Proposal 4: If needed, the SBFD-SBFD adjacent operator scenario can be considered in a future WI.

Proposal 5: When calculating average EVM for SBFD slots, only the data from SBFD DLs within the SBFD slots need to be taken into account.

- Reference

[1] R4-2504752, “Way Forward for [114bis][307] NR_duplex_evo_BSRF”, Huawei, RAN4#114bis

[2] R4-2501026, “Discussion on modification of existing Tx requirements for SBFD operation”, CATT, RAN4#114

[3] R4-2502380, “Way Forward for [114][307] NR_duplex_evo_BSRF”, Huawei, RAN4#114