3GPP TSG-RAN WG4 Meeting # 115                                                              R4-2506009

Malta, MT, 19th – 23rd May, 2025

Title: 	Discussion on potentially new requirements for SBFD operation

Source: 	CATT

Agenda item:	7.25.3.1

Document for:	Discussion

- Background

WF [1] of RAN4#114bis concluded that further studies are needed for potentially new RF requirements, in this contribution, we provide our further analysis of them.

- New requirements for SBFD operation

In WF [1], some in-channel new requirements need further analysis.

2.1 In-channel adjacent sub-band leakage requirement

- Sub-topic 4-3: In-channel adjacent subband leakage

Agreement:

- The requirement (In-channel adjacent subband leakage ratio) is renamed as “in-channel adjacent subband leakage requirement” (by removing “ratio”).

Issue 4-3-2: Assumption for in-channel adjacent subband leakage requirement

Way forward:

- FFS whether/how this assumption can be captured in the RAN4 requirement for in-channel adjacent subband leakage.

Issue 4-3-3: Absolute limit for in-channel adjacent subband leakage

Way forward:

- Absolute limit for in-channel adjacent subband leakage:

  - FFS by considering the following exemplary cases by performing analysis including measurement or simulation:

    - FR1:

      - DU/UD: 80MHz (DL) – 20MHz (UL)

      - DUD: 40MHz (DL) – 20MHz (UL) – 40MHz (DL)

      - Other cases are not precluded

    - FR2-1:

      - DU/UD: 50MHz (DL) – 50MHz (UL); 100MHz (DL) – 100MHz (UL)

      - DUD: 75MHz (DL) – 50MHz (UL) – 75MHz (DL)

      - Other cases are not precluded

  - Note: The study on different configuration does not mean RAN4 will define separate requirements for different configuration. RAN4 still try to use a unified requirements for absolute limit.

For {DU/UD} and {DUD} configurations, the value difference of the nonlinear products falling into the UL subband is relatively small, so the interference signal can be analyzed according to the same ACLR. The ACLR value can be decided based on feasibility analysis and simulations in SI stage. According to the required ACLR in SI stage, the absolute limit for in-channel adjacent subband leakage the Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104 can be reused.

Proposal 1: For absolute limit for in-channel adjacent subband leakage, the table 6.6.3.2-2 in TS 38.104 for FR1 and table 9.7.3.3-2 for FR2 in TS 38.104 can be reused.

2.2 In-channel adjacent subband blocking requirement

Issue 4-4-1: In-channel adjacent subband blocking requirement

Way forward:

- New requirement for in-channel adjacent subband blocking:

  - The requirement can be derived by considering co-site inter-sector case and/or inter-site case

    - If co-site inter-sector case is considered, FFS whether/how the assumption of isolation is reflected in specification.

  - The range of [-35dBm to -27dBm] can be used as the starting point for further study.

Issue 4-4-2: Assumption of gNB-to-gNB CLI handling

Way forward:

- FFS the assumption of gNB-to-gNB CLI handling for two cases:

  - Inter-site case:

    - Note: for the blocking requirement, RX digital beamforming cannot be considered.

  - Co-site inter-sector case:

    - Note: Inter-sector is near field interaction.

The in-channel adjacent subband blocking requirement needs to consider co-site inter-sector case and inter-site case.

The in-channel adjacent sub-band blocking requirement should be defined based on simulation result, -32dBm can be used for MR BS and -34dBm can be used for LA BS.

When consider interference from inter-sectors BSs, the isolation between two sectors need to be decided, but at last meeting, companies had different recommendations on the value of the sector isolation, and it may be difficult to reach a consensus. We can assume an isolation value between two sectors, and then define the in-channel adjacent sub-band blocking requirement based on the isolation assumption. The isolation between two sectors can be assumed to be 70dB.

Proposal 2: The in-channel adjacent sub-band blocking requirement should consider the interference from inter-sectors BSs and inter-site BSs.

Proposal 3: For in-channel adjacent sub-band blocking requirement, -32dBm can be used for MR BS and

-34dBm can be used for LA BS.

Proposal 4: The isolation between two sectors can be assumed to be 70dB.

- Summary

In this contribution, we have the following proposals.

Proposal 1: For absolute limit for in-channel adjacent subband leakage, the table 6.6.3.2-2 in TS 38.104 for FR1 and table 9.7.3.3-2 for FR2 in TS 38.104 can be reused.

Proposal 2: The in-channel adjacent sub-band blocking requirement should consider the interference from inter-sectors BSs and inter-site BSs.

Proposal 3: For in-channel adjacent sub-band blocking requirement, -32dBm can be used for MR BS and -34dBm can be used for LA BS.

Proposal 4: The isolation between two sectors can be assumed to be 70dB.

Reference

[1] R4-2504751, “Way Forward for [114bis][306] NR_duplex_evo_General”, Samsung, RAN4#114bis