3GPP TSG-RAN WG4 Meeting # 115                                                              R4-2506012

Malta, MT, 19th – 23rd May, 2025

Title: 	                 Discussion on A-IoT BS requirements

Source: 	CATT

Agenda item:	7.26.3.1

Document for:	Discussion

- Introduction

This contribution provides our further analysis for A-IoT BS Tx and Rx requirements according to the WF [1].

- A-IoT BS RF requirements analysis

2.1 General

In WID [2], scenario is D1T1-B, R2D is located in DL spectrum and D2R and CW are located in UL spectrum. Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. In this paper, the NR UEs distributed only outdoor.

2.2 A-IoT BS co-located with CW node analysis

If A-IoT BS and CW node are co-located, and the CW node works with other devices and other A-IoT BS, if the A-IoT BS receiver and the co-located CW node operate simultaneously, assume the coupling loss between them is 30dB, then the CW interference signal level at the A-IoT BS input will be large and it will have a great impact on the Rx blocking requirement of A-IoT BS receiver, even will block the A-IoT BS receiver. In addition, in the simulation of D1T1-B case in the SI phase, for each activated device, the nearest CW node will be activated during the simulation, and CW node is not co-located with any activated reader. So the scenario in which A-IoT BS receiver and the co-located CW node operate simultaneously is not recommended.

Proposal 1: CW nodes communicating with active devices can’t be co-located with any active readers.

2.3 Tx requirements

In WF [1], the Tx RF requirements for A-IoT BS in table 1 need to be further studied.

Table 1: Table 1: Tx RF requirements for A-IoT BS

| RF Requirement for A-IoT BS- TX requirements |  |
| --- | --- |
| Transmit signal quality | EVM |
| Unwanted emissions | Occupied bandwidth (OBW) |
|  | Adjacent Channel Leakage Power Ratio (ACLR) |
|  | Operating band unwanted emissions (OBUE) |
| Transmitter intermodulation |  |

2.3.1 Transmit signal quality

R2D signal is ook-4 modulation based on OFDM, and we can define envelope requirements similar to RFID, including modulation depth, RF envelope ripple, RF envelope rise time, RF envelope fall time and RF pulse. It is necessary to consider the differences between R2D signal and RFID signal, including modulation method, encoding method and waveforms, etc.

EVM is another specification that expresses signal modulation quality and does not need to be defined at the same time as envelope requirement. If EVM is introduced, it can be defined as 17.5% at on chip only.

Proposal 2: Either RF envelope or EVM requirements can be defined for R2D signal.

Proposal 3: If EVM is introduced, it can be defined as 17.5% at on chip only.

2.3.2 Unwanted emissions

2.3.2.1 Occupied bandwidth

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 (=0.5%) of the total mean transmitted power.

The same principle with existing NR (99% power should be within BS channel bandwidth) can be reused, and the minimum requirement for A-IoT BS occupied bandwidth shall be less than the A-IoT BS channel bandwidth.

Proposal 4: The same principle with existing NR can be reused, and the minimum requirement for A-IoT BS occupied bandwidth shall be less than the A-IoT BS channel bandwidth.

2.3.2.2 ACLR

Based on the simulation results of scenario 2-1, the ACLR for NB-IoT BS with standalone mode can be reused by A-IoT BS, i.e.ACLR1 is 40dB and ACLR2 is 50dB. For 1PRB transmission BW, if the 100kHz offset is located in the NR spectrum, it can be defined. In this case, the offset has no impact on the A-IoT channel bandwidth. For multiple PRB transmission, whether frequency offset is required needs to be considered.

Proposal 5: The ACLR for NB-IoT BS with standalone mode can be reused by A-IoT BS, i.e.ACLR1 is 40dB and ACLR2 is 50dB.

Proposal 6: For 1PRB transmission BW, if the 100kHz offset is located in the NR spectrum, it can be defined.

Proposal 7: For multiple PRB transmission, whether frequency offset is required needs to be considered.

2.3.2.3 OBUE

The channel bandwidth of standalone NB-IoT BS is 200kHz, and it has guardbands at the low and up ends inside the channel, transmission bandwidth configuration of it is 180kHz, so OBUE requirement for standalone NB-IoT MR BS can be reused for A-IoT BS.

Proposal 8: OBUE requirement for standalone NB-IoT MR BS can be reused for A-IoT BS.

2.3.3 Transmitter intermodulation

Traditionally the transmitter intermodulation requirement was created to guarantee emission levels in a co-location scenario. The requirement is based on robustness against injection of a reverse interference signal from a co-located BS. Scenario in which A-IoT BS receiver and the co-located CW node operate simultaneously is not recommended.  On the other hand, although the A-IoT BS R2D signal use NR BS DL spectrum, but the two BSs are not co-located, so the transmitter intermodulation requirement is not necessary. Even if BSs of other NR system are co-located with A-IoT BSs, and the operating frequency bands of the two co-located BSs are not adjacent, the transmitter intermodulation requirement is also not necessary.

Proposal 9: The A-IoT BS and NR BS are not co-located, so the transmitter intermodulation requirement is not necessary.

Observation 1: Even if BSs of other NR system are co-located with A-IoT BSs, and the operating frequency bands of the two co-located BSs are not adjacent, the transmitter intermodulation requirement is also not necessary.

2.4 Rx requirements

In WF [1], the Rx RF requirements for A-IoT BS in table 2 need to be further studied.

Table 2: Rx RF requirements for A-IoT BS

| A-IoT BS - RX RF Requirement |  |
| --- | --- |
| SNR |  |
| Dynamic range |  |
| Adjacent Channel Selectivity（ACS） |  |
| In-band blocking | In-band blocking |
|  | Narrowband blocking |
| Receiver intermodulation | Receiver intermodulation |
|  | Narrowband intermodulation |
| In-channel selectivity (ICS) |  |

2.4.1 ACS

Based on the simulation results of scenario 2-1, i.e. D1T1-B, NR UEs only distributed outdoor, R2D is located in DL spectrum and CW/D2R are located in UL spectrum, the ACS for NB-IoT standalone Medium Range BS  can be reused for A-IoT BS. The interfering signal centre frequency offset to the lower/upper BS RF Bandwidth edge is ±100kHz, and there is no additional frequency offset of ±100kHz. Therefore, additional frequency offset of ±100kHz is also not required for A-IoT BS.

Proposal 10: The ACS for NB-IoT standalone MR BS can be reused for A-IoT BS.

Proposal 11: For ACS requirement, additional frequency offset of ±100kHz is not required for A-IoT BS.

2.4.2 In-band blocking

In scenario 2-1, i.e. the NR UEs only distributed outdoor, the distances between NR UEs and A-IoT BSs are relatively far, the narrow-band blocking requirement is not necessary.

Proposal 12: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband blocking requirement is not necessary.

2.4.3 Receiver intermodulation

The scenario in which A-IoT BS receiver and the co-located CW node operate simultaneously is not recommended. If A-IoT BS and co-located CW node can’t run at the same time, in scenario 2-1, i.e. the NR UEs only distributed outdoor, the distances between NR UEs and A-IoT BSs are relatively far, receiver intermodulation and narrowband intermodulation requirements are not necessary. Even if BSs of other NR system are co-located with A-IoT BSs, and the operating frequency bands of the two co-located BSs are not adjacent, the interference signal can be considered by blocking requirement.

Observation 2: Even if BSs of other NR system are co-located with A-IoT BSs, and the operating frequency bands of the two co-located BSs are not adjacent, the interference signal can be considered by blocking requirement.

Proposal 13: For D1T1-B scenario and NR UEs only distributed outdoor, the receiver intermodulation requirement is not necessary.

Proposal 14: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband intermodulation requirement is not necessary.

2.4.4 ICS

ICS is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density. In SI simulation, only one active A-IoT device under one reader at one drop, so the ICS requirement is not necessary.

Proposal 15: If the reader activates only one device at a time, ICS requirement is not necessary.

- Summary

This contribution provides our further analysis for A-IoT BS requirements. We have the following proposals and observations.

Proposal 1: CW nodes communicating with active devices can’t be co-located with any active readers.

Proposal 2: Either RF envelope or EVM requirements can be defined for R2D signal.

Proposal 3: If EVM is introduced, it can be defined as 17.5% at on chip only.

Proposal 4: The same principle with existing NR can be reused, and the minimum requirement for A-IoT BS occupied bandwidth shall be less than the A-IoT BS channel bandwidth.

Proposal 5: The ACLR for NB-IoT BS with standalone mode can be reused by A-IoT BS, i.e.ACLR1 is 40dB and ACLR2 is 50dB.

Proposal 6: For 1PRB transmission BW, if the 100kHz offset is located in the NR spectrum, it can be defined.

Proposal 7: For multiple PRB transmission, whether frequency offset is required needs to be considered.

Proposal 8: OBUE requirement for standalone NB-IoT MR BS can be reused for A-IoT BS.

Proposal 9: The A-IoT BS and NR BS are not co-located, so the transmitter intermodulation requirement is not necessary.

Observation 1: Even if BSs of other NR system are co-located with A-IoT BSs, and the operating frequency bands of the two co-located BSs are not adjacent, the transmitter intermodulation requirement is also not necessary.

Proposal 10: The ACS for NB-IoT standalone MR BS can be reused for A-IoT BS.

Proposal 11: For ACS requirement, additional frequency offset of ±100kHz is not required for A-IoT BS.

Proposal 12: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband blocking requirement is not necessary.

Observation 2: Even if BSs of other NR system are co-located with A-IoT BSs, and the operating frequency bands of the two co-located BSs are not adjacent, the interference signal can be considered by blocking requirement.

Proposal 13: For D1T1-B scenario and NR UEs only distributed outdoor, the receiver intermodulation requirement is not necessary.

Proposal 14: For D1T1-B scenario and NR UEs only distributed outdoor, the narrowband intermodulation requirement is not necessary.

Proposal 15: If the reader activates only one device at a time, ICS requirement is not necessary.

Reference

[1] R4-2505097, “WF on A-IoT BS and CW requirements”, Huawei, RAN4#114bis

[2] RP-243326, “Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, RAN#106