3GPP TSG-RAN WG4 Meeting #115                                                                                                         R4-2506017

Malta, MT, 19th –23rd May, 2025

Source: 	CATT

Title: 	Further discussion on BS demodulation performance for MMSE-IRC

Agenda item:	7.17.4

Document for:	Approval

- Introduction

In RAN4#114bis meeting, a Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS was agreed, and the remaining issues on BS demodulation performance for MMSE-IRC in WF require further discussion.

In this contribution, we provide our views on BS demodulation performance for MMSE-IRC.

- Discussion

Manufacturer declaration on MMSE-IRC receiver

As per the WF [1], the agreement concerning manufacturer declaration on MMSE-IRC receiver is shown as below:

Manufacturer declaration on MMSE-IRC receiver - Agreement: - Introduce new manufactory declaration for BS supporting enhanced performance requirements. - Note: MMSE-IRC as specified in Rel-19 may have been widely deployed in the earlier release BS products. (This note is not going to be captured in the specification or technical report)

As above, it is agreed to introduce new manufactory declaration for BS supporting enhanced performance requirements, but how to introduce manufactory declaration for BS supporting enhanced performance requirements is missing. It is needed to add manufacturer declarations for MMSE-IRC receiver for BS type 1-C and BS type 1-H conducted test requirements in Table 4.6-1 of TS 38.141-1 as below:

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| [D.126] | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

And it is also needed to add manufacturer declarations for MMSE-IRC receiver for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements in Table 4.6-1 of TS 38.141-2 as below:

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| [D.129] | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | c | x | n/a |

Based on above, we propose,

Proposal 1: Add manufacturer declarations for MMSE-IRC receiver for BS type 1-C and BS type 1-H conducted test requirements in Table 4.6-1 of TS 38.141-1 as below:

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| [D.126] | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

Proposal 2: Add manufacturer declarations for MMSE-IRC receiver for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements in Table 4.6-1 of TS 38.141-2 as below:

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| [D.129] | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | c | x | n/a |

Applicability for different BS classes and test scenarios

As per the WF [1], the agreement and remaining open issue concerning applicability for difference BS classes and test scenarios is shown as below:

Applicability for different BS classes and test scenarios - Online agreement: - Follow the LTE approach – Only specifies that HomNet requirements are not applicable for Local Area BS. - FFS whether to consider additional applicability rule on how the test is performed: - Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped - Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

From agreement in WF [1] in RAN4#114bis, the TDLC300-100 is defined as propagation condition for HomNet, and TDLA30-10 is defined as propagation condition for HetNet. The delay of TDLC is larger than TDLA, and 300ns delay spread of TDLC300-100 is larger than 30ns delay spread of TDLA30-10, at same time, 100Hz Doppler of TDLC300-100 is larger than 10Hz Doppler of TDLA30-10, so from perspective of propagation, the TDLC300-100 is stringent than TDLA30-10, then, propagation condition of HomNet is worse than that of HetNet, so, in this case, if a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be passed,

From agreement in WF [2] in RAN4#112bis, the INR set1: (5.35.N/A) and (7.28.-2.52) are defined for HomNet, and INR set2: (9.82. NA/) and (12.38. -0.97) are defined for HetNet. The above INR sets show that I+N value for HomNet is less than that for HetNet, which seems to indicate that option 1 is not valid. But the interference level just impact on the SNR @ 70% of maximum throughput, and the TDLC300-100 is defined as propagation condition for interferers for HomNet and HetNet, so we think that the INR sets can’t be considered as factor for choosing Option1 and Option 2, propagation condition for serving UE as factor for choosing Option1 and Option 2 is sufficient.

Based on above analysis, since propagation condition of HomNet is worse than that of HetNet, so, it is reasonable to define that if a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be passed. So we propose,

Proposal 3: Adopt Option 1 as additional applicability rule for BS MMSE-IRC receiver as below:

- Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped

- Conclusion

This contribution provides analysis on NR BS MMSE-IRC demodulation. The following proposals and observations are concluded as follows:

Proposal 1: Add manufacturer declarations for MMSE-IRC receiver for BS type 1-C and BS type 1-H conducted test requirements in Table 4.6-1 of TS 38.141-1 as below:

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| [D.126] | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

Proposal 2: Add manufacturer declarations for MMSE-IRC receiver for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements in Table 4.6-1 of TS 38.141-2 as below:

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| [D.129] | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | c | x | n/a |

Proposal 3: Adopt Option 1 as additional applicability rule for BS MMSE-IRC receiver as below:

- Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped

- References

- R4-2504768, Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS, China Telecom, RAN4#114bis

- R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, China Telcom, RAN4#112bis

- R4-2502272, Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS, China Telecom, RAN4#114

- R4-2503453, Simulation results for BS requirement with MMSE_IRC receiver, CATT, RAN4#114bis