3GPP TSG-RAN WG4 Meeting #114bis                                                                                                      R4-2506020

Malta, MT, 07th –11st April, 2025

Source: 	CATT

Title: 	Further discussion on coexistence evaluations for Ku-band for NR NTN

Agenda item:	7.11.3

Document for:	Approval

- Introduction

In RAN4# 114bis meeting, a Way Forward for [114bis][313] NR_NTN_Ku_Band_Coexistence was agreed.

In this contribution, we provide our views on some coexistence evaluation for Ku-band for NR NTN.

- Discussion

Equivalent satellite antenna aperture and gain for SAN GEO

As per the WF [1], the agreement concerning equivalent satellite antenna aperture and gain for SAN GEO for Ku-band NTN is shown as below:

Following Table 2 illustrates agreed LEO and GEO Satellite parameters: Table 2 Satellite Parameters Satellite orbit; GEO; LEO-600. Satellite altitude; 35786 km; 600 km. Satellite antenna pattern; Section 6.4.1 in [2]; Section 6.4.1 in [2]. Payload characteristics for DL transmissions. Equivalent satellite antenna aperture (Note 1); Ku-band (i.e. 11 GHz for DL); 4 m; 0.91 m. Satellite EIRP density; 43 dBW/MHz; 7 dBW/MHz. Satellite Tx max Gain; 43 dBi; 38.5 dBi. 3dB beamwidth; 0.400 deg; 1.768 deg. Satellite beam diameter (Note 2); 250 km; 19 km. Payload characteristics for UL transmissions. Equivalent satellite antenna aperture (Note1); Ku-band (i.e. 14 GHz for UL); 4 m; 0.714 m. G/T; 16.5 dB K-1; 10.9 dB K-1. Satellite RX max Gain; 44 dBi; 38.5 dBi. NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB..

The antenna gain is dependent on 3dB beamwidth, and 3dB beamwidth is dependent on frequency(λ) and antenna diameter (D) as following equation.

2θ-3dB=2×Arcsin1.616×λπD                                                                                                                       (Eq.1)

For satellite antenna parameter, we can use 58.5 dBi for 20GHz and 0.1768degree 3dB beamwidth as reference, the antenna gain for other frequency and antenna diameter can be calculated by following Eq.2.

Gf1=Gf0-20log10⁡2θ-3dBf12θ-3dBf0                                                                                                                        (Eq.2)

Where,

Gf0=58.5

2θ-3dBf0=0.1768 degree, which is calculated by Eq.1 with frequency is 20GHz and antenna diameter is 5m.

2θ-3dBf1 can be calculated by Eq.1 for other frequency and antenna diameter.

Using above Eq.2 and Eq.1, the antenna gain and antenna diameter for parabolic Antenna are shown in the following table.

| Frequency range | Frequency (GHz) | Antenna diameter（m) | 3dB beamwidth (degree) | Gain (dBi) |
| --- | --- | --- | --- | --- |
| Ka-band | 20 | 5 | 0.1768 | 58.5 |
|  | 30 | 3.33 | 0.177 | 58.5 |
|  | 20 | 0.5 | 1.7684 | 38.5 |
|  | 30 | 0.333 | 1.7702 | 38.5 |
| Ku-band | 11 | 9.1 | 0.1767 | 58.5 |
|  | 14 | 7.14 | 0.1769 | 58.5 |
|  | 11 | 0.91 | 1.7666 | 38.5 |
|  | 14 | 0.714 | 1.7691 | 38.5 |
|  | 11 | 4 | 0.4019 | 51.4 |
|  | 14 | 3.14 | 0.4023 | 51.4 |

As above table, for the 4m parabolic Antenna with 0.400 degree 3dB beamwidth, its antenna gain is 51.4dBi instead of 43dBi. For parabolic antenna, if the same antenna is used for DL and UL transmission, the equivalent satellite antenna aperture for DL and UL transmission should be different. Equivalent satellite antenna aperture for UL should be 3.14m.

The G/T in dB can be calculated using following equation:

G/T = Ga – NF – 10*LOG (To)                                                                                        (Eq.1)

Where:

-	Antenna Gain: Ga in dBi

-	Ambient Temperature: T0 (usually 290 K)

-	Noise Figure: NF in dB.

RAN4 agreed that NF for SAN for Ku-band is 3dB. So for 51.4dBi Rx gain, the G/T for Ku-band GEO can be calculated as 23.8 dB K-1.

Based on above discussion, the satellite parameter in WF[1] should be updated as in following table.

| Satellite orbit |  |  | GEO | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 600 km |
| Satellite antenna pattern |  |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) |  | Ku-band (i.e. 11 GHz for DL) | 4 m | 0.91 m |
| Satellite EIRP density |  |  | 43 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  |  | 51.4 dBi | 38.5 dBi |
| 3dB beamwidth |  |  | 0.400 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  |  | 250 km | 19 km |
|  | Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) |  | Ku-band (i.e. 14 GHz for UL) | 3.14 m | 0.714 m |
| G/T |  |  | 23.8 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  |  | 51.4 dBi | 38.5 dBi |
|  | NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |

Based on above analysis, we propose the

Proposal 1: Adopt the following satellite Parameters for Ku-band co-existence.

Table 2 Satellite Parameters

| Satellite orbit |  |  | GEO | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 600 km |
| Satellite antenna pattern |  |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) |  | Ku-band (i.e. 11 GHz for DL) | 4 m | 0.91 m |
| Satellite EIRP density |  |  | 43 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  |  | 51.4 dBi | 38.5 dBi |
| 3dB beamwidth |  |  | 0.400 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  |  | 250 km | 19 km |
|  | Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) |  | Ku-band (i.e. 14 GHz for UL) | 3.14 m | 0.714 m |
| G/T |  |  | 23.8 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  |  | 51.4 dBi | 38.5 dBi |
|  | NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |

- Conclusion

This contribution provides analysis on coexistence assumptions for Ku-band for NR NTN, and the following proposal is concluded as follows:

Proposal 1: Adopt the following satellite Parameters for Ku-band co-existence.

Table 2 Satellite Parameters

| Satellite orbit |  |  | GEO | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 600 km |
| Satellite antenna pattern |  |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) |  | Ku-band (i.e. 11 GHz for DL) | 4 m | 0.91 m |
| Satellite EIRP density |  |  | 43 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  |  | 51.4 dBi | 38.5 dBi |
| 3dB beamwidth |  |  | 0.400 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  |  | 250 km | 19 km |
|  | Payload characteristics for UL transmissions |  |  |  |
| Equivalent satellite antenna aperture (Note1) |  | Ku-band (i.e. 14 GHz for UL) | 3.14 m | 0.714 m |
| G/T |  |  | 23.8 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  |  | 51.4 dBi | 38.5 dBi |
|  | NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |

- References

- R4-2504723, Way Forward for  [114bis][313] NR_NTN_Ku_Band_Coexistence, Moderator: Intelsat, RAN4#114bis

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#114

- R4-2502315, Co-existence simulation assumptions for NR NTN Ku band (Running document), Ericssion, RAN4#114

- R4-2419909, Way Forward for [112][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#113

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112bis

- R4-2416563, Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#112bis

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#106

- R4-2413525, Way Forward for [112][315] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#112

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104