3GPP TSG-RAN WG4 Meeting #115                                                                                                      R4-2506024

Malta, MT, 19th –23rd May, 2025

Source: 	CATT

Title: 	Discussion on UE RF requirements for RedCap PC2 FDD

Agenda item:	7.6.2

Document for:	Approval

- Introduction

In RAN4# 114bis meeting, A WF [1] on NR PC2 RedCap UE in FDD bands was agreed and some remaining issues in WF need further discussion.

So, in this contribution, we provide our views on UE RF requirements for RedCap PC2 UEs for FDD bands.

- Discussion

REFSENS for 1Rx in FD-FDD for PC2 RedCap UE

As per the WF [1], the remaining open issue concerning REFSENS for 1RX in FD-TDD for PC2 RedCap UE is shown as below:

Issue 1-1-1: REFSENS for 1Rx in FD-FDD Agreement: Consider the following formats for specifying REFSENS for 1Rx PC2 RedCap in FD-FDD - Option1: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on RSD ranges - Flat 3dB ΔR 1R for all CBW no larger than 20MHz or use values for FDD bands in table 7.3I.2-1 for bands whose PC2 1Tx/2RX RSD is 0dB. - Companies to evaluate REFSENS for other bands. - The format in which the REFSENS is shown in the specs is TBD pending evaluations. - Option2: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R may be specified based on the following conditions - ΔR 1R value may be considered from LTE TS 36.101 Table 7.3.1E-1A for bands and CBW which are already defined in TS 36.101 Table 7.3.1E-1A. The ΔR 1R value is calculated by comparing the REFSESN specified in TS 36.101 Table 7.3.1E-1A with TS 36.101 Table 7.3.1. - Companies are encouraged to bring measurement based evaluation for ΔR1R for other bands and CBW which are not covered by LTE TS 36.101 Table 7.3.1E-1A. - The format in which the REFSENS is shown in the specs is TBD pending evaluations. Way forward: It is encouraged for companies to provide analysis for option 1 and 2. Note: Companies are encouraged to evaluate all specified PC2 FDD bands. The WI is intended to be completed in May meeting according to WI plan.

In RAN4#114 meeting [2], the 2Rx RSD for 1Tx PC2 for RedCap UE is agreed as below:

Issue 1-2-2: REFSENS for 2Rx in FD-FDD Online agreement: - 2Rx RSD for 1Tx PC2 is provided for non-Redcap UE in Table 7.3.2-1c in TS38.101-1, and this requirement could apply to RedCap UE for channel BWs up to 20MHz.

As above, RAN4 agreed the 2Rx RSD for 1Tx PC2 provided for non-Redcap UE in Table 7.3.2-1c in TS38.101-1 for channel BWs up to 20MHz can apply to RedCap UE, so the REFSENS (1Tx (26dBm) /2Rx) in Option 1 and Option 2 are same, and main difference between Option 1 and Option 2 is value of ΔR1R.

For Option 2, the REFSENS for band 3 in Table 7.3.1-1, Table 7.3.1E-1A are copied as below:

Table 7.3.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 3 | -101.7 | -98.7 | -97 | -94 | -92.2 | -91 | FDD |

Table 7.3.1E-1A: Reference sensitivity for FDD and TDD UE category 0 QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 3 | -99.2 | -96.2 | -94.5 | -91.5 | -89.7 | -88.5 | FDD |

By subtracting REFSENS in Table 7.3.1-1 from REFSENS in Table 7.3.1E-1A for each channel bandwidth, we archive the ΔR1R value for each channel bandwidth is in following table.

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 3 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | FDD |

As above, the ΔR1R value is 2.5dB for each channel bandwidth up to 20MHz, so ΔR1R value for Option 2  is flat 2.5dB ΔR 1R for all CBW no larger than 20MHz.

For Option 1, the ΔR1R value for FDD bands in table 7.3I.2-1 is copied as below:

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3.0 |

As above, the ΔR1R value for FDD bands in table 7.3I.2-1 is band agnostic and is not flat to for all CBW no larger than 20MHz.

We summarize the ΔR1R value in Option 1(in table 7.3I.2-1), Option 1(Flat 3dB) and Option 2 in the following table.

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Option 1a(in table 7.3I.2-1) | Option 1b(Flat 3dB) | Option 2 |
| FDD band | 5 | 2.5 | 3.0 | 2.5 |
| FDD band | 10, 15, 20 | 3.0 | 3.0 | 2.5 |

From above table, the option 2 is stricter, Option 1b(Flat 3dB) is relatively relaxed, and Option 1a(in table 7.3I.2-1) is defined for PC3 RedCap UE. For PC2 RedCap UE, the Option 1(in table 7.3I.2-1) is more suitable.

So, we propose,

Proposal 1: Adopt Option1: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R use values for FDD bands in table 7.3I.2-1.

Base on above proposals, a draft CR on power class 2 RedCap for TDD bands and FDD bands for TS 38.101-1 is provided for endorsement.

- Conclusion

This contribution provides analysis on UE RF requirements for RedCap PC2 UEs for FDD bands, and the following proposal is concluded as follows:

Proposal 1: Adopt Option1: REFSENS (1Tx (26dBm) /2Rx) + ΔR1R : ΔR1R use values for FDD bands in table 7.3I.2-1.

Base on above proposals, a draft CR[5] on on power class 2 RedCap for TDD bands and FDD bands for TS 38.101-1 is provided for endorsement.

- References

- R4-2505231, WF on PC2 RedCap UE RF requirements, China Telecom, RAN4#114bis

- R4-2502874, WF on NR PC2 RedCap UE in FDD bands, China Telecom, Skyworks, ZTE, Qualcomm, CATT, Murata, Ericsson, Huawei, Apple, RAN4#114

- RP‑243267, Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom, RAN#106

- R4-2420357, (NR_PC2_RedCap_UE) CR to 38.101-1: PC2 Redcap support for TDD bands, Ericsson, RAN4#113

- R4-2506025, Draft CR for 38.101-1: Introduction on power class 2 RedCap for TDD bands and FDD bands, CATT, RAN4#114bis.