3GPP TSG-RAN WG4 Meeting #115	                                                                                            R4-2506033

Malta, MT, 19 May. – 23 May, 2025

Source: 	CATT

Title: 	Discussion on Ambient IoT device 1 RF requirements

Agenda item:	7.26.3.2

Document for:	Approval

- Introduction

In RAN4 #114 -bismeeting, a WF [1] on RF requirements for A-IoT was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

This contribution provides our analysis for device 1 RF requirements based on the discussion in previous meeting.

- Discussion

  - Tx requirements

The open issues on RF requirements for A-IoT were discussed during the previous meetings, as shown below:

## Topic 3-1: Transmit output power Issue 3-1-1: Transmit output power Agreement: - Performance metric of transmit output power: - Backscattering loss - Backscattering output power is defined as the minimum backscattered power of Device 1 at certain input CW Tx power - Minimum Backscattering power at device antenna= Input CW Tx power at device antenna-Backscattering loss - For the Input CW Tx power to define requirements: - FFS on the whether to use one value or a range of CW Tx power - FFS on the value(s) of CW Tx power - Consider following aspects to determine backscattering loss to define requirements: - modulation loss - antenna efficiency - backscattering circuitry loss - impairment margin - minimum distance to the reader - differential radar cross section in two different modulation states - differential reflection coefficient in two different modulation states - the spectral components of the backscatter signal to be included in the power measurement, e.g. only the 1st Lower SB and 1st Upper SB, excluding the reflected carrier itself - Other factors not excluded ## Topic 3-2: Modulation quality Issue 3-2-1: SFO requirement Agreement: - 10% SFO is assumed as the minimum performance for devices. - FFS on whether to define SFO requirement and how to test the SFO requirement if defined. Issue 3-2-2: Modulation quality requirements Agreement: - Define modulation quality requirements for both BPSK and OOK. - Consider to define same modulation quality metric for both BPSK and OOK if possible. - Reference measurement process needs to be discussed ## Topic 3-3: Emission requirements Issue 3-3-1: Emission requirements Agreement: - Discuss whether to define EIPR or TRP as performance metric for emission requirements - Spurious emission: reuse legacy NR UE spurious emission requirement and further discuss the OOB boundary. - FFS on CW maximum input power level for testing ## Topic 3-4: Reference sensitivity Issue 3-4-1: Reference sensitivity Agreement: - Define minimum requirements of REFSENS based on link budget (target is [15m]) and measurement results reported by companies - Consider to use inventory procedure for REFSENS testing, success rate ([90%]) or miss detection rate ([10%]) as the metric for inventory - At least to define maximum gain direction sensitivity requirements - FFS on how to different sensitivity requirements on different orientations ## Topic 3-5: Others Issue 3-5-1: Maximum input power Agreement: - Define maximum input level, consider following options: - Option 1: using [1m] distance assumption from reader to device - Option 2: using square law appliance range - Option 3: using dynamic range of device receiver Issue 3-5-2: Occupied bandwidth Agreement: Further discuss whether to define occupied bandwidth

Since A-IoT Device 1 will operate in backscatter mode, the term “transmit output power” does not accurately describe the Tx output power requirements. Considering that there is no transmitter in Device 1, the term “transmit” is not appropriate, and for aligning with the 3GPP style, we believe that the term Backscattering power could be a good choice.

Proposal 1: The terminology of Device 1 Tx output power requirements could be defined as Backscattering power.

The detail of Tx output power requirements should be discussed, and the CW Tx power should be defined as a range or fixed value was also discussed in previous meeting. The origin of Device Tx signal is come from the CW, hence for device 1 the output signal is generated by backscattering the signal from the CW.

Considering that RAN4 usually defines minimum requirement, from the device aspect, the backscattering performance will be guarantee when the CW transmitted under maximum power. Therefore we believe that the CW Tx power should be defined as the maximum CW Tx power, i.e. 33dbm. Therefore, the Device 1 radiated power requirement should be defined as the maximum backscattered power of Device 1 at the maximum CW Tx power and at a designated distance.

Proposal 2: Device 1 backscattering power requirement should be defined as the maximum backscattered power of Device 1 at the maximum CW Tx power and at a designated distance.

For modulation quality requirements, according to RAN1#120 meeting agreement, both OOK and BPSK for baseband modulation will be supported modulation scheme for D2R devices, as shown below:

Agreement For a D2R transmission, it is up to device implementation to use OOK or BPSK modulation. It is assumed that a device operates consistently with the same D2R modulation in the same inventory/command round. Agreement For D2R BPSK modulation - After line coding or square wave, chip “1” is mapped to the real-valued modulation symbol {1} and chip “0” is mapped to the real-valued modulation symbol {-1} in the baseband. Agreement For D2R OOK modulation - After line coding or square wave, chip “1” is mapped to the real-valued modulation symbol {1} and chip “0” is mapped to the real-valued modulation symbol {0} in baseband.

Since the modulation quality requirements are mainly used to limit the amplitude and phase downgrade of a signal, we believed that the modulation quality requirements are still required for Device 1.

However, the modulation quality requirements for Device 1 might be slightly different from NR, depending on the modulation scheme. For OOK modulation, EVM measurements can only be conducted when the transmitter is switched ON (no signal is transmitted when the transmitter is switched OFF), so how to apply the EVM requirements to Device 1 may still need to be investigated. Meanwhile, the above problems do not exist for BPSK modulation, so the current NR EVM requirements can be considered as a good starting point. In addition, considering the capability and cost of device 1, the EVM requirements can be appropriately relaxed.

Therefore, we believe that a less stringent EVM requirements than BPSK EVM requirement could be defined for Device 1 (only applies for ON chip).

Proposal 3: For modulation quality, a less stringent EVM requirements than BPSK EVM requirement could be defined for Device 1 (only applies for ON chip).

  - Rx requirements

For maximum input power, this requirement might be related to the maximum coupling loss (MCL). The maximum input power level for device could be calculated by maximum A-IoT BS Tx power minus the MCL, so if the Maximum Input Power requirements need to be discussed, the MCL needs to be determined first.

We noticed that the MCL has been discussed during the coexistence study of ambient IoT SI phase and was captured in TR 38.769. Therefore, we believed that the MCL value in TR 38.769 could be considered as a good reference and starting point.

Proposal 4: The MCL should be discussed first to check if maximum input power requirement is needed. If the MCL needs to be determined, the value of MCL in TR 38.769 could be considered.

- Conclusion

This contribution provides our initial views on A-IoT Device 1 requirements. The following proposals are concluded as follows:

Proposal 1: The terminology of Device 1 Tx output power requirements could be defined as Radiated power.

Proposal 2: Device 1 radiated power requirement should be defined as the maximum backscattered power of Device 1 at the maximum CW Tx power and at a designated distance.

Proposal 3: For modulation quality, a less stringent EVM requirements than BPSK EVM requirement could be defined for Device 1 (only applies for ON chip).

Proposal 4: The MCL should be discussed first to check if maximum input power requirement is needed. If the MCL needs to be determined, the value of MCL in TR 38.769 could be considered.

- References

[1] R4-2505230, “WF on requirements for system parameters and A-IOT device”, RAN4#114-bis, CMCC