3GPP TSG-RAN WG4 Meeting #115                                                                                             R4-2506035

Malta, MT, May. 19 –May. 23, 2025

Source: 	CATT

Title: 	Discussion on UE RF requirements for the NTN new S-band

Agenda item:	6.13.1

Document for:	Approval

- Introduction

In RAN4 #114-bis meeting, a WF [1] on NR NTN support new S-band UE RF requirement was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some views on UE RF requirements for introducing the NTN new S-band.

- Discussion

2.1 UE REFSENS

The agreements of UE REFSENS have been made during RAN4 #114-bis meeting, as shown below:

#### Issue 1-1-3: UE REFSENS - Consider the following table with the REFSENS values for further discussions Operating band / SCS / Channel bandwidth. SCS kHz; 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm). 15; -99.5; -96.3; -94.5; -93.3. 30; -96.6; -94.6; -93.5. 60; -97.0; -94.9; -93.7.

The REFSENS for band n252 NTN UE was also discussed at the last meeting. Based on the information provided by the companies, two candidate options were proposed, but remain controversial. Some companies believe that the REFSENS value of the n256 can be reused because the duplexer in the band n252 may reuse the n256. At the same time, some companies (such as operators) have certain requirements for network performance, so they hope that the REFSENS of UE can achieve better performance.

Considering the suggestion in WID that the work of this WI shall leverage the studies and requirements for NR NTN n256, n255, n254, the duplexer of NTN new S band may also need to consider whether to reuse the design of the existing band. During RAN4 114-bis meeting, the duplexer implementation has been discussed and confirmed that RAN4 does not make any assumptions on specific duplexer implementation. Considering that the operator has put forward a clear requirement for performance, we believe that there might be some room for improvement in the REFSENS of band n252.

Therefore, we believe that REFSENS of NTN new S band may consider the following values:

Table 2.1-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| n252 | 15 | -100 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

Proposal 1: For band n252, the UE REFSENS in table 2.1-1 could be considered.

2.2 Out-of-band Blocking

During RAN4 114-bis meeting, the OOBB requirements were discussed as well. One of the important factors about OOBB is the duplexer implementation. The agreements on duplexer are shown as below:

#### Issue 1-1-4: Duplexer Configuration - Agreement - RAN WG4 does not make any assumption on a specific duplexer implementation - RAN WG4 agrees that the OOB blocking requirements are decoupled from the implementation assumption of UE duplexer. - Capture these considerations in TR 38.863 and TR 36.764

However, the agreements on OOBB are shown as below:

#### Issue 1-1-5: Out-of-band Blocking - Agreement - Take option 5 as a baseline - Do not preclude the implementation supporting only n252 and not supporting n65. - The following out-of-band blocking requirements are agreed (based on option 5 above) Operating Band; Parameter; Unit; Range 1; Range 2; Range 3. Pinterferer; dBm; -44; -30; -15. n252; Finterferer (CW); MHz; -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750.

It is worth noting that option 5, which is currently considered to be the baseline, in fact already reflects a specific duplexer implementation. For duplexer supporting only n252, the current OOBB ranges seem to be a little bit wide, which is likely a range for the previously discussed co-band implementation.

Therefore, similar to the discussion of REFSENS above, we believed that for n252, the UE OOBB requirements should be defined as shown in the table below:

Table 2.2-1: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n252 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| n2542 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

Proposal 2: For band n252, the UE OOBB in table 2.2-1 could be considered.

- Conclusion

This contribution provides our views on UE requirements for introduction of the NTN new S-band. The following proposals are concluded as follows:

Proposal 1: For band n252, the UE REFSENS in table 2.1-1 could be considered.

Proposal 2: For band n252, the UE OOBB in table 2.2-1 could be considered.

- References

- R4-2505235, WF on NTN and IoT-NTN bands, Apple, RAN4 #114-bis