3GPP TSG RAN WG4 Meeting #115		R4-2506047

St Julian, Malta, May 19th – 23rd , 2025

Title: 	Discussion on updating duty cycle evaluation with the introduction of SBFD

Agenda Item:	7.25.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114bis, one issue was raised on whether or not or how to update HPUE duty cycle evaluation with the introduction of SBFD, and RAN4 reached the following common understanding [1]:

##### Issue 1-7-1: Update for HPUE duty cycle evaluation Way forward: - RAN4 has discussed the two following points: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. - FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE.

Under such a common understanding, we propose RAN4 specification changes accordingly in this contribution.

# 2 Discussion

The root cause for this issue is that a symbol can be configured as either only DL transmission or only UL transmission in a TDD band without introduction of SBFD, however, it is not the case when introducing SBFD, where an SBFD symbol consists of both UL and DL transmission in TDD UL-DL pattern, hence there could be different understandings on the current wording “uplink symbols transmitted” used for uplink duty cycle evaluation:

- Understanding #1: “uplink symbols transmitted” means “uplink only symbols transmitted”, then this requires update of RAN4 specs.

- Understanding #2: “uplink symbols transmitted” means “uplink transmission”, not referring to the symbol type in TDD UL-DL pattern, then there is no need to make update of RAN4 specs.

It seems that working groups other than RAN4 tend to have the former understanding, therefore, some clarification could be helpful.

To address this issue, there may be two alternatives to make changes on RAN4 specs:

- Alternative #1 (Generalization): change the wording “uplink symbols transmitted” used for uplink duty cycle evaluation to “uplink transmission”, which include both non-SBFD and SBFD symbols with uplink transmission.

- Alternative #2 (Patching): keep the wording “uplink symbols transmitted” used for uplink duty cycle evaluation unchanged, however, in some general sub-clause, one sentence can be patched to clarify that “uplink symbols transmitted” used for uplink duty cycle evaluation include both non-SBFD and SBFD symbols with uplink transmission.

Observation 1: there are two alternatives to make changes on RAN4 specs in order to ensure the correct uplink duty cycle evaluation for SBFD-aware UEs: one for generalization, the other for patching.

Both ways have their own pros and cons. For the generalization way, it is future-proof, however, there are more than 35 phrases spread in different sub-clauses in TS 38.101-1. For the patching way, just one sentence is needed, however, it is not next to each term “uplink symbols transmitted” used for uplink duty cycle evaluation, thus has less user-friendliness.

Observation 2: The generalization way is future-proof but requiring to update the same term in too many places throughout the spec, while the patching way is simple but is less user-friendly.

Though both of them are not perfect, the patching way requires less changes which is an important aspect at this stage. Therefore, we propose to agree on the patching way.

Proposal 1: RAN4 to take Alternative #2 (Patching) for the updates on uplink duty cycle evaluation.

As one example, the clarification sentence can be added in sub-clause 6.1 as “For an SBFD-aware UE, uplink transmissions in both non-SBFD and SBFD symbols shall be taken into account as uplink symbols transmitted for uplink duty cycle evaluation”, which is shown in our companion draftCR [2].

Proposal 2: RAN4 to add a clarification sentence under sub-clause 6.1 as “For an SBFD-aware UE, uplink transmissions in both non-SBFD and SBFD symbols shall be taken into account as uplink symbols transmitted for uplink duty cycle evaluation”.

# 3 Conclusion

In this contribution, we have the following observations and proposal for updating duty cycle evaluation with the introduction of SBFD:

Observation 1: there are two alternatives to make changes on RAN4 specs in order to ensure the correct uplink duty cycle evaluation for SBFD-aware UEs: one for generalization, the other for patching.

Observation 2: The generalization way is future-proof but requiring to update the same term in too many places throughout the spec, while the patching way is simple but is less user-friendly.

Proposal 1: RAN4 to take Alternative #2 (Patching) for the updates on uplink duty cycle evaluation.

Proposal 2: RAN4 to add a clarification sentence under sub-clause 6.1 as “For an SBFD-aware UE, uplink transmissions in both non-SBFD and SBFD symbols shall be taken into account as uplink symbols transmitted for uplink duty cycle evaluation”.

# Reference

[1] R4-2504752, “Way Forward for [114bis][306] NR_duplex_evo_General”, Samsung

[2] R4-250xxxx, “draftCR to TS 38.101-1 on updating duty cycle evaluation with the introduction of SBFD”, CATT