3GPP TSG RAN WG4 Meeting #115		R4-2506055

St Julian, Malta, May 19th – 23rd , 2025

Title: 	Further discussion on Testability for UE RF requirements for LP-WUS

Agenda Item:	7.28.3.4

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114bis, discussions continued on testability for UE RF requirements for LP-WUS, and a WF was approved with the following agreements on testability:

Issue 4-1-1: Whether LR can be tested directly Agreements: - Down select below two approaches for LP-WUR UE RF requirements: - Option 1: stick to 1% MDR LP-WUS as performance metric, but with a note in TS that LP-WUR ON is not mandated. - The note depends on RAN1 conclusion - Option 2: update it to 1% MDR of PDCCH triggered by LP-WUR Issue 4-1-4: UE test mode for LP-WUR Agreements: - Test mode design as well as the details of test mode for LP-WUS verification can be left to RAN5 with RAN4 inputs.

In this contribution, we present our views and propose to clarify at which point MDR is counted at first.

# 2 Discussion

As agreed in the previous meeting, only one RRC state needs to be tested. Of all three RRC states, there are several benefits if the UE is tested in the RRC_CONNECTED state:

- A dedicated test mode for the UE is unnecessary: test parameters and configurations to UE under test could utilize ordinary message delivering procedure.

- Avoid waking up MR frequently, thus reducing the required testing time: Statistics such as MDR and FAR can be stored during the test and sent via MR to TE by waking up MR at the end of test.

- No need for auxiliary tools

Therefore, we propose to test LP-WUS performance solely in the RRC_CONNECTED state.

Proposal 1: LP-WUS is tested only in RRC_CONNECTED state.

Actually, after the test configuration is completed, and LR is ready for receiving LP-WUS, NR MR can be switched to sleep mode during the test while statistics such as MDR and FAR etc could be countered and buffered. At the end of the test, NR MR can be wakened up and utilized to send packed test statistics to TE. That is to say, the detection rate for LP-WUS is counted without waking up MR during the test.

Proposal 2: MR could be switched to sleep mode and wakened up to send the test statistics to TE at the end of the test.

Proposal 3: Detection rate for LP-WUS is counted without waking up MR during the test.

For test metric, as pointed in our companion paper, RAN4 should firstly reach a common understanding on which point at the signal processing MDR is counted. And if MDR is counted at information bit level, then RAN4 may consider to use BLER as performance metric instead thus no MDR or false alarm is needed. Hence the potential options can be listed as following:

- Option 1a: 1% MDR is counted at OOK chip level of LP-WUS

- Option 1b: 1% MDR is counted at information bit level of LP-WUS

- Option 2: 1% MDR is counted on PDCCH triggered by LP-WUS

- Option 3: 5% BLER as test metric based on the information block

Proposal 4: RAN4 to firstly reach a common understanding on which point at the signal processing MDR is counted before conclude testability issues for UE RF requirements for LP-WUS with the following options:

- Option 1a: 1% MDR is counted at OOK chip level of LP-WUS

- Option 1b: 1% MDR is counted at information bit level of LP-WUS

- Option 2: 1% MDR is counted on PDCCH triggered by LP-WUS

- Option 3: 5% BLER as test metric based on the information block

# 3 Conclusion

In this contribution, we have the following proposals on testability for UE RF requirements for LP-WUS:

Proposal 1: LP-WUS is tested only in RRC_CONNECTED state.

Proposal 2: MR could be switched to sleep mode and wakened up to send the test statistics to TE at the end of the test.

Proposal 3: Detection rate for LP-WUS is counted without waking up MR during the test.

Proposal 4: RAN4 to firstly reach a common understanding on which point at the signal processing MDR is counted before conclude testability issues for UE RF requirements for LP-WUS with the following options:

- Option 1a: 1% MDR is counted at OOK chip level of LP-WUS

- Option 1b: 1% MDR is counted at information bit level of LP-WUS

- Option 2: 1% MDR is counted on PDCCH triggered by LP-WUS

- Option 3: 5% BLER as test metric based on the information block

# Reference

[1] R4-2505234, “WF for [114bis][131] NR_LPWUS_UERF”, vivo