3GPP TSG RAN WG4 Meeting #115		R4-2506056

St Julian, Malta, May 19th – 23rd , 2025

Title: 	Further discussion on BS RF requirements for LP-WUS

Agenda Item:	7.28.4

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114bis, discussions on BS RF requirements for LP-WUS continued and a WF was approved [1]. Some issues are still open as following:

Issue 1-1: How to specify the BS manufacturer declaration for LP-WUS - WF - Manufacturer shall declare the support of LP-WUS for each supported band, [frequency range] and channel bandwidth. - If the BS supports LP-WUS power boosting, manufacturer shall declare the LP-WUS power boosting level it could support with a minimum of +[X]dB. - X can be different for different channel bandwidth. - FFS on the value of X and use 3 as starting point. Issue 1-3: Whether to define EVM requirements for LP-WUS - WF - Further check in next meeting whether EVM for LP-WUS needs to be specified. Issue 1-4: Whether/how to verify EVM requirements for LP-WUS - WF - Consider together with EVM requirement for LP-WUS - If no EVM requirement is defined, no verification is needed - If EVM requirement is defined, FFS on whether verification is needed.

In this contribution, we present our views on these open issues. In particular, X dB power boosting does not mean that the EPRE ratio between LP-WUS and NR REs is increased by XdB according to the definition of power boosting, and the consequence is that for the same XdB, the EPRE ratio could be different for different channel bandwidth. Moreover, EVM requirements should be introduced to ensure the performance of OFDM-based detection with one-shot codepoint according to the latest signal design by RAN1.

# 2 Discussion

As agreed in RAN4, power boosting is defined as the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs) [2], which is similar to the legacy power dynamic range of NB-IoT. However, LP-WUS signal is not a narrow-band signal as NB-IoT, leading to the power dynamic range different from EPRE ratio, e.g., different EPRE ratios are associated with different channel bandwidth even for the same power boosting level XdB [3]:

Table – 1: Power boosting level Vs EPRE ratio with different channel bandwidth

| EPRE ratio (dB) | CBW 10MHz | CBW 20MHz | CBW 30MHz | CBW 40MHz | CBW 50MHz |
| --- | --- | --- | --- | --- | --- |
| +3dB | 1.3 dB | 2.1 dB | 2.4 dB | 2.5 dB | 2.6 dB |
| +6dB | 2.1 dB | 3.7 dB | 4.4 dB | 4.7 dB | 4.9 dB |
| +9dB | 2.5 dB | 4.4 dB | 5.3 dB | 5.8 dB | 6.1 dB |

It is important to clarify in the specification that the declared minimum power boosting level for LP-WUS does not represent the EPRE ratio between LP-WUS and NR; consequently, the actual EPRE ratio varies depending on the configured NR channel bandwidth.

Proposal 1: RAN4 to clarify in the specification that the declared minimum power boosting level for LP-WUS does not represent the EPRE ratio between LP-WUS and NR, and the actual EPRE ratio varies depending on the configured NR channel bandwidth.

With that bearing in minds, set X to 3dB as the minimum value if power boosting is declared for LP-WUS.

Proposal 2: RAN4 to set X to 3dB as the minimum value if power boosting is declared for LP-WUS.

As shown in Fig. 1 also described in more details in our companion paper [4], RAN1 is approaching to final version of LP-WUS signal design with several minor open issues.

Fig. 1, LP-WUS transmitter

There is also a path from information bits directly mapped to a codepoint which is overlaid on ON-chips. In such a way, an OFDM-based receiver can decode the information bits earlier. How the mapping is done will be decided in RAN1 soon. Whereas a symbol-rate processing comes afterward, where for OOK-4 with M chips, all bits are grouped into M-bit tuples, with each tuple is transmitted in one symbol via a DFT module, and then after that, the output is multiplexed across N sub-carriers with other NR modulated symbols followed by an IFFT operation to generate a time-domain signal for transmission.

The most important consideration to enable an OFDM-based receiver in the signal design is that there is one variant which can detect the overlaid codepoint with one shot thus can stop the receiver earlier than the transmission duration, and it is of significance and a good trade-off between the performance and power consumption, therefore, to ensure the feasibility of one-shot codepoint detection with earlier stopping, EVM requirements should be introduced.

Proposal 3: RAN4 to introduce EVM requirements for LP-WUS to ensure the feasibility of one-shot codepoint detection with earlier stopping.

# 3 Conclusion

In this contribution, we have the following proposals for BS RF requirements for LP-WUS:

Proposal 1: RAN4 to clarify in the specification that the declared minimum power boosting level for LP-WUS does not represent the EPRE ratio between LP-WUS and NR, and the actual EPRE ratio varies depending on the configured NR channel bandwidth.

Proposal 2: RAN4 to set X to 3dB as the minimum value if power boosting is declared for LP-WUS.

Proposal 3: RAN4 to introduce EVM requirements for LP-WUS to ensure the feasibility of one-shot codepoint detection with earlier stopping.

# Reference

[1] R4-2504744, “WF on NR LP-WUS BS RF”, Huawei

[2] R4-2503830, TR 38.774 v0.3.0, vivo

[3] R4-2407547, “On BS RF requirements for LP-WUS/LP-WUR”, CATT

[4] R4-2506053, “Further discussion on REFSENS for LP-WUS”, CATT