3GPP TSG RAN WG4 Meeting #115		R4-2506062

St Julian, Malta, May 19th – 23rd , 2025

Title: 	Further discussion on UE RF requirements for 7MHz channel bandwidth

Agenda Item:	7.8.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114bis, a WF was approved on requirements for 7MHz channel bandwidth, where some issues on UE RF requirements are still open [1]:

Agreement: - Evaluate A-MPR for: - NS_12 considering 7 MHz deployed within 814-824 MHz - NS_13 considering 7 MHz deployed in 817-824 MHz (Fc = 820.5 MHz). - NS_14 considering 7 MHz deployed within 824-849 MHz - NS_15 considering 7 MHz deployed within 814-845 MHz or within 841.5-849 MHz.

In this contribution, we propose region partitions for A-MPR for NS_12 based on our simulation.

# 2 Discussion

For NS_12, the additional emission requirement is shown in the table below [2]:

Table 1: Additional requirements NS_12

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |
| 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for E-UTRA carriers with lower channel edge at or above 814 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

The corresponding A-MPR requirements are specified as the following tables according to the current specifications:

Under the same simulation assumptions in [3] as following:

- LO leakage = -28dBc

- Image = -28dBc

- CIM3 = -60dBc

- CIM5 = -70dBc.

After PA model is calibrated with DFT-s-OFDM QPSK 20MHz with 1dB MPR, the following figures illustrate the heatmaps for channel bandwidth 10MHz and 7MHz respectively.

Fig. 1, A-MPR heatmap for 10MHz channel bandwidth

Fig. 2, A-MPR heatmap for 7MHz channel bandwidth

This PA model covers a region corresponding to less than 10 PRBs, similar to that used for a 5 MHz channel. However, to account for different UE implementations and provide flexibility for various waveforms, a region of 14 PRBs can be considered more appropriate. Therefore, we propose defining the NS_12 region as 14 PRBs (i.e., 14 × 0.18 MHz = 2.52 MHz) instead of 10 PRBs for 7MHz channel bandwidth, which proportionally lies between the 10 and 20 PRB allocations for 5 MHz and 10 MHz channel bandwidths.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7MHz |  | ≤2.52 | >0 | A1 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Proposal: RAN4 to specify A-MPR for NS_12 as the following table:

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7MHz |  | ≤2.52 | >0 | A1 |
| 10MHz |  | ≤3.6 | >0 | A1 |

# 3 Conclusion

In this contribution, we have the following proposal for A-MPR for NS_12:

Proposal: RAN4 to specify A-MPR for NS_12 as the following table:

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7MHz |  | ≤2.52 | >0 | A1 |
| 10MHz |  | ≤3.6 | >0 | A1 |

# Reference

[1] R4-2505098, “WF on requirements for 7MHz channel bandwidth”, Ericsson

[2] TS 38.101-1, v19.1.0

[3] R4-1913151, “n26 AMPR”, Qualcomm