3GPP TSG RAN WG4 Meeting #115		R4-2506065

St Julian, Malta, May 19th – 23rd , 2025

Title: 	TP to TR 38.755 on Clause 6

Agenda Item:	7.5.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114bis, a draft big TR for TR 38.755 was approved as version 0.2.0 [1]. In this contribution, we provide revisions on improving wording for clause 6.

# 2 Text proposal

<Start of change>

# 6	Study on power spectral density difference between carriers of co-located adjacent channel operators

Study on impact of power spectral density difference between carriers of inter-operators collocated adjacent channel on the UE is also included in the objective of current SI. The actual PSD difference between operators is for further study. Evaluation results contributed by different companies are captured as below.

Evaluation from company A:

According to the discussion in RAN4#112 as referring to [16], it’s very challenging to ensure that different operators are always co-located. Even in some cases where the operators share the same site, it’s very common that the infrastructures are not shared. As a result, the coverage of different operators is not fully aligned. Figure 6-1 below shows a possible multi-operator deployment case.

Figure 6-1: Multi-operator deployment case

In this example, the UE is located at the edge of its serving cell, but nearly in the center of the neighbouring operator cell. Assuming the BS antenna pattern from TR 38.769 (fixed beam direction) for both cells, the UE’s maximum receiving RSRP difference is equal to the antenna gain difference between horizontal angle = 0 degree and +/-60 degree. Figure 6-2 shows the RSRP difference could achieve 10dB in this case.

Figure 6-2: Delta RSRP Multi-operator deployment case

If the deployment includes both macro cell and micro cell, the possible RSRP difference will increase. Moreover the fast fading will introduce more fluctuation to the RSRP difference. For the purpose of this study, a higher level RSRP difference, e.g. 20dB can be considered.

Evaluation from company B:

Referring to [14], some system level simulation results based on coexistence frame work and incorporating large-scale parameters were also provided. The environment and configuration assumptions are illustrated as following bullets:

- Co-site case

- Same BW/Power/ANT # of 2 operators at n25 (1.9GHz)

    - 10 MHz(wanted)+10 MHz(unwanted) BW

- Care interference from the same site.

- Log RSRP difference between 2 operators à CDF

The detailed parameters are listed in the table below:

Table 6-1: Simulation parameters of PSD difference from UE reception point of view

| Parameter | setting |
| --- | --- |
| Scenario | Dense Urban |
| Channel model | UMa |
| ISD | 200m |
| Number of operators | 2 |
| Number of sites | 7/operator, each with 3 sectors |
| Number of UEs | 10/sectors |
| Carrier frequency | 1.9GHz |
| Channel BW | 10MHz |
| Antenna pattern | AAS is assumed (same as that in coexistence study) |
| BS antenna configuration | 32 ANT# : (M,N,P) = (4,4,2) 128 ANT# : (M,N,P) = (8,8,2) |
| Note: The simulation follows coexistence framework, which considers large-scale parameters only. |  |

Figure 6-3: Antenna patterns for the simulation

Figure 6-4: Simulated PSD difference at UE receiving antenna

From simulation result, it can be seen there’s no “interference level larger than wanted CC case” if the BW of both wanted CCs and unwanted CC(=Wgap) are same. The maximum ratio of unwanted CC level to wanted CC level is 0dB in this case. Other different BW/Wgap configurations can be reasonably scaled to estimate that existing requirements at in-gap ACS1/ACS2/IBB1/IBB2/NBB can fulfil the need for coexistence. On the other hand, the assumption ofinter-operator collocation might be overly simplified and not reflective of real-world deployments. Some companies propose to keep same ACS/IBB requirements and the proposal can be further considered for non-collocated BS cases. And even with the new UE capability where RF parts can support higher order CA configurations with more CCs, the baseband processing capability may still need to be extended to handle these higher order CA combos including more DL carriers as well as increased MIMO-layer processing capability.

Evaluation from company C:

This evaluation is based on system level simulation procedure long established in 3GPP, according to the assumptions listed in this section. The result metric provided is the CDF of the difference in RSRP between the serving operator BS and the strongest interfering BS from the other operator. Various large scale deployment parameters (Antenna Panel, Network Shift) have been varied.

NW shift of 50% and 100% are computed respectively using ISD*sqrt(3)/3/2 and ISD*sqrt(3)/3.

Table 6-2: System Level Simulation Parameters for Company C evaluation

| Parameter | Value | Unity of Measure |
| --- | --- | --- |
| Number of Operators | 2 | - |
| Deployment Scenario | Dense Urban | - |
| Channel Model | UMa (38.901) | - |
| Building Entry Loss Model | P.2109 | - |
| Frequency | 1.9 | GHz |
| Channel Bandwidth | 10 | MHz |
| ISD | 750 | meters |
| Antenna Model | - Sector Antenna, - 32Tx AAS (4,4,2) - 128Tx AAS (8,8,2) | - |
| Active UEs per sector | 1 | - |
| Number of sites/sectors | 19 sites with 3 sectors each | - |
| Network shift across different operators | - Co-site - 50% - 100% | - |
| Sector orientation across different operators | Same | - |

Figure 6-5: Difference between Own BS RSRP and Strongest Interfering BS RSRP [ISD = 750m]

The Figure 6-5 aligns with the Figure 6-4 but provides a broader view as shown in [18].

<End of change>

# Reference

[1] R4-2505158, “draft TR 38.755 v0.2.0”, MediaTek