3GPP TSG-RAN WG4 Meeting #115	R4-2506093

# Malta , MT, 19th – 23rd May, 2025

Title: 	Discussion on less than 5MHz for NTN RRM performance requirements

Source: 	CATT

Agenda item:	7.10.4.5

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 has reached good progress for remaining issues of core requirements [1]. In this contribution, we discuss and provide our views on the performance requirements based on the agreed core requirements.

- Discussion

For NTN with less than 5MHz, according to the agreement reached in the last meeting, RAN4 agreed to not define the some new test cases and there were still FFS for IDLE/inactive mode mobility. The agreement is copied as follows [1]:

RAN4#114-bis： Agreement: - Not define the new test cases for the following requirements RRM requirements; NTN requirements in TS38.133 v18.5.0. RRC connection mobility control; 6.2C RRC Connection Mobility Control for Satellite Access. UE Tx timing, MTTD/ MRTD, timer accuracy, TA accuracy; 7.1C, 7.2C, 7.3C. - Further discuss whether to define the new test case for the following requirement IDLE/inactive mode mobility; 4.2C Cell Re-selection for NR UE for Satellite Access.

In our view, in legacy test cases for TN with less than 5MHz, there was a test case for cell reselection to FR1 intra-frequency NR, which defined in Clause A.6.1.1.9.

In the above test case, different values of TSI-NR (1320 ms) is assumed compared to those in legacy TN (1280 ms). TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell, which is up to RAN2 spec and is not reflected in the RAN4 core requirement.

Observation 1: In legacy, there was a test case for cell reselection to FR1 intra-frequency NR for TN with less than 5MHz, which defined in Clause A.6.1.1.9.

Therefore, even though there was no new requirement for core requirements, we think that RAN4 can follow the same logic and to define a new test case for cell reselection to FR1 intra-frequency NR case in IDLE mode.

Proposal 1: For NTN with less than 5MHz, RAN4 to define new test case for Cell reselection to FR1 intra-frequency NR case only in IDLE mode.

In addition, the agreement for test cases to verify the new core requirements was reached as below [1]:

RAN4#114-bis： Agreement: - Define the new test cases for the core requirements for NTN less than 5MHz introduced in Rel19.e.g. - HO-1: Intra-frequency HO, [unknown cell] - CHO-1: [Inter-frequency CHO], time based with L3 measurement - SatSwitch-1: Hard switch - RLM-1: RLM OOS for 15RB - RLM-2: RLM IS for 15RB - BFR-1: BFR - MR-1: Intra-frequency with SSB index reading - [MR-2: Inter-frequency without SSB index reading] - FFS the detailed configuration for each test case

There were still some square brackets for the above test cases. In our view, in order to avoid a large number of similar test cases, we suggest implementing different configurations in different test cases as much as possible, so we agree to remove the brackets and propose to only define the test cases for the following:

•	HO-1: Intra-frequency HO, unknown cell

•	CHO-1: Inter-frequency CHO, time based with L3 measurement

•	SatSwitch-1: Hard switch

•	RLM-1: RLM OOS for 15RB

•	RLM-2: RLM IS for 15RB

•	BFR-1: BFR

•	MR-1: Intra-frequency with SSB index reading

•	MR-2: Inter-frequency without SSB index reading

Proposal 2: RAN4 to define the following new test cases for NTN with less than 5MHz:

•	HO-1: Intra-frequency HO, unknown cell

•	CHO-1: Inter-frequency CHO, time based with L3 measurement

•	SatSwitch-1: Hard switch

•	RLM-1: RLM OOS for 15RB

•	RLM-2: RLM IS for 15RB

•	BFR-1: BFR

•	MR-1: Intra-frequency with SSB index reading

•	MR-2: Inter-frequency without SSB index reading

For general test configuration, we think that at least two configurations for GSO and NGSO are needed for NTN with less than 5MHz.

Proposal 3: At least two configurations for GSO and NGSO are needed for NTN with less than 5MHz.

- Conclusion

This document discussed the issues for RRM performance requirements for R19 NTN with less than 5MHz and presented following observation and proposals:

Observation 1: In legacy, there was a test case for cell reselection to FR1 intra-frequency NR for TN with less than 5MHz, which defined in Clause A.6.1.1.9.

Proposal 1: For NTN with less than 5MHz, RAN4 to define new test case for Cell reselection to FR1 intra-frequency NR case only in IDLE mode.

Proposal 2: RAN4 to define the following new test cases for NTN with less than 5MHz:

•	HO-1: Intra-frequency HO, unknown cell

•	CHO-1: Inter-frequency CHO, time based with L3 measurement

•	SatSwitch-1: Hard switch

•	RLM-1: RLM OOS for 15RB

•	RLM-2: RLM IS for 15RB

•	BFR-1: BFR

•	MR-1: Intra-frequency with SSB index reading

•	MR-2: Inter-frequency without SSB index reading

Proposal 3: At least two configurations for GSO and NGSO are needed for NTN with less than 5MHz.

- Reference

[1]	R4-2504896, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Xiaomi