3GPP TSG-RAN WG4 Meeting #115	R4-2506094

# Malta , MT, 19th – 23rd May, 2025

Title: 	Discussion on enhancements for Rel-19 ATG RRM core requirements

Source: 	CATT

Agenda item:	7.12.5

Document for:	Discussion

- Introduction

In last RAN4 meeting, RRM requirements for Rel-19 ATG enhancement were discussed. The conclusions for ATG RRM requirements for CA were captured in the approved WF, and there were still some issues for FFS [1]. In this paper, we further discuss the potential RRM impact for Rel-19 ATG RRM requirements and give our views.

- Discussion

  - 2.1 UE Beam Type capability signaling

- The Report granularity of UE capability

In the last meeting, RAN4 has agreed the definition of UE Beam Type capability signaling, but the report granularity of this UE capability were still FFS, and the agreement and WF are copied as follows [1]:

Issue 1-1-2-1: The description of capability definition Agreement: - Indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC. If the field is absent, UE supports two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. - The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band of the supported Band combination. Issue 1-1-2-2: The Report granularity of UE capability FFS: - Per BC

The controversial part is whether the report granularity of this UE capability is per BC or per UE.

In our view, this UE capability signaling is to indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC, it is mainly used for distinguishing Case 4 UE type and Case 5 UE type for inter-band CA.

The reason for distinguishing between Case 4 and Case 5 UE type is that the supported band combination of DL inter-band CA in WID is CA_n3A-n39A, and the two bands are very close. Therefore, the UE can use a common Rx beam to receive, which is due to the characteristics of this band combination. For the same UE, if it can support more band combinations introduced by ATG in the future, and the two bands in the new band combination are not very close, then the UE still cannot use a common Rx beam for reception.

Therefore, we support that the report granularity of UE capability is per BC.

Observation 1: The reason for distinguishing between Case 4 and Case 5 UE type is that the supported band combination of DL inter-band CA in WID is CA_n3A-n39A, and the two bands are very close.

Proposal 1: The report granularity of UE capability is per BC.

- The Report feature

For the report feature, the agreement in the last meeting is copied as below [1]:

Agreement: - Conditional mandatory - The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band [of the supported Band combination]

Since we support the report granularity of UE capability is per BC, so we think that the current description is clear for us, and we suggest to remove the squre braket.

Proposal 2: The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band of the supported Band combination.

  - 2.2 Sharing factor between L1 measurement and RRM measurement

In RAN4#114 meeting, RAN4 has agreed the sharing factor between L1 measurement and RRM measurement, and the agreement is copied as follows [2]:

RAN4#114: Agreement: Agree on the following Proposal 1 for the intra-frequency measurement and L1-RSRP measurement, and apply the similar principle to other related requirements. - Proposal 1: - The Klayer1_measurement and Psharing factor in CA scenario should be updated as (NOTE: take the intra-frequency measurement and L1-RSRP measurement as an example): For UE with the antenna array, - Klayer1_measurement=1, - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions [configured] by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4] or - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols [configured] by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement; Klayer1_measurement=1.5, otherwise.

In the last meeting, the proposal for the sharing factor Klayer1_measurement is copied as follows [1]:

RAN4#114-bis: - Proposal 1: - Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g., - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

Firstly, the proposal 1 is not clear enough for us, and it is just a condition, what is the value of Klayer1_measurement under this condition need to be further clarified. Secondly, if UE supports [capability of case 4], we don’t think RAN4 need to specifically define the Klayer1_measurement and the condition, and this case can be included in otherwise Klayer1_measurement=1.5. Therefore, we support to follow the legacy method for defining the sharing factor and only to specify the case for Klayer1_measurement=1 and otherwise Klayer1_measurement=1.5.

However, the wording of bold attributive for SSB symbols and the RSSI symbols can be referred to replace ‘configured by’, which can be the following:

Proposal 3: RAN4 to follow the legacy method for defining the sharing factor and only to specify the case for Klayer1_measurement=1 and otherwise Klayer1_measurement=1.5.

- Klayer1_measurement=1,

- if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

- if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4] or

- if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or

- if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

- Klayer1_measurement=1.5, otherwise.

  - 2.3 Measurement enhancement for ATG UE with antenna array

In the last meeting, RAN4 discussed the measurement enhancement for ATG UE with antenna array but still had no consensus. The candidate options are copied as below [1]:

RAN4#114-bis: Candidate options: - UE could skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell} by network control. - Option 1: this can already be achieved by the existing configuration. - Option 2: Consider enhancement for Rel-19 ATG CA

In our view, the  UE could skip the neighbour cell measurement of SCC and/or CC with deactivated SCell by network control, the it can be achieved by the existing network configuration. The existing network can explicitly delete or disable measurement objects related to neighbour cells on SCC through RRC configuration, or filter neighbour cells on SCC through limitations on measurement object parameters (e.g. allowedPCI-List or blackPCI-List), or delete related measurement task via dedicated signaling, allowing UE to skip neighboring cell measurements in certain situations.

Observation 2: The existing network can explicitly delete or disable measurement objects related to neighbour cells on SCC through RRC configuration, or filter neighbour cells on SCC through limitations on measurement object parameters (e.g. allowedPCI-List or blackPCI-List), or delete related measurement task via dedicated signaling,

Proposal 4: UE could skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell} by network control, which has already be achieved by the existing configuration.

- Conclusion

This document discussed the issues for R19 ATG RRM core requirements and presented following observations and proposals:

Observation 1: The reason for distinguishing between Case 4 and Case 5 UE type is that the supported band combination of DL inter-band CA in WID is CA_n3A-n39A, and the two bands are very close.

Proposal 1: The report granularity of UE capability is per BC.

Proposal 2: The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band of the supported Band combination.

Proposal 3: RAN4 to follow the legacy method for defining the sharing factor and only to specify the case for Klayer1_measurement=1 and otherwise Klayer1_measurement=1.5.

- Klayer1_measurement=1,

- if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

- if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4] or

- if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or

- if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

- Klayer1_measurement=1.5, otherwise.

Observation 2: The existing network can explicitly delete or disable measurement objects related to neighbour cells on SCC through RRC configuration, or filter neighbour cells on SCC through limitations on measurement object parameters (e.g. allowedPCI-List or blackPCI-List), or delete related measurement task via dedicated signaling,

Proposal 4: UE could skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell} by network control, which has already be achieved by the existing configuration.

- Reference

[1]	R4-2504898, WF on RRM requirements for NR_ATG_enh, CMCC

[2]	R4-2502591, WF on RRM requirements for NR_ATG_enh, CMCC