3GPP TSG-RAN WG4 Meeting #115	R4-2506097

# Malta , MT, 19th – 23rd May, 2025

Title: 	Discussion on enhancements for Rel-19 ATG RRM performance requirements

Source: 	CATT

Agenda item:	7.12.6

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 has reached good progress in defining the core RRM requirements and CRs split work has been done [1]. In this contribution, we discuss and provide our views on the performance requirements based on the agreed core requirements.

- Discussion

- Test setups

In the last meeting, RAN4 discussed test setups for R19 ATG CA, and the agreement are copied as below [1]:

Agreement: - Following test setups agreed in R18 can be reused: - TDD pattern: The legacy TDD pattern are used for PCell, FFS for SCell - Channel model: AWGN with residual doppler channel model - Number of neighbour cells: one neighbour cell in the test if needed - GNSS and UE mobility setup: UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system. - BS location: The specific gNB reference location is emulated by test system - RMC: the legacy NR RMCs defined in section A.3.1 of TS 38.133 are reused for ATG test cases. - OCNG: - Generic OCNG pattern for all unused REs defined in A.3.2.1.1 in TS 38.133. - Generic OCNG pattern for unused REs in the same bandwidth as CORESET defined in A.3.2.1.3 of TS 38.133. - Generic OCNG pattern for all unused REs outside SSB slot(s) defined in A.3.2.1.4 in TS 38.133

For TDD pattern, RAN4 agreed that the legacy TDD pattern can be used for PCell, but there was still FFS for SCell. In our view, for intra-band CA, n79 is TDD band, so aggregated TDD cell need to have the same TDD configuration, which is to ensure HARQ timing synchronization, avoid conflicts in timing alignment, and ensure cross carrier scheduling. For inter-band CA, the example band combination is n3_n39, which is FDD and TDD band, so if SCell is operating in TDD band, then the TDD pattern or SCell can be independently configured and the pattern used in R18 ATG can also be reused.

Observation 1: For intra-band CA, the aggregated TDD cells need to have the same TDD configuration, which is to ensure HARQ timing synchronization, avoid conflicts in timing alignment, and ensure cross carrier scheduling.

Proposal 1: For intra-band CA, TDD pattern for SCell need to be the same with PCell.

Proposal 2: For inter-band CA, if SCell is operating in TDD band, then the TDD pattern for SCell can be independently configured and the pattern used in R18 ATG test cases can also be reused.

- Test method and scaling factor for UE with antenna array

In the last meeting, RAN4 discussed the test method and scaling factor for UE with antenna array, but there was still FFS. The WF is copied as follows [1]:

FFS: - The scaling factor related with antenna array will not be applied in test requirements - The test parameters, test requirements for UE with antenna array on both PCC and SCC

In R18, RAN4 discussed the test method and scaling factor for UE with antenna array for a long time in conjunction with RF, and agreed to consider scaling factor = 1 for antenna array related test requirements. It is also agreed that conductive test can be used.

In our view, R18 has put in a lot of effort to determine test method and scaling factor for UE with antenna array, and the difficulties of using OTA measurement faced in R18 also exist in R19 ATG CA. Also, currently we haven't seen any significant reasons to change the test method and scaling factor in the related test cases. Therefore, we support to apply scaling factor = 1 for antenna array related test requirements and to use conductive test method.

Observation 2:  R18 has put in a lot of effort to determine test method and scaling factor for UE with antenna array, and the difficulties of using OTA measurement faced in R18 also exist in R19 ATG CA.

Proposal 3: RAN4 to apply scaling factor = 1 for antenna array related test requirements and use conductive test method.

- Test scope

For the test scope, acoording to the agreement in core part, we use the following table from the WF to present our specific viewpoints [1], which is:

Proposal 4: At least the following test cases can be defined for R19 ATG CA:

| Test category | Test cases |
| --- | --- |
| Interruption | Interruptions during measurements on deactivated NR SCC in FR1 |
|  | Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot |
|  | Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot |
|  | Interruption at NR SRS antenna port switching |
| SCell Activation and deactivation | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |
|  | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |
|  | SCell Activation and deactivation of unknown SCell in FR1 in non-DRX |
|  | Direct SCell activation at SCell addition of known SCell in FR1 |
|  | Direct SCell activation at handover with known SCell in FR1 |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |
|  | SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle |
|  | TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX |
| Active BWP Switch | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs |
|  | Simultaneous RRC-based Active BWP Switch on multiple CCs |
| SCell dormancy switch | SCell dormancy switch |
| deactivated SCell measurement | Event triggered reporting on SCC with deactivated SCell test under non-DRX |
| Pre-configured measurement gap | Test case for Pre-configured measurement gap requirement |

- Conclusion

This document discussed the issues for R19 ATG RRM performance requirements and presented following observations and proposals:

Observation 1: For intra-band CA, the aggregated TDD cells need to have the same TDD configuration, which is to ensure HARQ timing synchronization, avoid conflicts in timing alignment, and ensure cross carrier scheduling.

Proposal 1: For intra-band CA, TDD pattern for SCell need to be the same with PCell.

Proposal 2: For inter-band CA, if SCell is operating in TDD band, then the TDD pattern for SCell can be independently configured and the pattern used in R18 ATG test cases can also be reused.

Observation 2:  R18 has put in a lot of effort to determine test method and scaling factor for UE with antenna array, and the difficulties of using OTA measurement faced in R18 also exist in R19 ATG CA.

Proposal 3: RAN4 to apply scaling factor = 1 for antenna array related test requirements and use conductive test method.

Proposal 4: At least the following test cases can be defined for R19 ATG CA:

| Test category | Test cases |
| --- | --- |
| Interruption | Interruptions during measurements on deactivated NR SCC in FR1 |
|  | Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot |
|  | Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot |
|  | Interruption at NR SRS antenna port switching |
| SCell Activation and deactivation | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |
|  | SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |
|  | SCell Activation and deactivation of unknown SCell in FR1 in non-DRX |
|  | Direct SCell activation at SCell addition of known SCell in FR1 |
|  | Direct SCell activation at handover with known SCell in FR1 |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle |
|  | Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle |
|  | SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle |
|  | TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX |
| Active BWP Switch | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs |
|  | Simultaneous RRC-based Active BWP Switch on multiple CCs |
| SCell dormancy switch | SCell dormancy switch |
| deactivated SCell measurement | Event triggered reporting on SCC with deactivated SCell test under non-DRX |
| Pre-configured measurement gap | Test case for Pre-configured measurement gap requirement |

- Reference

[1]	R4-2504898, WF on RRM requirements for NR_ATG_enh, CMCC