3GPP TSG-RAN WG4 Meeting #115		                                                        R4-2506100

# Malta , MT, 19th – 23rd May, 2025

Title:	Discussion on RRM requirements for On-demand SIB1

Source:	CATT

Agenda Item:	7.27.3.3

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 had discussion on potential RRM impact for On-demand SIB1 and the WF captured the related agreement [1]. In this paper, we will further discuss the RRM impact for On-demand SIB1 and give our views.

- Discussion

RRM requirements impacts for OD-SIB1

Paging interruption

In the last meeting, RAN4 discussed the OD-SIB1 related RRM impacts on paging interruption requirements, there was no consensus on this issue, and the Way forward is copied as follows [1]:

Way forward: - Option A: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. - Option B: No impacts on core requirements. Discuss whether to extend TSI-NR in test case. - Option C: No RRM impacts on the requirements, i.e., UL-WUS can be postponed during the paing time.

For paging procedure, when UE performing target cell relevant SIBs reception, cell re-selection is interrupted and RAN4 defined the corresponding maximum interruption in paging reception requirements. The legacy paging reception related requirements are copied as below [3]:

#### 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms. TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell. These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

In our view, legacy interruption time include TSI-NR, which is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 for an NR cell. However, RAN4 didn’t define the exact value for system information data receiving in TS 38.133 [2], so the time for additional paging interruption can be included in TSI-NR and core requirements will have no impact.

However, it can’t be guaranteed that UL-WUS transmission and paging reception will never conflict at any time, as paging is random. Also considering the OD-SIB1 receiving, we prefer to consider the extension of TSI-NR in the test case.

Observation 1: In legacy maximum interruption in paging reception requirements, RAN4 didn’t define the exact value for system information data receiving in TS 38.133.

Proposal 1: There is no impact on core requirements in TS 38.133.

Proposal 2: The extension of TSI-NR should be considered in the test case.

- Conclusion

In this paper, we have some further discussions on On-demand SIB1, and the following observations and proposals are given:

Observation 1: In legacy maximum interruption in paging reception requirements, RAN4 didn’t define the exact value for system information data receiving in TS 38.133.

Proposal 1: There is no impact on core requirements in TS 38.133.

Proposal 2: The extension of TSI-NR should be considered in the test case.

- Reference

- R4-2504907, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei

- 3GPP TS 38.133, NR Requirements for support of radio resource management