3GPP TSG-RAN WG4 Meeting #115	R4-2506106

# Malta , MT, 19th – 23rd May, 2025

Title: 	Discussion on conditional Intra-CU LTM for Rel-19 mobility enhancements

Source: 	CATT

Agenda item:	7.29.3.3

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_Mob_Ph4_Part2 was approved and some agreements were reached [1], but there still many issues left [2]. This document will further discuss the remaining RRM issues about Conditional LTM/Subsequent Conditional LTM, and give our understanding and proposals.

- Discussion

Sub-topic 1-1: Conditional LTM delay

- The components of conditional LTM delay

In the last meeting, RAN4 discussed the components of Tinterrupt, and the agreement and Way forward are copied as follows [1]:

RAN4#114-bis: Issue 1-2-4: Tinterrupt < Agreement > - Keep either TCLTM_execution or TCLTM-RRC-processing as one component for the conditional LTM delay. - Note: The legacy value(s) in previous releases will be reused. - TCLTM_execution or TCLTM-RRC-processing is not included in Tinterrupt. - Not include the components for TCI state activation and/or PDCCH order PRACH in C-LTM delay. - TEvent_DU can be updated based on RAN2 progress if needed. - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell. - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered. - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO. - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19. < Way forward > FFS - Tfirst-RS and TRS-proc are included in Tinterrupt - Reuse the values in Rel-18 LTM, and further discuss whether the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM can be reused or not.

In the last meeting, one controversial part is how to define Tfirst-RS and TRS-proc. One direct way is to reuse the values in Rel-18 LTM. However, some companies think that current T/F fine time tracking time is mainly used for waiting for SSB, and actually the data transmission can be uninterrupted during the time for waiting for SSB. Considering that the source cell does not know when the UE will start switching to the target cell in CHO, so further reducing interruption time is beneficial for the network. To remove the time for waiting for SSB from interruption time can be regarded as an optimization. From our point of view, we prefer to reuse the legacy value in R18 LTM, but the further optimization is also fine for us.

Proposal 1: Prefer to reuse the legacy value of Tfirst-RS and TRS-proc in R18 LTM, but the further optimization on interruption time is also fine for us.

- TRRC

In the last meeting, the Way forward for TRRC is copied as below [1]:

< Way forward > FFS the following proposals: - Proposal 1: TRRC is the RRC procedure delay.

Since RAN4 has agreed the starting point of conditional LTM delay in the last meeting, which is the end of the last TTI containing the RRC command for conditional LTM, so TRRC should be included in the conditional LTM delay and the legacy defination of TRRC can be reused, which is the RRC procedure delay.

Proposal 2: TRRC is the RRC procedure delay.

- TLTM-processing

In the last meeting, the Way forward for TLTM-processing is copied as below [1]:

< Way forward > FFS the following proposal: - Proposal 1: TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Proposal 1a: TLTM-processing can be up to 20ms.

For TLTM-processing, we think that the legacy defination in R18 LTM can be reused, which is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

Proposal 3: TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- TLTM-IU

In the last meeting, the Way forward for TLTM-IU is copied as below [1]:

< Way forward > FFS the following proposal: - Proposal 1: TLTM-IU is the interruption uncertainty during LTM cell switch.

For TLTM-IU, we think that the legacy defination in R18 LTM can be reused, that is the interruption uncertainty during LTM cell switch.

More specifically, since both RACH-less and RACH based conditional intra-CU LTM have been supported, so TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell for RACH-based LTM cell switch, and TLTM-IU is the uncertainty on transmitting the first uplink transmission on the target cell for RACH-less LTM cell switch [2].

Proposal 4: TLTM-IU is the interruption uncertainty during LTM cell switch.

- For RACH-based LTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell.

- For RACH-less LTM cell switch, TLTM-IU_is the uncertainty on transmitting the first uplink transmission on the target cell.

Sub-topic 1-2: Subsequent Conditional LTM

For Subsequent Conditional LTM delay, the starting point has been agreed as follows [1]:

RAN4#114-bis: < Agreement > - The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

In our view, only the different starting points of the delay between Conditional LTM and Subsequent Conditional LTM, and TRRC will not be considered in subsequent conditional LTM delay based on the conclusion of conditional LTM delay.

Apart from the above, there is no difference between Conditional LTM and Subsequent Conditional LTM, to simplify the spec, we also suggest defining the initial and subsequent CLTM cell switch delay requirements in the same section under Clause 6.3 [2].

Observation 1: Only the different starting points of the delay between Conditional LTM and Subsequent Conditional LTM.

Proposal 5: TRRC will not be considered in subsequent conditional LTM delay based on the conclusion of conditional LTM delay.

Proposal 6: RAN4 to define the initial and subsequent CLTM cell switch delay requirements in the same section under Clause 6.3.

- Conclusion

This document discussed the issues for conditional Intra-CU LTM and presented following observations and proposals:

Proposal 1: Prefer to reuse the legacy value of Tfirst-RS and TRS-proc in R18 LTM, but the further optimization on interruption time is also fine for us.

Proposal 2: TRRC is the RRC procedure delay.

Proposal 3: TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

Proposal 4: TLTM-IU is the interruption uncertainty during LTM cell switch.

- For RACH-based LTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell.

- For RACH-less LTM cell switch, TLTM-IU_is the uncertainty on transmitting the first uplink transmission on the target cell.

Observation 1: Only the different starting points of the delay between Conditional LTM and Subsequent Conditional LTM.

Proposal 5: TRRC will not be considered in subsequent conditional LTM delay based on the conclusion of conditional LTM delay.

Proposal 6: RAN4 to define the initial and subsequent CLTM cell switch delay requirements in the same section under Clause 6.3.

- Reference

[1]     R4-2504910, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

[3]     3GPP TS 38.133, NR Requirements for support of radio resource management