3GPP TSG RAN WG4 Meeting #115		R4-2506113

St Julian, Malta, May 19th – 23rd , 2025

Title: 	TP to TR 38.774 on transmitted signal quality (clause 7.2.4) for LP-WUS

Agenda Item:	7.28.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

This contribution provides a very initial version on Clause 7.2.4 for transmit signal quality, and is expected to be revised further pending on the outcome of the discussion on this sub-topic.

# 2 Text proposal

< Start of change>

## 7.2.4 Transmitted signal quality

##### 7.2.4.1 Transmitted signal generation for LP-WUS

Fig. 7.2.4.1-1, Transmitted signal generation for LP-WUS

Figure 7.2.4.1-1 illustrates the LP-WUS signal generation process based on the latest RAN1 signal design as defined. Up to five information bits are channel coded, followed by rate matching of the coded bits. The output is then Manchester coded, doubling the bit length. This process is performed on a per-block basis. There is also a path from information bits directly mapped to a codepoint which is overlaid on ON-chips. In such a way, an OFDM-based receiver can decode the information bits earlier. How the mapping is done will be decided in RAN1 soon. Whereas a symbol-rate processing comes afterward, where for OOK-4 with M chips, all bits are grouped into M-bit tuples, with each tuple is transmitted in one symbol via a DFT module, and then after that, the output is multiplexed across N sub-carriers with other NR modulated symbols followed by an IFFT operation to generate a time-domain signal for transmission.

##### 7.2.4.2 EVM

<TBD>

<End of change>