3GPP TSG-RAN WG4 Meeting #115	R4-2506118

Malta, MT, May 19th – May 23rd, 2025

Source:	Nokia

Title:	TP for TR 38.774 on LP-WUS

Agenda item:	7.28.2

Document for:	Discussion

Introduction

In this text proposal for TR 38.774, we provide our inputs on section 7.1. These inputs are based on the agreements achieved in the previous meetings.

- References

- R4-2505234, WF for [114bis][131] NR_LPWUS_UERF, Vivo, RAN4#114bis, Wuhan, China, 7th – 11th April, 2025.

- R4-2503028, WF on UE RF requirements for LP-WUS, Vivo, RAN4#114, Athens, Greece, 17th – 21st Feb, 2025.

- R4-2420528, WF on UE RF requirements for LP-WUS, Vivo, RAN4#113, Orlando, US, 18th – 22nd Nov, 2024.

- R4-2417112, WF on UE RF requirements for LP-WUS, Vivo, RAN4#112bis, Hefei, China, 14th – 18th Oct, 2024.

- R4-2414309, WF on UE RF requirements for LP-WUS, Vivo, RAN4#112, Maastricht, Netherlands, 19th – 23rd August, 2024.

****************************************** Start of TP ******************************************

### 7.1.1	General

The minimum requirements will be determined assuming there is only one receiver. A 1% MDR criterion will be used for verification of all core RF requirements. Following LP-WUS parameters will be used

Table 7.1.1-1. Common reference channel parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Waveform type |  | OOK-4 |
| Chip rate |  | M=4 |
| SCS | KHz | 30 |
| Information bit length | bits | 5 |
| Length | NR OFDM symbols | 16 |
| Overlay sequence mapping |  | Raw information bits |
| Number of sequences |  | 4 |

************************* UNCHANGED CLAUSES HAVE BEEN OMITTED *************************

### 7.1.4	REFSENS requirements

It was agreed to derive the REFSENS for LP-WUR in the typical RF manner with AWGN being the channel for which SNR is derived. Thus, the REFSENS for LR is defined as

REFSENSLR (dBm) = -174 + BWLR + NFLR + IMLR + SNRLR.

Here, BWLR corresponds to the bandwidth of the LP-WUS signal in hertz. RAN1 has decided to have 11 RBs for LP-WUS irrespective of SCS, thus the LP-WUS bandwidth will be 1.98 MHz and 3.96 MHz, for 15 and 30 KHz SCS, respectively. Further, there are two set of values for both NFLR + IMLR and SNRLR. There is only single receiver so there is no diversity gain included in the REFSENS.

****************************************** End of TP ******************************************