3GPP TSG-RAN WG4 Meeting #115	R4-2506132

Saint Julian’s, Malta, May 19th –May 23rd, 2025

Source:	Nokia

Title:	Discussion on Less than 5MHz Phase 2 performance requirements

Agenda item:	7.3.4

Document for:	Discussion

Introduction

RAN4 has started to discuss details of the test cases for rel-19 < 5MHz phase 2 work item. Related to this, the following has been agreed to be implemented as a test cases:

Issue 2-1-1: Confirm the agreed test cases list

Table 1 Test cases list for less than 5MHz CA/DC

| No. | Item of core requirements | Type of test cases | FR | Subclause | Note | Company |
| --- | --- | --- | --- | --- | --- | --- |
| 3-1 | Deactivated SCell/PSCell measurements | SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB for a deactivated SCell | FR1 | New A.6.6.x.x | Modify existing test cases. Tested together with neighbour cell measurements. | Nokia |

This contribution discusses about the details of the test case SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB for a deactivated SCell.

Discussion

SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB for a deactivated SCell

This test case is testing SSB index reading on SCell when the 12 PRB SSB is transmitted on the same band as SCell.

Similarly to rel-18, RAN4 has agreed to use delta approach in the test cases. This means that only parameters that differ from the baseline test cases are described. In this particular test case, the usual delta approach cannot be applied because  the test case is a new one.

Delta approach cannot be used directly for the SA event triggered test case with SCell because it’s a new test case.

To describe the test case further, the test case has three cells, 10MHz PCell on Carrier 1, and SCell on carrier 2 with a neighbouring cell that is to be detected. The reason for neighbouring cell detection is that the deactivated SCell should not need index detection, which is the main feature to be tested. The following table illustrates the configuration.

| Parameter | Test configuration | Value |  |
| --- | --- | --- | --- |
| Active cell | 10 MHz PCell | Cell 1 | Carrier 1 |
| Neighbour cell | 3MHz with 12 PRB SSB | Cell 2 | Carrier 2 |
| Deactivated SCell | 3MHz with 12 PRB SSB | Cell 3 |  |

PCell is on carrier 1

Deactivated SCell doesn’t need index detection

Scell and neighbouring cell are on carrier two with intra-frequency relationship

The following figure illustrates the test case design:

During the test the PCell and SCell levels are kept unchanged but the Cell 2, which is an intra-frequency neighbouring cell for the Scell, is detected by the UE and 12 PRB SSB index is read by the UE. We think this way we can test that the intra (SCell) neighbouring cell index reading is operational with 12 PRB and deactivated SCell.

Test 12 PRB neighbouring cell index reading on the same carrier (carrier 2) as the deactivated SCell. PCell is using legacy SSB on carrier 1.

Conclusion

The following Observations and Proposals were made:

Observation 1: Delta approach cannot be used directly for the SA event triggered test case with SCell because it’s a new test case.

Observation 2: PCell is on carrier 1

Observation 3: Deactivated SCell doesn’t need index detection

Observation 4: Scell and neighbouring cell are on carrier two with intra-frequency relationship

Proposal 1: Test 12 PRB neighbouring cell index reading on the same carrier (carrier 2) as the deactivated SCell. PCell is using legacy SSB on carrier 1.