3GPP TSG-RAN4 Meeting #115	R4-2506133

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to 38.133 on Test case for SA event triggered reporting test without gap under non-DRX with SSB index reading and 12 PRB SSB for a deactivated SCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW_Ph2-Perf |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR introduces a test case for SA event triggered reporting test without gap under non-DRX with SSB index reading and 12 PRB SSB for a deactivated SCell |  |  |  |  |  |  |  |  |
| Summary of change: |  | A.6.6.x.x SA event triggered reporting test without gap under non-DRX with SSB index reading and 12 PRB SSB for a deactivated SCell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The core requirements remain untested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | In the test case it is assumed that the PCell FR1 10MHz cell 1, while the SCell is cell 2 supporting 12 PRBs. TS 38.133 section 9.2.5.1 Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured: - For a UE indicating support3MHz-ChannelBW-Symmetric-r18 and configured to perform measurements on either a deactivated SCell or a deactivated PSCell with 12PRB SSB in FR1, TSSB_time_index_intra_less_than_5MHz is given in table 9.2.5.1-24 (deactivated SCell) or in table 9.2.5.1-25 (deactivated PSCell). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

#### A.6.6.x.x	SA event triggered reporting test without gap under non-DRX with SSB index reading and 12 PRB SSB for a deactivated SCell

##### A.6.6.x.x.1	Test purpose and Environment

The purpose of this test is to verify that the UE supporting support-3MHz-ChannelBW-r18 makes correct reporting of an event on an intra-frequency layer measurement performed without measurement gaps when the UE is also configured deactivated SCell. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

##### A.6.6.x.x.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1) Carrier 1, FR1 Neighbouring cell on Carrier 2 (Cell 2) and a FR1 SCell (Cell 3) on Carrier 2. Carrier 2 is on a different frequency than the PCell.

The test parameters for PCell and neighbour cell are given in table A.6.6.x.x.x.x-1, A.6.6.x.x.x-2 and A.6.6.x.x.x.x-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and the SCell and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.x.x.x-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |
| NOTE: Configuration 1 is used for PCell and configuration 2 is used for SCell within the same test case. |  |

Table A.6.6.1.9.2.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1 with 12 PRB SSB for SCell

| Parameter | Unit |  | Test configuration |  | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Active cell |  |  | 1 |  | Cell 1 |  |  |  |
| Neighbour cell |  |  | 2 |  | Cell 2 |  |  | Cell to be identified. |
| Deactivated SCell |  |  | 2 |  | Cell 3 |  |  | Deactivated throughout the test. |
| RF Channel Number |  |  | 1, 2 |  | 1: Cell 1 2: Cell 2 and Cell 3 |  |  | 1: Cell 1 is PCell 2: Cell 2 is neighbouring cell to be detected |
| SSB configuration |  |  | 1 |  | SSB.1 FR1 |  |  |  |
|  |  |  | 2 |  | SSB.13 FR1 |  |  | 12 PRB SSB |
| SMTC configuration |  |  | 1 |  | SMTC.2 |  |  |  |
|  |  |  | 2 |  | SMTC.1 |  |  |  |
| A3-Offset | dB |  | 1 |  | -4.5 |  |  |  |
| CP length |  |  | 2 |  | Normal |  |  |  |
| Hysteresis | dB |  | 1, 2 |  | 0 |  |  |  |
| Time To Trigger | s |  | 1 |  | 0 |  |  |  |
| Filter coefficient |  |  | 1, 2 |  | 0 |  |  | L3 filtering is not used |
| DRX |  |  | 1 |  |  |  |  | OFF |
| measCycleScell | ms |  | 2 |  | 160ms |  |  |  |
| Time offset between serving and neighbour cells |  |  | 1 |  | 3 ms |  |  | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  |  | 2 |  | 3 s |  |  | Synchronous cells |
| T1 | s |  | 1, 2 |  | 5 |  |  |  |
| T2 | s |  | 1, 2 |  | 5 |  |  |  |

Table A.6.6.x.x.x.x-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1 with 12 PRB SSB for SCell

| Parameter |  |  |  | Unit |  | Test configuration |  |  |  | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |  | T1 |  | T2 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.2 FDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  |  |  |  |  | 1 |  |  |  | CCR.1.1 FDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
|  |  |  |  |  |  | 2 |  |  |  | CCR.1.7 FDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
| Dedicated CORESET RMC configuration |  |  |  |  |  | 1 |  |  |  | CCR.1.1 FDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
|  |  |  |  |  |  | 2 |  |  |  | CCR.1.7 FDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
| OCNG Patterns |  |  |  |  |  | 1, 2 |  |  |  | OP.1 |  |  |  | OP.1 |  |  |  | OP.1 |  |  |  |
| TRS Configuration |  |  |  |  |  | 1 |  |  |  | TRS.1.1 FDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
|  |  |  |  |  |  | 2 |  |  |  | TRS.1.1 TDD |  |  |  | N/A |  |  |  | N/A |  |  |  |
| IInitial BWP configuration |  |  |  |  |  | 1, 2 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Active DL BWP configuration |  |  |  |  |  | 1, 2 |  |  |  | DLBWP.1.1 |  |  |  | DLBWP.1.1 |  |  |  | DLBWP.1.1 |  |  |  |
| Active UL BWP configuration |  |  |  |  |  | 1, 2 |  |  |  | ULBWP.1.1 |  |  |  | ULBWP.1.1 |  |  |  | ULBWP.1.1 |  |  |  |
| RLM-RS |  |  |  |  |  | 1, 2 |  |  |  | SSB |  |  |  | SSB |  |  |  | SSB |  |  |  |
| Note 2 |  | dBm/SCS |  |  |  |  | 1 |  |  | -98 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 2 |  |  | -98 |  |  |  |  |  |  |  |  |  |  |
| Note 2 |  | dBm/15 kHz |  |  |  |  | 1 |  |  | -98 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |  |  |
|  |  | 2 |  | s |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |  |  |
|  |  | 2 |  |  |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS kHz |  |  |  |  | 1 |  |  | -94 |  | -94 |  | -Infinity |  | -94 |  | -94 | -94 |  |
|  |  |  |  |  |  |  | 2 |  |  | -94 |  | -94 |  | -Infinity |  | -94 |  | -94 | -94 |  |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |  |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |  |  |
| Propagation Condition |  |  |  |  |  | 1, 2 |  |  |  | AWGN |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.6.X.X.3	Test requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1120 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index in this test. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of change 1>