3GPP TSG-RAN WG4 Meeting #115	R4-2506134

Saint Julian’s, Malta, May 19th –May 23rd, 2025

Source:	Nokia

Title:	Discussion on LP-WUR Simulation assumptions and results

Agenda item:	7.28.5.1

Document for:	Discussion

Introduction

This contribution discusses about simulation assumptions and results for LP-SS.

Discussion

I & Q branches

Our simulations use both I & Q branches. Our understanding is that this has also been the assumption in RAN1 in most of the companies. Assuming only I-branch operation will create requirements that are not very realistic. If companies want to support I-branch only, then this can be defined as a separate capability / incapability.

RAN4 simulations assume both I & Q branches. If needed, define single branch as a capability in core requirements.

Number of samples

From the results in R4-2506135 we observed that the number samples do not impact significantly to the observed accuracy and timing error.  From our simulation results, we observed that RSRP accuracy has some dependance on sequence between serving and interfering cells.  To fulfil the 3.5dB accuracy requirement, we think 2 samples is enough.

Number of samples does not significantly impact the accuracy of the LP-SS measurements across different scenarios

Use 2 LP-SS samples as a baseline to define RAN4 LP-WUR core measurement requirements.

Conclude on the measurement accuracy in core requirements with simulations with final sequence design once RAN1 has agreed the LP-SS design.

OOK interfering cell

Issue 2-1-7: On OOK interference

Currently RAN4 is simulating with an interference pattern that is selected from the available patterns from RAN1. Selecting overly good pattern will bias the RAN4 simulation results, and eventually the UE requirements as well as selecting opposite pattern will cause problems. In real deployment scenario it is highly likely that there will be some time offset between neighbouring OOK patterns, hence the deployment in the scenario is not realistic.

We observed some performance degradation for the case when fully opposite sequence is used for serving and neighbour cell, and we are ok to exclude the case or no need to have any particular consideration for this case. If this case needs to be treated, then it’s ok for us to apply time offset for the interfering cell.

For the case when fully opposite sequence is used for serving and neighbour cell, no need to have any particular consideration for this case. If consideration is needed, RAN4 can have a time-offset for the interfering signal.

Simulation timeline

For the RAN4#114 meeting, companies were requested to provide simulations for high level alignment. Running simulations can require a quite an effort from companies and there are sometimes unexpected delays. RAN4 should aim to finalise the simulation campaign on RAN4#116 meeting.

Finalise simulation campaign latest in RAN4#116 meeting.

Conclusion

The following Observations and Proposals were made:

Proposal 1: RAN4 simulations assume both I & Q branches. If needed, define single branch as a capability in core requirements.

Observation 1: Number of samples does not significantly impact the accuracy of the LP-SS measurements across different scenarios

Proposal 2: Use 2 LP-SS samples as a baseline to define RAN4 LP-WUR core measurement requirements.

Proposal 3: Conclude on the measurement accuracy in core requirements with simulations with final sequence design once RAN1 has agreed the LP-SS design.

Proposal 4: For the case when fully opposite sequence is used for serving and neighbour cell, no need to have any particular consideration for this case. If consideration is needed, RAN4 can have a time-offset for the interfering signal.

Proposal 5: Finalise simulation campaign latest in RAN4#116 meeting.