3GPP TSG-RAN4 Meeting #115	R4-2506137

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS 38.133 on LP-WUR serving cell measurement and evaluation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR implements NR_LPWUS UE requirements for serving cell measurement and evaluation. - General description - LP-WUR evaluation requirement (OOK and OFDM based) - LP-WUR measurement periodicity, filtering and [measurement requirements] (OOK and OFDM based) - LR-WUR measurement accuracy，represented by margins (OOK and OFDM based) |  |  |  |  |  |  |  |  |
| Summary of change: |  | 4.x.2 LP-WUR serving cell measurement and evaluation requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No UE requirements for the feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Agreements implemented in this CR: RAN4#110bis Agreement: - At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following: - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state Other related requirements are FFS RAN4#112bis Agreement: From RAN4 requirement perspective, combine measurement results from different receivers shall not be considered Agreement: - RAN4 will further discuss whether any technical issues regarding the LS from RAN1. - From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2. Issue for further discussion based on contribution driven: - For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.] Agreement: - Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity. RAN4#113 Agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. Agreement: - When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity. RAN4#114 Agreement: - In Rel-19, the same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS regardless of whether overlaid OFDM sequence is configured or not. Agreement: - Define the requirements for OFDM-based LP-WUR serving cell measurement based only on LP-SS - Reuse the same requirement for LP-SS OOK based Agreement: - At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation. Agreement: - When both MR and LR threshold are configured, MR or LR evaluates whether its own entry/exit threshold is satisfied separately. - MR and LR evaluation requirements are defined separately. Agreement: - Define a single LR evaluation requirement for both LR entry and exit threshold evaluation for LP-WUS monitoring, case 1 and case 3, i.e., LR evaluation requirement is not differentiated on entry or exit; or on different cases. - Note: RAN2’s discussion on timer will not impact on RAN4’s evaluation requirement. Agreement: - LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. - Number x and y are defined as measurement requirement as [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR) RAN4#114bis Agreement: - No dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin relative to the threshold in the core requirement. Agreement: For FR1: - [3.5] dB can be used for core requirements for LP-RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for LP-RSRP accuracy, under the side conditions Ês/Iot [x, x = -3] dB - [3.5] dB can be used for core requirements of SSB based RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for SSB based RSRP accuracy, under the side conditions Ês/Iot [y, y = -3] dB Note: [2 or 2.5] dB RF impairment margin is assumed Agreement: - Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. Agreement: - The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

# 4	SA: RRC_IDLE state mobility

## 4.x	Low Power Wake Up Radio Requirements

### 4.x.2	LP-WUR serving cell measurement and evaluation requirements

#### 4.x.2.1	General description

This sub-clause specifies requirements for LP-WUR serving cell measurements, measurement evaluation and measurement accuracy requirements based on LP-SS and PSS/SSS at Idle/Inactive state.

The requirements in the clause 4.x.2 apply when:

- The LP-WUR is in ON state. Before entering LP-WUS, RRM offloading and RRM relaxation modeand after exiting LP-WUS, RRM offloading and RRM relaxation mode, it is up to LP-WUS UE implementation when and how to turn the LP-WUR to ON state for serving cell measurement, and

- The LP-WUR LP-SS or PSS/SSS are configured on the same carrier as MR PSS/SSS

LP-WUR evaluation requirements in 4.8.2.2 and 4.8.2.3 apply for both LP-WUR entry and exit threshold evaluation.

#### 4.x.2.2	LP-WUR measurement and evaluation requirements for PSS/SSS

The requirements in this section apply for FR1 PSS/SSS based RSRP and PSS/SSS based RSRQ measurements configured for LP-WUR. The UE is required to evaluate conditions according to higher layer configuration.

The UE shall measure the SS-RSRP and SS-RSRQ level once every LO cycle, Tmeasure-LP-WUR-PSS/SSS as in table Table 4.x.2.2-1. The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurement samples. Within the set of measurements used for the filtering, at least two measurement samples shall be spaced by LO-periodicity / 2.

The UE shall be capable of evaluating that the following conditions defined in TS 38.304 [1], if configured, are met within Tevaluate-LP-WUR-PSS/SSS

-	[entry condition for LP-WUS monitoring]

-	[entry condition for RRM relaxation]

-	[entry condition for RRM offloading]

-	exit condition for LP-WUS monitoring

-     exit condition for RRM offloading

Table 4.x.2.2-1: Tmeasure-LP-WUR-PSS/SSS, and Tevaluate-LP-WUR-PSS/SSS

| LO periodicity [s] Note 1 | Tmeasure-LP-WUR-PSS/SSS [number of LO Cycles] | Tevaluate-LP-WUR-PSS/SSS [number of LO Cycles] |
| --- | --- | --- |
| 0.32 | 0.32 [1] | 0.32 * [y1] |
| 0.64 | 0.64 [1] | 0.64 * [y1] |
| 1.28 | 1.28 [1] | 1.28 * [y1] |
| 2.56 | 2.56 [1] | 2.56 * [y1] |
| Note 1: The LO periodicity is the same as the configured DRX cycle length |  |  |

Editor’s note: y1 is not yet agreed,

UE configured by higher layers for PSS/SSS shall evaluate and consider the corresponding entry criteria fulfilled within Tevaluate-LP-WUR-PSS/SSS, provided the LP-WUR measurement is higher than or equal the configured entry threshold by a margin [at least] [±5.5 or ±6] dB in FR1 when Ês/Iot ≥ -3dB

UE configured by higher layers for PSS/SSS shall evaluate and consider the corresponding exit criteria fulfilled within Tevaluate-LP-WUR- PSS/SSS, provided the LP-WUR measurement is lower than or equal the configured exit threshold by a margin [at least] [±5.5 or ±6] dB in FR1 when Ês/Iot ≥ -3dB.

Note: When a configured entry or exit condition is fulfilled, the UE shall perform corresponding actions as defined in clause XX in TS 38.304 [1].

#### 4.x.2.3	LP-WUR measurement and evaluation requirements for LP-SS

The requirements in this section apply for [FR1] LP-RSRP and LP-RSRQ measurements configured for LP-WUR. The UE is required to evaluate conditions according to higher layer configuration.

The UE shall measure the LP-RSRP and LP-RSRQ level once every LP-SS cycle, Tmeasure-LP-WUR-LP-SS as in table Table 4.x.2.3-1. The UE shall filter the LP-RSRP and LP -RSRQ measurements of the serving cell using at least 2 measurement samples. Within the set of measurements used for the filtering, at least two measurement samples shall be spaced by LP-SS-periodicity / 2.

The UE shall be capable of evaluating that the following conditions defined in TS 38.304 [1] are met within Tevaluate-LP-WUR-LP-SS

-	[entry condition for LP-WUS monitoring]

-	[entry condition for RRM relaxation]

-	[entry condition for RRM offloading]

-	exit condition for LP-WUS monitoring

-     exit condition for RRM offloading

Table 4.x.2.3-1: Tmeasure-LP-WUR--LP-SS , and Tevaluate-LP-WUR-LP-SS

| LP-SS periodicity [s] | Tmeasure-LP-WUR-LP-SS [number of LP-SS Cycles] | Tevaluate-LP-WUR-LP-SS [number of LP-SS Cycles] |
| --- | --- | --- |
| 0.16 | 0.16 [1] | 0.16 x * [x1] |
| 0.32 | 0.32 [1] | 0.32 x * [x1] |

UE configured by higher layers for PSS/SSS shall evaluate and consider the corresponding entry criteria fulfilled within Tevaluate-LP-WUR-LP-SS, provided the LP-WUR measurement is higher than the configured entry threshold by a margin [at least] [±5.5 or ±6] dB in FR1 when Ês/Iot ≥ -3dB

UE configured by higher layers for PSS/SSS shall evaluate and consider the corresponding exit criteria fulfilled within Tevaluate-LP-WUR- LP-SS, provided the LP-WUR measurement is lower than the configured exit threshold by a margin [at least] [±5.5 or ±6] dB in FR1 when Ês/Iot ≥ -3dB.

Note: When a configured entry or exit condition is fulfilled, the UE shall perform corresponding actions as defined in clause XX in TS 38.304 [1].

<End of change 1>