3GPP TSG-RAN WG4 Meeting #115	R4-2506138

Malta, MT, May 19th –May 23rd, 2025

Source:	Nokia

Title:	Discussion on LP-WUS performance requirements

Agenda item:	7.28.6

Document for:	Discussion

Introduction

This contribution discusses about testability of LP-WUS-WUR and in case the feature is testable, what to test.

RAN1 has previously made the following agreements:

Agreement Confirm the following working assumption with the addition of the note at the bottom: Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion.

Discussion

About testability of LP-WUS requirements

Based on the current RAN1 agreements the UE may monitor LP-WUS depending on whether the entry / exit conditions are fulfilled. This means that the UE requirements cannot be tested in such a way that the UE would be guaranteed to monitor LP-WUS signal, and pass 90% of the test runs. From this point of view, there are (at least) two options: Not to define test cases for LP-WUS or to define a test mode for LP-WUS functionality which guarantees that the UE enters the LP-WUS mode. The first option is not very desirable due to the feature not being deployable without testing.

The UE may choose whether to monitor LP-WUS based on RAN1 agreement. This means that the feature is untestable without a separate test mode.

Define test mode for LP-WUR functionality specified in the rel-19

The purpose of the test mode would be to guarantee that the UE enters correct state, for instance, monitors LP-WUS at the beginning of the test case. Similar approach can also be used for LR entry measurements as it is up to UE implementation when to start LR measurements.

LP-WUR entry measurements are untestable without a test mode.

Use test mode for LP-WUR entry requirements. FFS for the details of how to do this in RRM.

Scenarios to test

From the core requirements, we think that basic test coverage for the main features should be ensured. This would mean that PSS/SSS and LP-SS requirements would have similar but separate test cases.

Introduce separate sets of test cases for UE supporting PSS/SSS based LP-WUR and UE supporting LP-SS based LP-WUR

For both LP-SS and PSS/SSS based test cases, it is important to test the margin i.e., the LP-WUR accuracy defined in the requirements.

Introduce separate test cases to test LP-WUR accuracy for PSS/SSS and LP-SS

Evaluation period is one of the durations that should be tested. RAN4 should at least aim to test that the UE performs exit evaluation correctly and within the margin in core requirements. Whether this is embedded with the margin test case, or it is a separate test case can be further discussed.

Introduce a test case for to test LR Evaluation (delay) requirements

Since we now have MR relaxation of 16 x, we need to make sure that the cell reselection still works with the scaling factor. Therefore, we suggest testing MR relaxation with reselection with case 3.

Introduce a test case for MR reselection with case 3 and with 16x relaxation.

High priority frequency search test case should be included to make sure it is operational.

Introduce test case for high priority frequency search with case 1 and case 3.

Finally, RAN4 should make sure that entry and exit thresholds are tested for case 1 and case 3 as well as for LP-WUS monitoring thresholds. We should also make sure that the tests cover the scenarios where MR is used for entry and where both MR and LR are used for entry.

The LP-WUS exit can be tested so that the UE ends up initiating RACH procedure.

Introduce test cases for LP-WUS monitoring entry / exit with MR sync delays and paging reception.

Introduce MR only entry test case, and MR + LR entry test cases for RRM measurements.

Conclusion

The following Observations and Proposals were made:

Observation 1: The UE may choose whether to monitor LP-WUS based on RAN1 agreement. This means that the feature is untestable without a separate test mode.

Proposal 1: Define test mode for LP-WUR functionality specified in the rel-19

Observation 2: LP-WUR entry measurements are untestable without a test mode.

Proposal 2: Use test mode for LP-WUR entry requirements. FFS for the details of how to do this in RRM.

Proposal 3: Introduce separate sets of test cases for UE supporting PSS/SSS based LP-WUR and UE supporting LP-SS based LP-WUR

Proposal 4: Introduce separate test cases to test LP-WUR accuracy for PSS/SSS and LP-SS

Proposal 5: Introduce a test case for to test LR Evaluation (delay) requirements

Proposal 6: Introduce a test case for MR reselection with case 3 and with 16x relaxation.

Proposal 7: Introduce test case for high priority frequency search with case 1 and case 3.

Proposal 8: Introduce test cases for LP-WUS monitoring entry / exit with MR sync delays and paging reception.

Proposal 9: Introduce MR only entry test case, and MR + LR entry test cases for RRM measurements.