3GPP TSG-RAN4 Meeting #115	R4-2506142

# Malta, Malta, 19th May 2025 - 23rd May 2025

Source:			Ericsson

Title:			NTN Ku bands - bands numbering

Agenda item:		7.11.2

Document for:		Approval

# Introduction

In last RAN#107, the NTN KU-band WI was revised ([1]) including support for 15 kHz SCS.

In RAN4#114 meeting, RAN4 agreed ([2]) on the definition of 4 new NTN Ku-bands, 2 in FR1-NTN and 2 in FR2-NTN. This contribution is discussing about possibly synchronizing NTN Ku-bands numbering in both frequency range.

# Discussion

In RAN4#114 meeting, RAN4 agreed on the definition of the 4 following bands:

Following RAN decision, the intention was to specify any NTN Ku-band in both FR1-NTN and FR2-NTN so that both FR numerology could be used on the same frequency range.

It can be observed that:

- n509 is defined with the same frequency range than n248 but in FR2-NTN.

- n508 is defined with the same frequency range than n247 but in FR2-NTN.

Nothing is wrong with those definition. Nevertheless, it would be more convenient if there would some common similitude between the bands number in FR1-NTN and FR2-NTN when they are covering the same frequency range. We would then propose to use instead the band numbers n508 and n507 for the 2 FR2-NTN Ku-bands according to the following table:

| Satellite operating FR2-NTN band | Satellite operating FR1-NTN band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- | --- |
| n508 | n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127504 MHz | FDD |
| n507 | n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127504 MHz | FDD |
| NOTE 4: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. |  |  |  |  |

At least, the last digit of the FR1-NTN band and the FR2-NTN band would be common when both bands are covering the exact same frequency range. By doing so, we would then have:

- n508 defined with the same frequency range than n248 but in FR2-NTN.

- n507 is defined with the same frequency range than n247 but in FR2-NTN.

The NTN band number n509 could be reused for any new NTN band that will be defined in FR2-NTN.

Note that this might be difficult to maintain in the long term, if many other NTN bands are defined. But at least, at the beginning, this looks easy and help identifying the NTN Ku-bands defined with the same frequency range in FR1-NTN and FR2-NTN.

Proposal: To better recognize FR1-NTN and FR2-NTN Ku-bands covering the same frequency range, update the NTN Ku-bands number n509 and n508 with respectively numbers n508 and n507, as highlighted in the following table:

| Satellite operating FR2-NTN band | Satellite operating FR1-NTN band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- | --- |
| n508 | n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127504 MHz | FDD |
| n507 | n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127504 MHz | FDD |
| NOTE 4: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. |  |  |  |  |

# Conclusion

In this contribution, we discussed how to create some link between the FR1-NTN and FR2-NTN Ku-bands when they are covering the same frequency range to better recognize them. We made the following proposal:

Proposal: To better recognize FR1-NTN and FR2-NTN Ku-bands covering the same frequency range, update the NTN Ku-bands number n509 and n508 with respectively numbers n508 and n507, as highlighted in the following table:

| Satellite operating FR2-NTN band | Satellite operating FR1-NTN band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- | --- |
| n508 | n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127504 MHz | FDD |
| n507 | n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127504 MHz | FDD |
| NOTE 4: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. |  |  |  |  |

# References

- RP-250799, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Eutelsat