3GPP TSG-RAN4 Meeting #115	R4-2506148

# Malta, Malta, 19th May 2025 - 23rd May 2025

Source:			Ericsson

Title:			mmWave and EESS protection – TS 38.101-2 earlier Releases update

Agenda item:		6.17.2

Document for:		Approval

# Introduction

In RAN#105 meeting, a WI ([1]) was adopted to consider China and EU/CEPT Regulation related to the 24GHz bands updates and the EESS protection.

This contribution is further discussing the rationale of the updates made in the draft Rel-15 CR ([3]) endorsed in last RAN4#114bis meeting.

# Discussion

## Background

All BS and UE aspects have been clarified in previous meetings and the relevant CRs were agreed. Based on Rel-18 version, TS 38.104 Rel-19 is already aligned with RAN4 agreements and clarification on EESS protection in the 24GHz frequency range.

On the UE side, CR to TS 38.101-2 Rel-19 ([2]) was also agreed in past meetings capturing RAN4 agreed update to correctly consider existing worldwide regulation related to EESS protection in the 24GHz frequency range.

In last RAN4#114bis meeting, a draft CR to TS 38.101-2 Rel-15 was endorsed ([3]).

To our view, this draft CR and doesn’t need any further update:

- NS_202 definition shall be updated with the stringent limit applicable from 1st January 2024.

- NS_205 shall also be added to the specifications.

We are further elaborating on those 2 aspects in the following.

## NS_202 update

In previous version of TS 38.101-2 Rel-15, NS_202 was specified with the relaxed value to protect EESS in bands n257 and n258. From 1st January 2024, the stringent limit is now applicable, and any device brought on the market shall be compliant with this limit since then.

If the Rel-15 specification of NS_202 is not updated with the stringent limit, while the modifiedMPR Annex is updated referring to NS_202 Rel-19, this will create a major inconsistency in TS 38.101-2 Rel-15: the NS_202 definition will be ambiguous, referring to different limits in clause 6.5.3.2.3 and Annex H, which would be source of confusion.

To avoid any misunderstanding and ensure high quality of the specifications, NS_202 definition shall then be updated with the stringent limit in TS 38.101-2 Rel-15.

## NS_205 addition

NS_205 was not specified in previous version of TS 38.101-2 Rel-15. If NS_205 is only added to the modifiedMPR Annex H, this would create again some ambiguity and inconsistency.

Indeed, according to the modifiedMPR Annex (endorsed draft CR[3]), UE operating in bands n257 and n258, will have to support NS_205:

But, if TS 38.101-2 Rel-15 is updated without considering the NS_205 specification, there would be some confusion with the NS tables 6.2.3.1-1 and 6.2.3.1-2, where NS_205 would not be listed and n257/n258 would not be requested to support NS_205 either, as shown below:

Again, to avoid any misunderstanding and ensure high quality of the specifications, NS_205 definition shall be added to TS 38.101-2 Rel-15.

## Conclusion

Based on the above rationale, we would make the following proposals.

Proposal1: Agree on the TS 38.101-2 Rel-15 updates according to the endorsed draft CR R4-2505131, no other modification is needed.

Proposal2: TS 38.101-2 Rel-16, Rel-17 and Rel-18 updates shall be based on the endorsed draft CR R4-2505131 for Rel-15.

# Conclusion

In this contribution, we discussed the remaining open related to the WI on EESS protection in the 23.6-24.0 GHz frequency range. We made the following proposals:

Proposal1: Agree on the TS 38.101-2 Rel-15 updates according to the endorsed draft CR R4-2505131, no other modification is needed.

Proposal2: TS 38.101-2 Rel-16, Rel-17 and Rel-18 updates shall be based on the endorsed draft CR R4-2505131 for Rel-15.

# References

- RP-242371, WID on mmWave UE spurious emission, China Unicom, Deutsche Telekom, Vodafone, CATT, Huawei, HiSilicon, OPPO, Qualcomm, Xiaomi, ZTE

- R4-2501408, Big CR on UE RF requirements for WI NR_mmWave_protect, China Unicom

- R4-2505131, draft CR to 38.101-2 EESS (Rel-15), Huawei, Hisilicon, China Unicom, China Southern Power Grid, Ericsson, NTT DOCOMO, INC, Qualcomm Incorporated, ZTE, CATT