3GPP TSG-RAN4 Meeting #115	R4-2506149

# Malta, Malta, 19th May 2025 - 23rd May 2025

Source:			Ericsson

Title:			7 MHz channel bandwidth – UE RF and system impacts – RAN2 LS reply

Agenda item:		7.8.3

Document for:		Approval

# Introduction

In RAN#106, a WI ([1]) has been approved to specify 7 MHz channel bandwidth for NR bands n5 and n26.

Based on the agreed WF ([2]) from last RAN4#114bis meeting, this contribution shares our initial A-MPR simulation results. It also discusses the RAN2 LS Reply received by RAN4 ([3]).

# Discussion

## A-MPR simulation results

### Assumptions

The following assumptions have been considered for our A-MPR simulations:

- Power Class 3

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: -28 dBc

- I/Q image: -28 dBc

- CIM3: -60 dBc

### NS_12

As agreed in the WF ([2]), we considered 7 MHz deployment in the 814-824 MHz range and the additional requirement is -42 dBm/6.25 kHz in the 806-813.5 MHz frequency range.

The following A-MPR area and corresponding A-MPR value could be considered for NS_12 (QPSK):

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤2.7 | >0 | A1 |

### NS_13

As agreed in the WF ([2]), we considered 7 MHz deployment in the 817-824 MHz range and the additional requirement is -42 dBm/6.25 kHz in the 806-816 MHz frequency range.

The following A-MPR area and corresponding A-MPR value could be considered for NS_13 (QPSK):

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 820.5 | ≤2.16 | <1.62 | A4 |
|  |  | ≤2.16 | ≥1.62 | A3 |

### NS_14

As agreed in the WF ([2]), we considered 7 MHz deployment within the 824-849 MHz range and the additional requirement is -42 dBm/6.25 kHz in the 806-816 MHz frequency range.

Considering there is already 1dB MPR specified for QPSK, no A-MPR should be needed for NS_14 (QPSK).

## RAN2 LS

Replying to RAN4 LS ([4]), RAN2 sent another LS ([3]) requesting the following information:

From the LS Reply, RAN2 has not identified any problem to support 7 MHz channel bandwidth from Rel-15 and will reserve one of the spare bits to indicate this, to be captured in TS 38.306.

However, additional signalling would be needed for the UE to indicate if 7 MHz channel bandwidth is the minimum or the maximum bandwidth. RAN2 asks then RAN4 via this LS if this configuration should be considered and so if the corresponding additional signalling shall be introduced.

RAN4 discussed in past meetings that 7 MHz should not be the minimum channel bandwidth supported in a band (to avoid RAN1 impact). But, in the future, 7 MHz might be the maximum channel bandwidth in a band. Similarly, to be future proof, RAN4 should already consider that 7 MHz might be the minimum or the maximum channel bandwidth of a band in a bands’ combination.

Observation: 7 MHz could only be the maximum channel bandwidth in a band, not the minimum.

Proposal: Reply to RAN2 LS that 7 MHz could be the minimum or the maximum channel bandwidth supported by a band in a bands’ combination and the required signalling should be introduced in Rel-19 (or earlier release).

# Conclusion

In this contribution, we shared our A-MPR simulations results and made the following observation and proposal when replying to RAN2 LS:

Observation: 7 MHz could only be the maximum channel bandwidth in a band, not the minimum.

Proposal: Reply to RAN2 LS that 7 MHz could be the minimum or the maximum channel bandwidth supported by a band in a bands’ combination and the required signalling should be introduced in Rel-19 (or earlier release). 7 MHz could only be a maximum channel bandwidth in a band, not the minimum.

# References

- RP-243323, New WID on 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-Mobile USA

- R4-2505098, WF on requirements for 7 MHz channel bandwidth, Ericsson

- R4-2505319, Reply LS on Signalling for 7 MHz Channel Bandwidth, T-Mobile USA

- R4-2503017, LS on Signalling for 7 MHz Channel Bandwidth, T-Mobile USA

- R2-2502571 Introduction of 7MHz channel bandwidth, Ericsson , T-Mobile