3GPP TSG-RAN4 Meeting #115	R4-2506150

# Malta, Malta, 19th May 2025 - 23rd May 2025

Source:			Ericsson

Title:			7 MHz channel bandwidth – BS impacts

Agenda item:		7.8.4

Document for:		Approval

# Introduction

In RAN#106, a WI ([1]) has been approved to specify 7 MHz channel bandwidth for NR bands n5 and n26.

Based on the agreed WF ([2]) from last RAN4#114bis meeting, this contribution further discusses the only remaining open issue related to BS RF, which is the ICS requirement.

# Discussion

## Open issues

### In-channel selectivity

We propose to specify the ICS requirement based on the requirement for 5 MHz channel bandwidth, reusing G-FR1-A1-7 FRC and a 10RBs interferer, as shown in below table.

Table 7.8.2-1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-20 | -102.8 | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5, 7 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -98.7 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |

In last RAN4#114bis meeting, it was proposed to specify an interferer with 14 RBs, aiming to occupy as much as possible the full 7 MHz channel BW (35 RBs). The 14 RBs option was not possible as the interferer is a DFT-s -OFDM signal and its bandwidth shall comply with the basic DFT principle specified in TS 38.211. Nevertheless, we don’t think it would be needed to target occupying as much as possible the maximum transmission bandwidth configuration for 7 MHz channel bandwidth.

Indeed, comparing with the ICS requirement for 20 MHz (for example), the considered RFC has 25 RBs and the interferer has 25 RBs while the maximum transmission bandwidth configuration for 20 MHz is 106 RBs, meaning only 47% of the maximum transmission bandwidth configuration is occupied.

Considering our proposal (“reusing” 5 MHz channel BW requirement), the wanted signal has 15 RBs and the interferer has 10 RBs while the maximum transmission bandwidth configuration for 7 MHz is 35 RBs, meaning 71% of the maximum transmission bandwidth configuration is occupied, which is already much higher than the ICS requirement for 20MHz channel bandwidth.

Proposal1: Specify the In-channel selectivity requirements for 7 MHz channel BW based on the requirements for 5 MHz channel BW.

# Conclusion

In this contribution, we discussed the remaining open issue related to the BS specification when introducing 7 MHz channel bandwidth. We made the following proposal:

Proposal1: Specify the In-channel selectivity requirements for 7 MHz channel BW based on the requirements for 5 MHz channel BW:

Table 7.8.2-1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-20 | -102.8 | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5, 7 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -98.7 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |

# References

- RP-243323, New WID on 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-Mobile USA

- R4-2505098, WF on requirements for 7 MHz channel bandwidth, Ericsson