3GPP TSG-RAN WG4 Meeting #115	R4-2506164

# St Julian’s, Malta, 19th – 25th May 2025

Title:	Nominated BW use for 249 and freq error

Source:	Qualcomm Incorporated

Agenda item:	7.33.5

Release:	Rel-19

SI: 	IoT_NTN_TDD-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

RAN4 has received an LS from RAN1 [9] related to the Doppler pre-compensation. In the LS RAN1 confirms what Ran4 also agreed in previous meeting that the UE can perform the pre-compensation between UL bursts. In this paper we discuss how there maybe a better way to include the agreement in the normative specifications compared to what was agreed in WF [8].

We also discuss how to manage the emission requirements in ETSI EN 301 441 that is assumed to apply to this band.

# 2. 	Discussion

## 2.1 	Freq error

The essential part of the LS is this sentence: “The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes”

LS says “UE adjusts” where as the WF agreement was “Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts”

If the pre-compensation is a feature itself or an allowance or means to meet the Frequency error requirement is the key question. For example, if UE observes no Doppler shift, UE does not need to adjust anything, this would support using “allowed” and not using imperative in the spefication since if UE adjusts or not can not be observed anyway.

Observation: Pre-compensation is an allowance to the UE to meet the frequency error requirement.

The fundamental behaviour is described in TS 38.300.

RAN4 frequency error specification is written in a way that defines UE to perform the pre-compensation functionality. The clause is copied below with essential parts highlighted.

### 6.4B.1	Frequency error for UE category NB1 and NB2

For UE category NB1 and NB2, the UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift based on received ephemeris information of the SAN in IE EphemerisInfo (TS 36.331 [6]), its own location and UL carrier frequency signalled to the UE by the SAN (according to TS36.300 [8] clause 23.21.2.2).

The UE pre-compensated modulated carrier frequency shall be accurate to within the limits in Table 6.4B.1-1, observed over a period of one time slot (0.5 ms for 15 kHz sub-carrier spacing and 2 ms excluding the 2304Ts gap for 3.75 kHz sub-carrier spacing) and averaged over 72/LCtone slots (where LCtone = {1, 3, 6, 12} is the number of sub-carriers used for the transmission), compared to the ideally pre-compensated reference uplink carrier frequency.

When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift corresponding to the estimated Doppler frequency at the beginning of the transmission.]

Ran4 could focus on relieving the conditions when pre-compensation is not allowed in the specification as outlined in [11] and then address that. The agreement in WF does that however, the part where it implies adding the informative note may not be desirable for clear specification language. We propose below more streamline way to say what RAN4 agreed:

Proposal 1: Adopt the following language to accommodate Doppler pre-compensation between uplink bursts

When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 UE is allowed to perform pre-compensation before the beginning of the uplink burst.

Note that we implemented a change from “between uplink bursts” to “before the uplink burst” for the reason that this way UE can incorporate the updated information until the beginning of the uplink. One could argue that if the UE updates the pre-compensation right at the end of the uplink burst, then information it uses is expired before the beginning of the next uplink burst. This is also aligned with the LS text.

We are separately providing an LS reply to the LS from RAN1 indicating RAN4 has no issues with the working agreement RAN1 made for this feature. We also propose to cc RAN2 since the pre-compensation is specified in RAN2 maintained specification.

Proposal 2: Agree the text in [12] as reply to the LS [9]

## 2.2	Regulatory emission requirements

IN WF [8], the option 2 for regulatory requirements is states as ETSI EN 301 441 and ITU-R M.1343-1.

Luckily, requirements from these documents have been already written in 36.102 under NS_04N for band 254. There is an ongoing discussion how to define the AMPR for that band as the current specification has TBD for it. We are also proposing to use nominated bandwidth approach in our companion paper [11].

One way to handle the issue is to define a nominated bandwidth for 249 that is larger than the channel BW and this way the requirements could be met without any AMPR. This is outlined in Annex B of ETSI 301 441 The nominated BW would signify the edge where the ΔfOOB is applied. For 249, this is being discussed in [11]

Proposal 3: Use nominated bandwidth larger than channel bandwidth for 249 in the context of satisfying requirements  in ETSI EN 301 441 and ITU-R M.1343-1 that are currently under NS_04N in 36.102

This would need to be added to the TS 36.102 so that when ETSI creates a harmonized standard applicable for NTN TDD, the nominated bandwidth approach is applied correctly. RAN4 could define the use of nominated BW in an appendix similarly as ETSI 301 441 has.

Proposal 4: Define the use of nominated BW in 36.102

And in the process of doing so, RAN4 has to agree a suitable nominated bandwidth and we propose the following:

Proposal 5:  Use 400 kHz as nominated bandwidth for 249.

Significant amount of specification work should be done to properly write these requirements.

# Conclusion

We discussed Doppler pre-compensation for TDD NTN system and observed:

Observation: Pre-compensation is an allowance to the UE to meet the frequency error requirement.

And proposed based on RAN1 LS:

Proposal 1: Adopt the following language to accommodate Doppler pre-compensation between uplink bursts

When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 UE is allowed to perform pre-compensation before the beginning of the uplink burst.

Proposal 2: Agree the text in [12] as reply to the LS [9]

Proposal 3: Use nominated bandwidth larger than channel bandwidth for 249 in the context of satisfying requirements  in ETSI EN 301 441 and ITU-R M.1343-1 that are currently under NS_04N in 36.102

Proposal 4: Define the use of nominated BW in 36.102

Proposal 5:  Use 400 kHz as nominated bandwidth for 249.

# Reference

[1]	RP-243293, “Revised WID on introduction of IoT-NTN TDD mode”, Iridium Satellite LLC, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	R4-2419795, “Way Forward for [113][317] IoT_NTN_TDD”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, 18th – 22nd November, 2024

[3]	R4-1711536, “Reply LS to R1-1715304 LS on minimum time for DL-to-UL and UL-to-DL switching on one NB-IoT carrier for TDD NB-IoT UEs”, RAN4 (Ericsson), 3GPP TSG RAN WG2 Meeting #84bis, Dubrovnik, Croatia, 9th – 13th October 2017

[4] R1-2500003, Report of RAN1#119 meeting

[5]	R4-2502292, “Way Forward for [114][318] IoT_NTN_TDD_UE_SAN_RF”, THALES, 3GPP TSG-RAN4 Meeting #114 Athens, Greece,  16th – 21st February, 2025

[6]	R4-2500491, “Initial views on RF impact for NTN IoT TDD”, Qualcomm, 3GPP TSG-RAN4 Meeting #114 Athens, Greece,  16th – 21st February, 2025

[7]	R4-2501153, “AMPR value for NB-IoT in IoT NTN band b254”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #114, Athens, GR, 17th – 21st February, 2025

[8]	R4-2504719, “Way Forward for [114bis][316] IoT_NTN_TDD_UE_SAN_RF”, Thales, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[9]	R4-2505311, “LS on precompensation for NB-IoT NTN TDD mode”, RAN1, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th – 25th May 2025

[10]	R4-2503564, “Frequency error and UE regulatory requirements”, Qualcomm, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[11] R4-2506174, “On completing band 254 requirements”, Qualcomm,  3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th – 25th May 2025

[12] R4-2507733, “LS Reply on precompensation for NB-IoT NTN TDD mode”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th – 25th May 2025