# 3GPP TSG RAN WG4 Meeting#115	R4-2506168

# St. Julians, Malta, May 19 – May 23, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements of LP-WUS/WUR for NR

Agenda item:	7.28.7

Document for:	Discussion

# 1. Introduction

# The low power wake-up signal and receiver for NR were initially examined under a Study Item (SI) in Rel-18. This SI explored LP WUS, focusing on power-saving benefits and coverage, receiver structure, and higher layer protocols. Based on the findings from this SI, it was proposed to define LP-WUS/WUR requirements in Rel-19 [1]. In this paper, we share our perspectives on demodulation performance requirements for this WI.

2. General

2.1 Demodulation requirements

### Objective of Performance part WI

- Specify the UE demodulation performance and test cases for LP-WUS/WUR (RAN4).

  - Study testability of above requirements

Under the objective of the performance part, it is suggested to study the feasibility of demodulation requirements for LP-WUS/WUR. The relevant excerpt from the WID is provided below [1].

The scope of WI, as outlined in the WID, encompasses both RRC idle/inactive and RRC connected modes. We note that the LP-WUS triggers the paging monitoring occasion for MR when in idle mode and the PDCCH monitoring occasion when in connected mode. The WUR is expected to detect the WUS and notify the MR about the paging or PDCCH occasion based on the connection status.

Although LP-WUS transmission periodicity aligns with the UE DRX cycle, the performance of WUS detection or misdetection may vary based on UE implementation. Additionally, the WUR might wake up to receive a WUS even when no WUS transmission occurs. Consequently, the misdetection rate (MDR) or false alarm rate (FAR) can significantly affect the demodulation and power saving performance, respectively.

Observation 1: The WUR might fail to detect a WUS, resulting in a misdetection, or it might incorrectly indicate the presence of a WUS when no transmission has occurred.

Observation 2: A high MDR or FAR can adversely affect demodulation and power saving performance.

In our view, MDR is anticipated to have a more significant impact on demodulation performance, as in this case the MR will not monitor paging or PDCCH occasions. Conversely, FAR may not affect the demodulation performance but will result in a power penalty, as the MR will incorrectly wake up to monitor paging or PDCCH occasions. Therefore, it is more suitable to consider MDR as part of the demodulation performance requirements.

Proposal 1: Discuss the feasibility of introducing MDR requirements for LP-WUS/WUR.

Since the LP-WUS transmission periodicity aligns with the CDRX cycle, testing this scenario may require a significant amount of time to achieve acceptable MDR and FAR.

Observation 3: Given that LP-WUS transmission periodicity aligns with the CDRX cycle, achieving acceptable MDR may require a considerable amount of time.

Proposal 2: Evaluate the feasibility of testing MDR, considering the extended duration required to achieve acceptable MDR.

With acceptable MDR/FAR performance, the demodulation performance for PDSCH, PDCCH, PBCH, and CSI reporting requirements is expected to remain unaffected. Furthermore, MR will be tested using legacy test cases, eliminating the need for additional test cases to ensure performance.

Proposal 3: Do not introduce any PDSCH, PDCCH, PBCH, or CSI reporting requirements.

2.2 MDR testing

Given the alignment of WUS periodicity with CDRX periodicity, testing MDR in idle mode is quite impractical. Therefore, idle mode testing of MDR may not be feasible as it may take a long time to complete a test. An alternative approach is to test MDR in connected mode via PDCCH detection rate. We note that a certain minimum performance rate in PDCCH detection can inherently guarantee a corresponding MDR. For instance, a 1% PDCCH detection rate inherently ensures at least a 1% MDR.

Proposal 4: To test MDR, consider using the PDCCH detection rate, which will inherently guarantee MDR.

2.3 Choice of WUR

Based on the RAN1 design, there can be three different types of WUR depending on the UE capability. For example, a UE could declare capability for an OOO receiver only, an OFDM receiver only, or OOK overlaid with OFDM. The detection techniques for these receivers may differ and can impact detection performance, such as MDR. For instance, OOK can be based on envelope detection, whereas an OFDM receiver can be based on coherent detection. Therefore, it is essential to discuss the WUR types for the purpose of defining MDR.

Proposal 5: Discuss the WUR type to be considered for MDR testing.

3. Conclusions

In this paper, we share our perspectives on introducing demodulation requirements for LP-WUS/WUR. The following proposals have been made.

Observation 1: The WUR might fail to detect a WUS, resulting in a misdetection, or it might incorrectly indicate the presence of a WUS when no transmission has occurred.

Observation 2: A high MDR or FAR can adversely affect demodulation and power saving performance.

Proposal 1: Discuss the feasibility of introducing MDR requirements for LP-WUS/WUR.

Observation 3: Given that LP-WUS transmission periodicity aligns with the CDRX cycle, achieving acceptable MDR may require a considerable amount of time.

Proposal 2: Evaluate the feasibility of testing MDR, considering the extended duration required to achieve acceptable MDR.

Proposal 3: Do not introduce any PDSCH, PDCCH, PBCH, or CSI reporting requirements.

Proposal 4: To test MDR, consider using the PDCCH detection rate, which will inherently guarantee MDR.

Proposal 5: Discuss the WUR type to be considered for MDR testing.

4. References

[1] RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR),” 3GPP TSG RAN WG Meeting #105, Melbourne, Australia, Sept 9 – 12, 2025.