3GPP TSG-RAN WG4 Meeting #115	R4-2506172

Malta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on Spatial Channel Model for Demodulation Performance Requirements

Agenda item:	7.16.3

Document for:	Discussion

Introduction

The study item on Spatial Channel Model (SCM) for demodulation performance requirements (FS_NR_demod_SCM) resulted in the agreed WF [1] at RAN4#114bis.

This contribution addresses the open issues captured in the last WF [1] and the discussions surrounding it. Specific emphasis is placed on the following topics:

- CDL candidate implementation alignment

  - Definition and down selection between A2/A4 table variants.

  - Alignment criteria with outlier selection.

  - Aligning the simulation summary delivery.

- Confirmation of “A4” truncated CDLC table.

- Comparison cases

  - Performance Comparison of Methodologies vs. Channel Properties Comparison.

  - Comparison methodology

In this bis meeting we see it as the most important task to facilitate alignment of the CDL SCM candidate implementations, which is the basis for starting the comparisons.

CDL based methodology/model

In this section we try to agree on a common CDL based methodology; typically called 38.753 CDL. A lot of progress has been made in the last meetings, but some aspects are still to be clarified among options.

Terminology and naming conventions

Having commonly agreed upon naming conventions for options and modifications would be helpful in the discussions, alignment, and comparison to avoid confusion.

We use (and propose) the following conventions:

38.753 CDL: The final, hopefully single, CDL model candidate remaining at the end of the study. Including Tx/Rx AAV and CDL coefficient generation.

AAV: Antenna array virtualization, including AAV3, with one AE per BB port. AAV 1Y being the mapping of each BB port to a column of AEs, with overall arrar/panel size increasing with increasing number of ports.

CDL A2 (vanilla A2): CDL parameter table created by scaling/denormalizing TR 38.901 link level CDL tables with AS_desired and RMS_delay_desired.

CDL A2_20: Variant of A2 with all midpath related sub-clusters re-merged. It has 20 clusters remaining, instead of the 24 in A2.

CDL A2_12: Variant of A2 or A2_20, where truncation to 12 clusters but no quantization or rounding has been applied.

CDL A4: CDL A2_20 truncated and quantized to 12 clusters and 2ns, and angles truncated (or rounded) to one digit after the decimal point.

Channel model candidate tables

Following the agreements from the last meeting [1] there are two CDL channel model parameter table candidates considered.

- Option A2: based on Table 7.7.1-3 in TR 38.901 without truncation (and without quantization).

- Option A4: based on Table 7.7.1-3 in TR 38.901 with midpath combining, truncation (12 clusters) and quantization (2 ns).	
The derivation method is described in the appendix of [1], the AAV filtering step makes the table dependent on the specific AAV with which it will be used with.

Option A2

The A2 table has been quite stable for the last meetings, and is generally agreed to be

Table 1: Option A2 - UMa CDLC channel model parameters

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -38.222 | -96.938 | 96.764 | 98.141 |
| 2 | 76.6135 | -1.25 | -21.829 | 118.89 | 98.451 | 71.056 |
| 3 | 80.9935 | -3.4684 | -21.829 | 118.89 | 98.451 | 71.056 |
| 4 | 85.0085 | -5.2294 | -21.829 | 118.89 | 98.451 | 71.056 |
| 5 | 79.424 | -2.5215 | -34.158 | -124.42 | 100.86 | 67.561 |
| 6 | 232.359 | 0 | -5.9189 | 171.16 | 99.173 | 76.647 |
| 7 | 235.352 | -2.2185 | -5.9189 | 171.16 | 99.173 | 76.647 |
| 8 | 239.44 | -3.9794 | -5.9189 | 171.16 | 99.173 | 76.647 |
| 9 | 240.316 | -7.4215 | 44.223 | 51.882 | 106.4 | 62.843 |
| 10 | 289.6275 | -7.1215 | -50.55 | 63.395 | 94.476 | 56.552 |
| 11 | 299.7745 | -10.7215 | 49.113 | -42.068 | 107.48 | 69.832 |
| 12 | 340.764 | -11.1215 | -73.004 | 43.065 | 92.308 | 50.785 |
| 13 | 448.4025 | -5.1215 | -44.214 | 65.055 | 104.59 | 103.38 |
| 14 | 477.5295 | -6.8215 | -50.413 | -63.435 | 105.2 | 50.086 |
| 15 | 792.196 | -8.7215 | -60.193 | 78.954 | 91.706 | 51.659 |
| 16 | 989.3325 | -13.2215 | 64.61 | 26.262 | 105.2 | 121.03 |
| 17 | 1554.4985 | -13.9215 | 62.199 | -22.591 | 93.994 | 53.931 |
| 18 | 1679.1095 | -13.9215 | 55.036 | -1.6394 | 91.826 | 61.619 |
| 19 | 2003.923 | -15.8215 | -76.31 | -19.791 | 90.742 | 37.505 |
| 20 | 2046.8105 | -17.1215 | 57.378 | 4.4803 | 108.21 | 53.057 |
| 21 | 2301.8725 | -16.0215 | 58.136 | 0.22765 | 91.706 | 35.757 |
| 22 | 2422.651 | -15.7215 | 67.296 | -4.8548 | 91.585 | 52.882 |
| 23 | 2570.5855 | -21.6215 | 76.181 | -28.296 | 106.4 | 46.417 |
| 24 | 3158.0895 | -22.8215 | -91.394 | 29.27 | 109.53 | 44.669 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3775 | 15.558 | 3.6133 | 12.232 | 7 |  |

    - Option A4

The A4 table derivation methodology was agreed in the last meeting, and it has been mostly aligned offline between the meetings. However, we see a single different cluster in A4 for AAV1Y, so we present our derivation step by step in the following.

Starting point and wrapping

We start with the non-quantized and full floating-point accuracy A2 version (Table 1), which is the same for both AAV3 and AAV1Y.

This A2 table contains 10 degrees mechanical downtilt, Tx side AE array factor, and broadsight beam patterns (for 1Y). Furthermore, this table was derived using the wrapping rules implicitly contained in TR 38.827, which apply if ray directions of departures/arrival (DoDs/DoAs) are outside the ]-180º, 180º] and ]0º,180º] ranges expected for azimuth and elevation, respectively.

In our understanding, TR 38.827 per ray scaling can be expressed as follows:

ϕn,m,AOA,scaled=wrapASdesiredASmodel∙wrapϕn,m,AOA-μϕ,model+μϕ,desired

CASA,scaled=ASdesiredASmodel∙CASA,scaled

Where the function y = wrap(x), wraps angles into the interval ]-180º, 180º] and ]0º,180º] ranges for azimuth and elevation, respectively.

Note: Only elevation wrapping was needed in our derivation of A2, as well as, when deriving the 38.827 tables earlier.

Starting midpath re-merging

The next step is to take A20 and merge the mid-paths as described in the WF (followed by re-normalization of strongest cluster to 0dB). This results in a 20-cluster table, and is still independent of the chosen AAV:

Table 2: Option A2_20

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -97 | 96.8 | 98.1 |
| 2 | 76.5 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 79.3 | -5.5 | -34.2 | -124.5 | 100.9 | 67.6 |
| 4 | 231.9 | 0 | -5.9 | 171.1 | 99.2 | 76.7 |
| 5 | 239.8 | -10.4 | 44.2 | 52 | 106.4 | 62.9 |
| 6 | 289 | -10.1 | -50.6 | 63.5 | 94.5 | 56.6 |
| 7 | 299.2 | -13.7 | 49.1 | -42.2 | 107.5 | 69.8 |
| 8 | 340.1 | -14.1 | -73 | 43.2 | 92.3 | 50.8 |
| 9 | 447.5 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 10 | 476.5 | -9.8 | -50.4 | -63.6 | 105.2 | 50.1 |
| 11 | 790.5 | -11.7 | -60.2 | 79 | 91.7 | 51.7 |
| 12 | 987.3 | -16.2 | 64.6 | 26.4 | 105.2 | 121 |
| 13 | 1551.3 | -16.9 | 62.1 | -22.4 | 94 | 53.9 |
| 14 | 1675.6 | -16.9 | 55 | -1.5 | 91.8 | 61.6 |
| 15 | 1999.8 | -18.8 | -76.3 | -19.6 | 90.7 | 37.5 |
| 16 | 2042.6 | -20.1 | 57.3 | 4.6 | 108.2 | 53.1 |
| 17 | 2297.1 | -19 | 58.1 | 0.4 | 91.7 | 35.8 |
| 18 | 2417.6 | -18.7 | 67.2 | -4.7 | 91.6 | 52.9 |
| 19 | 2565.2 | -24.6 | 76.1 | -28.1 | 106.4 | 46.4 |
| 20 | 3151.5 | -25.8 | -91.4 | 29.4 | 109.5 | 44.7 |

AAV filtering

Based on A2_20, we then filter the clusters with the chosen AAV (either AAV_3 or AAV_1Y).

For AAV_1Y, the linear per cluster power after “AAV(8,1)” filtering is for us:

Table 3: AAV_1Y per cluster liner powers, 12 strongest highlighted in green, cluster 7 highlighted in blue.

| Cluster # | Power [linear] |
| --- | --- |
| 1 | 0.55668 |
| 2 | 4.4207 |
| 3 | 0.97112 |
| 4 | 7.7452 |
| 5 | 0.045109 |
| 6 | 0.14003 |
| 7 | 0.0068795 |
| 8 | 0.0096006 |
| 9 | 0.14129 |
| 10 | 0.042319 |
| 11 | 0.041936 |
| 12 | 0.0013473 |
| 13 | 0.013226 |
| 14 | 0.017299 |
| 15 | 0.0022239 |
| 16 | 0.00019793 |
| 17 | 0.0088516 |
| 18 | 0.0051181 |
| 19 | 1.87E-06 |
| 20 | 4.95E-06 |

Please note that cluster 7 seems to be the “12th” strongest cluster selected in the email discussion for AAV_1Y.	
We invite other companies to share and compare their post-AAV-filtering cluster powers to compare this discrepancy.

Our “A4 for AAV_1Y” parameter table differs in one cluster from the circulated version. We have disclosed post-AAV filtering/pre-truncation cluster powers to compare with other contributors. Our “A4 for AAV_3” table matches the circulated version.

Truncation and quantization

The final steps to achieve A4 include truncating A2_20, based on the 12 strongest clusters identified prior, followed by quantization and rounding. This results in the following tables for AAV_3 and AAV_1Y:

Table 4: Option A4_AAV3 (1,1).

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -97 | 96.8 | 98.1 |
| 2 | 132 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 138 | -5.5 | -34.2 | -124.5 | 100.9 | 67.6 |
| 4 | 402 | 0.0 | -5.9 | 171.1 | 99.2 | 76.7 |
| 5 | 416 | -10.4 | 44.2 | 52.0 | 106.4 | 62.9 |
| 6 | 502 | -10.1 | -50.6 | 63.5 | 94.5 | 56.6 |
| 7 | 518 | -13.7 | 49.1 | -42.2 | 107.5 | 69.8 |
| 8 | 776 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 826 | -9.8 | -50.4 | -63.6 | 105.2 | 50.1 |
| 10 | 1372 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 11 | 1712 | -16.2 | 64.6 | 26.4 | 105.2 | 121.0 |
| 12 | 2906 | -16.9 | 55.0 | -1.5 | 91.8 | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.4 | 15.5 | 3.6 | 12.2 | 7 |  |

Table 5: Option A4_AAV1Y (8,1).

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -97 | 96.8 | 98.1 |
| 2 | 122 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 126 | -5.5 | -34.2 | -124.5 | 100.9 | 67.6 |
| 4 | 370 | 0.0 | -5.9 | 171.1 | 99.2 | 76.7 |
| 5 | 382 | -10.4 | 44.2 | 52.0 | 106.4 | 62.9 |
| 6 | 462 | -10.1 | -50.6 | 63.5 | 94.5 | 56.6 |
| 7 | 542 | -14.1 | -73.0 | 43.2 | 92.3 | 50.8 |
| 8 | 714 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 760 | -9.8 | -50.4 | -63.6 | 105.2 | 50.1 |
| 10 | 1260 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 11 | 2474 | -16.9 | 62.1 | -22.4 | 94.0 | 53.9 |
| 12 | 2672 | -16.9 | 55.0 | -1.5 | 91.8 | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.4 | 15.5 | 3.6 | 12.2 | 7 |  |

All clusters are then treated as “weak cluster”, i.e., no ray-splitting is applied in link level simulations.

During our derivation of the tables, we made the following observations.

Flooring of numbers in the dB-domain may introduce large absolute linear errors, rounding is generally preferred.

Different AAVs result in different parameter tables.

Following all the above table derivation observations, we finally propose

For A4 table derivation: (1) Use per ray wrapping, (2) use rounding of values not flooring, (3) keep full floating-point precision until last step, (4) confirm AAV_1Y selected clusters after AAV-filtering.

Comparison of A2 and A4

In this section we will make our case for selecting the final TR 38.753 CDL channel model among the options A2 and A4, which were derived and presented in the prior section.	
To provide a basis for our reasoning we will highlight some simulation results, delivered in our companion tdoc [2], here to compare the channel options.

PDSCH 8T8R8L

Figure 1: PDSCH 8T8R8L: A2 vs. A4, rand PMI, AAV 1Y (top) +AAV3 (bottom), 10Hz.

We observe

In 8T8R8L. A2 and A4 perform similarly, with AAV3 having a slight advantage due to higher diversity. Both A2 and A4 have enough degrees of freedom to support 8 layers. AAV3 has slightly less inter-CW gap, i.e., less spatial effects.

Additionally, the higher SNR saturation of the prior A3 models is no longer observed in A4, as the condition number of the channel is no longer artificially curtailed to be below rank 8.

PDSCH 4T4R4L

Figure 2: PDSCH 4T4R4L: A2 vs. A4, rand PMI, AAV 1Y (top) + AAV3 (bottom), 10Hz.

We observe

In 4T4R4L, A2 and A4 perform quite similarly in random precoding. Under AAV 1Y random precoding is still similar, but fixed precoding impacts A4 much more significantly.

PMI 8T4R4L

Figure 3: PMI 8T4R4L: A2 vs. A4, rand PMI, AAV 1Y (top) +AAV3 (bottom), 10Hz.

We observe

In eTypeII follow PMI 8T4R4L with AAV 1Y, we see A2 and A4 performing relatively the same. Using AAV 3, the channel model A4 performance better than A2.

MU PDSCH 4T4R2+2L

Figure 4: MU PDSCH 4T4R2+2L: A2 vs. A4, random typeI PMI, AAV 1Y (top) / AAV3 (bottom), linear IRC vs. eIRC.

We observe

In MU PDSCH 4T4R2+2L under both AAV 1Y and A3, with linear IRC vs. eIRC, we see that both A2 and A4 barely exhibits a gap between IRC and eIRC performance. Albeit that A4 has a bigger gap at low and high SRN. This is due to higher diversity/better channel condition in A2, which “naturally decorrelates/statistically orthogonalizes” the two UEs even with SU linear receivers, which is what we expect from deployment. A4 is lacking some diversity and thus receivers need to implement inter-UE IRC to perform optimally. However, the gap may be within acceptable limits

Conclusion

Thus, we propose

RAN4 to retain either A2 or A4 as candidate option for CDL based SCM, with slight preference for A2, but both are feasible.

Note: MU PDSCH shows the weakness of relatively lower spatial diversity in A4, when compared to A2. This would need to be considered in future applications of A4.

Extensions for MU-MIMO

To fully support MU-MIMO use cases and channel scenarios, the spatial consistency of CDL model for more than one UE should be considered.

In the channel scenario for MU-MIMO case, more than one UE is involved in the same cell or in two neighbor cells for the network. Therefore, the physical channel environment should be considered as special consistent.

It means that if two UEs are served as MU-MIMO cases, both UEs should see the same clusters from their own physical locations in the wireless environment. The key aspect in the channel models is that the cluster generated for both the CDL model #1 used by UE1 and the CDL model #2 used by UE2 should be the same clusters. Then, it is considered reasonable to have some variation on cluster power, angle, and delay between those two CDL models.

In MU-MIMO scenario related to two UEs, the CDL model #1 used by UE1 and the CDL model #2 used by UE2 should contain the same clusters. Then, it is considered reasonable to have some variation on cluster power, angle, and delay between those two CDL models.

For the equation of generating the channel coefficients of fast fading model in TR 38.901 for MU-MIMO case can consider generating one channel model first, then generate the second channel model based on the modification of the first one by adjust the λ(f):

Similarly, for using the CDL tables, i.e., CDL-A/B/C/D/E in TR 38.901, to generate two CDL models for both UE1 and UE2, the second CDL model should be adjusted based on the first CDL model by power, delay and angles. However, due to lack of the description in the standard, then it is implementation agnostic.

The spatial consistency should be considered for MU-MIMO scenarios during the simulation. The channel model generated for both UE1 and UE2 should be based on the same clusters. One model can be generated first and the other model can be adjusted based on the first model.

Alignment of spatial channel model implementations

Follow up on SNR normalization

In [1] it was requested to bring the “linear channel gain factor” (average squared Frobenius norm of all REs) again, for implementation alignment intermediate steps:

SNR Normalisation with CDL Way Forward Assess results alignment at RAN4#115, FFS on the method to define normalisation method if required. Encourage companies to provide normalisation factor at next meeting and align accordingly. Agreement Simulation results to be provided with AAV gain (including AE pattern) normalized, such that AAV gain is taken away as part of the channel normalization.

Our channel implementation is fully in the time domain and is applied in the time sample domain. However, we have attempted to convert the “simulation measurements” to the RE domain for comparison. 	
The resulting channel gain values being:

| AAV, Tx/Rx | A2 | A4 |
| --- | --- | --- |
| AAV1Y, 8x8 | 17.9115 | 18.3277 |
| AAV1Y, 8x4 | 24.6467 | 25.8269 |
| AAV1Y, 4x4 | 18.6632 | 15.8527 |
| AAV3, 8x8 | 5.7732 | 6.1704 |
| AAV3, 8x4 | 6.486 | 6.6592 |
| AAV3, 4x4 | 5.9241 | 6.9328 |

Table 6: Long term average channel model gains.

Assumptions and definitions:

- Values here are given in the linear power domain and constitute the average per RX branch.

- 1/MeNe AAV weight normalization for AAV 1Y.

- Average over the whole simulation duration (10’000 slots).

- Average over 100 random starting seeds (all seeds converge to same value eventually)

Alignment criteria and thresholds

We remind ourselves that the “alignment” step of this study is to align the SCM implementations between different contributors. The comparison between the SCM candidates and legacy models is a separate, but not independent, step that we agreed to call “comparison”.

Criteria for PxSCH simulations

Looking back at Rel-15 (NR_newRAT), it was generally understood to check for alignment at the operating point of the SNR results delivered for requirement derivation.

For UE DMD the (ideal) results were immediately accepted as aligned, if the SNR points were within 2.5dB span and larger spans were treated on a case-by-case basis [3].

For BS DMD, the (ideal) results were declared aligned, if there were at least 3 contributions within 2dB (later often extended to 2.5dB) [4]. Outlier removal was/is automated.

Precoding was usually chosen to be random in PDSCH, and fixed identity in PUSCH requirement delivery.

We would like to propose a similar alignment criteria and thresholds to aim for in this study:

For SCM implementation alignment in PxSCH simulations, aim to have at least 3 contributions be within 2.5dB of SNR at the [70]% TPUT operating point, under random precoding. Encourage evaluation of full curves to decide the relative TPUT operating point for alignment.

Note: The “have at least 3 contributions be within 2.5dB of SNR” step above constitutes outlier removal.

Criteria for PMI simulations

For PMI results alignment, we could not find as clear-cut agreements in Rel-15 as are available for PxSCH results. However, we assert it being reasonable to use the same approach, under follow PMI, and further discussion on the operating point. Note that follow PMI selection implementation is not specified and brings in another element of uncertainty, hence requiring larger alignment thresholds.

The initially delivered Rel-15 results for PMI were of varying alignment degrees and depending on the number of RX branches presenting spans of up to 3.5dB; 3 contributors were taken as sufficient [5].

For SCM implementation alignment in PMI simulations, aim to have at least 3 contributions be within [3.5]dB of SNR at the [TBD]% TPUT operating point under [type1 and/or eType2] follow precoding in 4Rx conditions. Encourage evaluation of full curves to decide the relative TPUT operating point for alignment.

We explicitly encourage outlier removal and discourage alignment on gamma values as this value showed a high degree of variability in Rel-15. Thus, we prefer to align on “follow TPUT”.

Note also that in [6] it was agreed to “First check alignment with PMI Random (type I)”, which is counter to our PMI proposal above.

Decision on implementation alignment

A decision can only be taken after collecting simulation results and giving ample opportunity for bug-fixes and sharing information (e.g., via full TPUT curve delivery).

  - Simulations alignment subset

The simplified simulations spreadsheet exercise for SU cases in RAN4#114bis, gave a quick overview and we propose to repeat it.

Repeat the simplified simulations subset/spreadsheet exercise for SU cases from RAN4#114bis.

Comparison of spatial channel model candidates and legacy

Legacy channel configuration

Initial agreements on legacy channel configurations for comparison have been discussed in [6]:

#### Comparison with Legacy TDL Agreement: The following Legacy TDL models for comparison: - Option 1: TDLC 300-10/100 with following correlation: - Option 1a: Low - Option 1b: XP Medium - Option 1c: XP High - Option 1d: Medium

We see it as important to cover the most commonly used legacy channel models in this studies comparison. This can help to illuminate potential blind spots in our current performance requirements. In particular, blind spots that could be avoided with a reliable channel model, whose default behaviors cover effects without additional effort in model tailoring/adaptation.

Additionally, the legacy “XP” and “low” configurations are much less impacted by the questionable ULA antenna correlation formulation, which scales unrealistically with number of antennas and does not consider phase. Finally, both “XP” and “low” are rarely used in current performance requirements.

Comparison to cover the most commonly used legacy channel models, for example TDLC 300-10/100 with low and medium correlation. Alternatively, TDLA 30-10 med and high correlation.

PDSCH fixed PMI selection for comparison

In RAN4#114bis, it was agreed to have fixed PMI as FFS for comparison [1]:

PDSCH ‘Fixed’ PMI Selection Way Forward For alignment a random PMI will be used, for comparison cases with a ‘fixed’ PMI FFS on approach.

In our earlier contributions, a non-random precoder choice was found to be useful to capture the impact of typical MIMO effects, e.g., quality difference between MIMO layers and midterm stable spatial preferences, on the receiver implementation performance (e.g., regularization strategies, spatial distinction via covariance matrix estimation, or other means).	
Any non-random precoding strategy, combined with a spatial channel model, can serve to model these effects, but for requirement and test simplicity it is desirable to have a procedure to arrive at a performance comparable fixed precoding choice between different implementations and contributors.

In RAN4#114 a number of options was discussed concerning how a fixed PMI could be chosen [6]:

#### PDSCH PMI Selection Way Forward: Random PMI already agreed. Regarding potential fixed PMI: - Option 1: Random PMI from sub-set - Option 2: Collectively chosen fixed PMI - Option 3: Independently chosen fixed PMI Note: Companies to propose detailed method description for interested options for RAN4#114-bis.

We are a proponent of option 3 (“independently chosen fixed PMI”), as we reason that it is the easiest method to align.

With option 2 (“collectively chosen fixed PMI”) the exact implementation of BB port to SCM AEs may cause differences in PMI to spatial transmission characteristics, which becomes another source of error for alignment. E.g., while two SCM implementations may be equivalent from a performance point of view, they may result in different spatial characteristics of the same PMI.	
As such fixing the PMI value collectively will make the alignment more complicated that necessary.

Concerning option 1 (“random PMI from sub-set”), this seems to be a mixture of option 2 and 3. I.e., select a subset of PMIs that are seen in all implementations to be aligned with the spatial properties of each implementation, and then select randomly amongst them so on average the fixed PMI (set) works for all implementations.	
The performance of this approach seems hard to predict, and this approach will require additional work to select the sub-set.

Hence, we propose

RAN4 to allow for independently chosen fixed PMI.

However, we also want to propose how to independently choose the fixed PMI across multiple implementations.

It is feasible to make a fixed PMI choice based on the most frequently chosen “PMI_follow” under any given channel model that exhibits midterm stable large-scale parameters. This is achieved, using the most common PMI from a histogram of signaled/chosen follow PMIs (see section 3 of [7]) at reasonably high relative throughput operation points.

Please find hereunder the PMI_follow histogram for CDL 827 (previous A1 option) + AAV 3, as an example of the approach we used to derive the fixed precoder in our results collection contributions:

827 CDLC UMa – AAV 3 SNR = 22dB TP CW0 = 97% TP CW1 = 92%

Figure 5: 827 CDLC UMa – AAV 3 follow PMI histogram.

With this approach PDSCH performance is comparable between implementation, without disclosing or mandating a specific PMI that might not fit the expectations of the receiver algorithm.

RAN4 to choose the most common PMI from a histogram of signaled/chosen follow PMIs as fixed precoder in SU PDSCH SCM evaluations at about [95]% TPUT operating point.

The selection of the SNR operating point for the SNR histogram should not be chosen to high (i.e., for 100% TPUT) as follow PMI selection algorithms may perform erratically, if many PMI achieve 100% TPUT. Similarity, a SNR operating point that is too low, may obscure the fixed PMI choice that match mid-term stable large scale channel directions.	
A value of 95% (or 90%) was observed to deliver stable results.

AAV choice and number of Tx for comparison

Following last meeting’s agreement to use AAV3 for alignment, the next open issue is the AAV choice for comparisons.

We understand that it will be required to have a look at the first set of comparison cases, before a comparison AAV choice can be made. Especially, it depends on the inclusion of 32Tx, if an AAV_3 (with lambda/2 vertical spacing) and AAV_3 (with 4lambda vertical spacing) need to be distinguished, as to observe anticipated eTypeII over typeI CB gains.

Include the first comparisons with both AAV_3 and AAV_1Y, before downselecting to a single CDL candidate in the study.

Include 32Tx in the first comparisons, before downselecting to a single CDL candidate in the study.

Comparison method and TR conclusions

In general, channel model performance comparisons cannot be reduced to a simple numerical metric. Such numeric metric comparisons are only feasible, if a ground truth is present (e.g., known good measurement results to align with).	
Comparison in our case refers to evaluating in a relative sense, whether the channel model allows for detection and minimum performance bounding of baseband features, where the baseband feature performance should be sensitive to typical MIMO effects. I.e., whether such typical MIMO effects are present or not, should be visible in the relative performance scaling of the feature under test.	
However, without a ground truth, there is a degree of subjectivity in the comparison of channel model candidates for RAN4 demod minimum performance requirements.

Hence, we propose to select a number of use cases/feature configurations, where multiple proponents agree that spatial channel effects in deployment should have an impact on use case performance. Then the study item collects subjective evaluations from all contributors using the following procedure:

Figure 6: Comparison results collection procedure.

Or, in a less comprehensible written proposal form:

To compare channel models, have each contributor set performance or channel property expectations for each comparison case. Then, each contributor compares their observations from simulations against their expectations, to subjectively decide whether channel models fit expectations. Context on importance of expectations for RAN4 minimum performance requirements is also given. See block diagram for simpler reading.

The TR conclusion would then collect and aggregate/cluster the observations and contextualization given by all contributors:

Figure 7: Comparison results collection procedure.

Or, in a less comprehensible text proposal form:

The TR conclusion to capture comparison results by collecting and aggregating the feedback given by all contributors. Additionally, it may be attempted to make a general statement on the necessity of SCMs in performance requirements setting and to select one specific SCM candidate in particular.

Performance and channel properties

The question of metrics for performance comparison of test cases under different channel models has been brought up many times during the past meetings. Until now we did not fully formalize (nor did we need to formalize) such metrics, as SCM candidates were not stable enough to predict their behaviours.	
Our intent was to check all channel models in MIMO test cases, with respect to whether typical MIMO effects that are either observed in deployment or expected from MIMO systems, are well reflected in the simulation output.

In the last meeting it was agreed to make comparisons in two groups of performance metrics [1]:

#### Performance Metrics #### Way Forward: #### Channel Model candidates to be compared in two aspects: - Performance Properties - Channel Properties (captured in an annex)

In our understanding, the former analyzes the impact on potential demodulation performance figures, while the latter directly compares with channel property measurements and expectations contributed to prior meetings.

In our prior contributions (e.g. [8]) we have highlighted several trends that we attribute to expected typical MIMO effects and can be taken as a basis for performance properties comparison metrics; assuming said trends are replicated by other contributors.

To be more specific, expect to see at least most of the following typical MIMO effects in a demodulation channel model:

- SU PDSCH 2CW

  - SDM receiver performance differences per CW.

- SU PDSCH 1CW

  - Gains of fixed/follow precoder over random precoder.

  - SDM receiver performance (through covariance estimation) impacted by channel evolution rates.

- MU PDSCH

  - Similar performance of linear receivers with or without inter-user IRC.

  - Linear receivers perform worse, but not unreasonably so, than non-linear receivers.

- SU PMI

  - CSI report selection performance and implementation impacted by channel evolution rates.

  - Relative CB PMI selection performance difference impacted by number of layers.

- MU PMI

  - CSI report selection performance and implementation impacted by interference presence (through receiver SDM bias choice).

- All (also non-MIMO)

  - Typical SNR operating points.

Additional expectations for demodulation channel models, outside of typical MIMO effects, are:

- Performance trends shall be explainable from channel model construction.

- Alignment with channel measurements, e.g., mid-term stable principal directions and angle diversity (subjective).

- Alignment with performance in deployment (subjective).

- Alignment with RAN1 feature development assumptions (subjective).

- Usefulness of model RAN4, e.g., re-use by AIML work, etc. (subjective).

Performance wise comparison

The last meeting’s first stab at defining performance properties is captured in the WF [1]:

Performance Properties include: - [TPUT with follow precoding] - [TPUT with fixed precoding] - [TPUT with random precoding] - [Metrics Statistics and Spread] - [SNR Operating Point] - [Convergence time]

These properties are valid performance properties; however, we need to define complete comparison cases to be able to give feedback on channel models. Specifically, we propose:

PxSCH focused comparison cases

SU PDSCH 2CW:

Compare the per CW TPUT vs. SNR curves of an 8T/8R/8L PDSCH setup with random and follow precoding, given different channel model candidates, by collecting feedback from the group on what TPUT vs. SNR scaling would be expected in deployment and how well the channel model candidates match expectations.

SU PDSCH 1CW:

Compare TPUT vs. SNR curves of an 4T4R4L PDSCH setup with random precoding and [3] & [30]kph mobility given different channel model candidates, by collecting feedback from the group on what TPUT vs. SNR vs. mobility scaling would be expected in deployment and how well the channel model candidates match expectations.

MU PDSCH:

Compare TPUT vs. SNR curves of an 4T4R2+2L MU PDSCH setup with random (but not overlapping) precoding and linear receiver with and without inter-user IRC and R-ML receiver, given different channel model candidates, by collecting feedback from the group on what TPUT vs. SNR vs. receiver implementation scaling would be expected in deployment and how well the channel model candidates match expectations.

All PDSCH:

Compare SNR operating point levels of an 4T4R4L PDSCH setup given different channel model candidates. Contributors to state what operating points would be expected in deployment, as well as are feasible for test chambers, and how well the channel model candidates match expectations.

PMI and other comparisons

Considering that there is a limit to how many use cases can be discussed at one meeting, we would leave further cases (esp., PMI related) to the next meeting.

Though we already note that using gamma values vs. SNR, for PMI cases, may not be a good choice as gamma was rather unstable and misaligned in Rel-15. Additionally, it was often seen to not reflect expected gains. Follow vs. follow PMI TPUT curves are likely a better choice.

Discuss additions to the performance comparison use cases, e.g., with PMI cases using follow PMI of several CBs.

Channel properties wise comparison

The last meeting’s first stab at defining performance properties is captured in the WF [1]:

Channel Properties include: - [Eigenmodes and Eigenvectors] - [Power Angular Distribution] - [Time Coherence] - [Frequency Coherence] - [PMI Preference]

As was more deeply discussed in the prior section, during the last meeting it was proposed to split comparison into “performance comparison” and “channel properties comparison”. The latter can openly compare channel property measurements and expectations contributed to prior meetings (e.g., [9] and [10]) with SCM candidate observations.	
In our understanding channel properties refer mostly to physical properties -as well as- statistical properties of physical aspects, signal processing related quantities (e.g., SINR), as well as test relevant quantities (e.g., convergence time).

Hence, we would like to compare the channel candidates vs. each other vs. measurements at least with respect to

- DoD and DoA direction, powers, diversity, and stability

  - Can be shown via Bartlett/ESPRIT/etc. analysis.

  - Sometimes call PAS (power angular distribution).

- Post-EQ SINR histograms.

- Convergence times.

- Optionally, frequency coherence.

As with performance comparisons, there is a subjective element, and comparison is carried out by expressing expectations and subjective alignment with said expectations.

Compare channel properties in terms of at least DoD and DoA direction, powers, diversity, and stability, as well as post-EQ SINR histograms and convergence times of channel candidates vs. each other, and vs. measurements.
This is done by collecting feedback from the group on expectations from deployment or measurements, as well as feedback on how well the channel model candidates match expectations.

Priorities for comparison cases

In our prior contributions (e.g. [8]) we have highlighted several trends that we attribute to expected typical MIMO effects. Following these earlier results, we see the highest priorities for comparison cases as follows:

- SU MIMO PDSCH: 8L/2CW TPUT.

- MU MIMO PDSCH: Advanced IRC receivers (e.g., IRC vs. eIRC vs. R-ML) TPUT.

- SU PMI: 4L eTypeII vs. TypeI follow PMI TPUT.

- MU PMI: 2+2L eTypeII vs. TypeI follow PMI TPUT.

If a prioritization of comparison cases is required, then we shall follow the order (1) SU PDSCH 2CW, (2) MU PDSCH IRC vs. eIRC vs. R-ML, (3) SU PMI 4L eTypeII vs. TypeI follow PMI (4) MU PMI 2+2L eTypeII vs. TypeI follow PMI.

Potential optional requirements

Since this is a study item, no requirements will be introduced. However, a study can and shall make recommendations on how potential requirements could be introduced by a future work item, in ways that conform to any findings made during the study.

It has been discussed in this study that SCM introduction would likely require an update of the established 5G conformance testing setup, hardware, and/or software. Additionally, reservations have been raised whether specific algorithmic updates to DUTs may be necessary to adhere to new SCM based performance requirements (even though any algorithms that work in practical deployment will not have issues with a spatial channel model).

From our perspective it seems clear that any potential 5GA requirements should be encapsulated, using an optional UE capability (or BS manufacturer declaration) and applicability rules. For example:

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| SCM compatible MIMO receiver | FR1 TDD | PDSCH | Clause X.X.X Test X-X |  |

This is similar to the current requirement status of the “Simplified SU-MIMO 8Rx receiver” UE capability and applicability rule.

Or put in the proposal form:

The study TR shall contain a recommendation that any SCM requirements in 5GNR are recommended to be optional via optional UE capability/BS declaration and corresponding applicability rules.

Conclusion

In this contribution we addressed the open issues captured in the last meeting’s WF and the questions raised in the discussions surrounding it, but specific emphasis is placed on the following CDL candidate implementation alignment (including down selection of variants and ways to improve facility alignment outcomes). Comparison setups and criteria have been discussed as well.

In summary, we made the following observations and proposals:

= CDL based methodology/model

== Channel model candidate tables

=== Option A4

- Our “A4 for AAV_1Y” parameter table differs in one cluster from the circulated version. We have disclosed post-AAV filtering/pre-truncation cluster powers to compare with other contributors. Our “A4 for AAV_3” table matches the circulated version.

- Flooring of numbers in the dB-domain may introduce large absolute linear errors, rounding is generally preferred.

- Different AAVs result in different parameter tables.

- For A4 table derivation: (1) Use per ray wrapping, (2) use rounding of values not flooring, (3) keep full floating-point precision until last step, (4) confirm AAV_1Y selected clusters after AAV-filtering.

== Comparison of A2 and A4

In 8T8R8L. A2 and A4 perform similarly, with AAV3 having a slight advantage due to higher diversity. Both A2 and A4 have enough degrees of freedom to support 8 layers. AAV3 has slightly less inter-CW gap, i.e., less spatial effects.

In 4T4R4L, A2 and A4 perform quite similarly in random precoding. Under AAV 1Y random precoding is still similar, but fixed precoding impacts A4 much more significantly.

In eTypeII follow PMI 8T4R4L with AAV 1Y, we see A2 and A4 performing relatively the same. Using AAV 3, the channel model A4 performance better than A2.

In MU PDSCH 4T4R2+2L under both AAV 1Y and A3, with linear IRC vs. eIRC, we see that both A2 and A4 barely exhibits a gap between IRC and eIRC performance. Albeit that A4 has a bigger gap at low and high SRN. This is due to higher diversity/better channel condition in A2, which “naturally decorrelates/statistically orthogonalizes” the two UEs even with SU linear receivers, which is what we expect from deployment. A4 is lacking some diversity and thus receivers need to implement inter-UE IRC to perform optimally. However, the gap may be within acceptable limits

RAN4 to retain either A2 or A4 as candidate option for CDL based SCM, with slight preference for A2, but both are feasible.

== Extensions for MU-MIMO

In MU-MIMO scenario related to two UEs, the CDL model #1 used by UE1 and the CDL model #2 used by UE2 should contain the same clusters. Then, it is considered reasonable to have some variation on cluster power, angle, and delay between those two CDL models.

The spatial consistency should be considered for MU-MIMO scenarios during the simulation. The channel model generated for both UE1 and UE2 should be based on the same clusters. One model can be generated first and the other model can be adjusted based on the first model.

= Alignment of spatial channel model implementations

== Alignment criteria and thresholds

For SCM implementation alignment in PxSCH simulations, aim to have at least 3 contributions be within 2.5dB of SNR at the [70]% TPUT operating point, under random precoding. Encourage evaluation of full curves to decide the relative TPUT operating point for alignment.

For SCM implementation alignment in PMI simulations, aim to have at least 3 contributions be within [3.5]dB of SNR at the [TBD]% TPUT operating point under [type1 and/or eType2] follow precoding in 4Rx conditions. Encourage evaluation of full curves to decide the relative TPUT operating point for alignment.

== Simulations alignment subset

Repeat the simplified simulations subset/spreadsheet exercise for SU cases from RAN4#114bis.

= Comparison of spatial channel model candidates and legacy

== Legacy channel configuration

Comparison to cover the most commonly used legacy channel models, for example TDLC 300-10/100 with low and medium correlation. Alternatively, TDLA 30-10 med and high correlation.

== PDSCH fixed PMI selection for comparison

RAN4 to allow for independently chosen fixed PMI.

RAN4 to choose the most common PMI from a histogram of signaled/chosen follow PMIs as fixed precoder in SU PDSCH SCM evaluations at about [95]% TPUT operating point.

== AAV choice and number of Tx for comparison

Include the first comparisons with both AAV_3 and AAV_1Y, before downselecting to a single CDL candidate in the study.

Include 32Tx in the first comparisons, before downselecting to a single CDL candidate in the study.

== Comparison method and TR conclusions

To compare channel models, have each contributor set performance or channel property expectations for each comparison case. Then, each contributor compares their observations from simulations against their expectations, to subjectively decide whether channel models fit expectations. Context on importance of expectations for RAN4 minimum performance requirements is also given. See block diagram for simpler reading.

The TR conclusion to capture comparison results by collecting and aggregating the feedback given by all contributors. Additionally, it may be attempted to make a general statement on the necessity of SCMs in performance requirements setting and to select one specific SCM candidate in particular.

== Performance and channel properties

=== Performance wise comparison

Compare the per CW TPUT vs. SNR curves of an 8T/8R/8L PDSCH setup with random and follow precoding, given different channel model candidates, by collecting feedback from the group on what TPUT vs. SNR scaling would be expected in deployment and how well the channel model candidates match expectations.

Compare TPUT vs. SNR curves of an 4T4R4L PDSCH setup with random precoding and [3] & [30]kph mobility given different channel model candidates, by collecting feedback from the group on what TPUT vs. SNR vs. mobility scaling would be expected in deployment and how well the channel model candidates match expectations.

Compare TPUT vs. SNR curves of an 4T4R2+2L MU PDSCH setup with random (but not overlapping) precoding and linear receiver with and without inter-user IRC and R-ML receiver, given different channel model candidates, by collecting feedback from the group on what TPUT vs. SNR vs. receiver implementation scaling would be expected in deployment and how well the channel model candidates match expectations.

Compare SNR operating point levels of an 4T4R4L PDSCH setup given different channel model candidates. Contributors to state what operating points would be expected in deployment, as well as are feasible for test chambers, and how well the channel model candidates match expectations.

Discuss additions to the performance comparison use cases, e.g., with PMI cases using follow PMI of several CBs.

=== Channel properties wise comparison

Compare channel properties in terms of at least DoD and DoA direction, powers, diversity, and stability, as well as post-EQ SINR histograms and convergence times of channel candidates vs. each other, and vs. measurements.
This is done by collecting feedback from the group on expectations from deployment or measurements, as well as feedback on how well the channel model candidates match expectations.

== Priorities for comparison cases

If a prioritization of comparison cases is required, then we shall follow the order (1) SU PDSCH 2CW, (2) MU PDSCH IRC vs. eIRC vs. R-ML, (3) SU PMI 4L eTypeII vs. TypeI follow PMI (4) MU PMI 2+2L eTypeII vs. TypeI follow PMI.

= Potential optional requirements

The study TR shall contain a recommendation that any SCM requirements in 5GNR are recommended to be optional via optional UE capability/BS declaration and corresponding applicability rules.

- References

- R4-2504756, Way Forward for [114][322] NR_SCM, Moderator (Nokia), RAN4#114bis.

- R4-2506173, Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#115.

- R4-1816488, Way forward on NR UE PDSCH demodulation requirements and General aspects, Intel Corporation, RAN4#89.

- R4-1904713, WF on requirement SNR derivation procedure, Nokia, Nokia Shanghai Bell, ZTE, RAN4#90-Bis.

- R4-1903383, Summary of FR1 TDD CSI simulation results, Samsung, RAN4#90-Bis.

- R4-2502378, Way Forward for [114][322] NR_SCM, Moderator (Nokia), RAN4#114.

- R4-2418043, Discussion on Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#113.

- R4-2500427, Discussion on Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#114.

- R4-2411557, Scenarios and Requirements for Spatial Channel Modelling, BT plc, RAN4#112.

- R4-2419338, Further observations on the properties of MIMO channels in typical deployments, BT plc, Ericsson, RAN4#113.