3GPP TSG-RAN WG4 Meeting #115	R4-2506174

# St. Julian’s, Malta, 19 – 23 May 2025

Agenda item:		5.2

Source: 		Qualcomm Inc.

Title: 		On completing band 254 requirements

Document for:		Approval

## 1	Introduction

Band 254, also known as LS-band, was introduced in rel-18 and RAN4 RF work was closed in December 2023. The specification still has some TBD, and CRs were brought in previous meeting to address the TBDs. This contribution discussed the missing requirements and how to complete the requirements.

## 2	Discussion

When the RF core work for IoT NTN LS-band was recommended to be closed, a WF was agreed at the same meeting [1]

It can be seen that leaving A-MPR as TBD has been intentional. This explains why there has not been many A-MPR evaluations available.

Observation 1: Leaving TBDs for the NS_04N and NS_05N has been intentional.

Initial proposal for NS_04N and NS_05N A-MPR was made in [2] and [3]. Firstly, it can be noted that the proposed A-MPR does not seem to take into account results provided in [4]. Figures 2 and 3 are taken from [4] and show clear A-MPR need for single tone as well as full 12-tone allocation.

Figure 2: Single tone results in [4]

Figure 3: 12-tone results in [4]

Observation 2: A-MPR proposals in [2] and [3] do not consider results provided in [4]

So far we did not find any results for 3.75 kHz single-tone transmission. It might be even worse than 15 kHz single-tone, as all power is concentrated closer to the channel edge.

Observation 3: Evaluations for 3.75 kHz single-tone transmission have not been presented earlier.

Finally some further measurement input was provided in [5] showing high A-MPR values.

Given the differences in earlier A-MPR results, measurement and simulation analysis was done to check needed A-MPR. While there are no commonly agreed calibration points for NB-IoT, PA 0dB MPR point was approximately 1 dB higher than achieved for NR DFT-s-OFDM signal, with post-PA loss applied to set output power at measurement point to 23 dBm with MPR = 0 dB. Waveforms used met the general SEM, spurious and ACLR requirements with using only MPR, guaranteeing that the emission performance is representative of a device meeting 3GPP minimum requirements.

Transceiver impairments were aligned with minimum requirements, with -25 dBc IQ-image and DC-leakage and CIM3 applied with -60 dBc level.

Analysis was performed for NS_04N and NS_05N, both for 15 kHz and 3.75 kHz SCS. The results are summarized in Table 1 to Table 4.

Table 1: A-MPR for NS_04N, 15 kHz SCS

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 8 | 9 | 10 | 11 |
| A-MPR (dB) | ≤11 |  | ≤5.5 | ≤3.5 | ≤2 | ≤0.5 | ≤0.5 | ≤2 |  | ≤3.5 | ≤5.5 | ≤ 11 |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  | 9-11 |  |  |  |  |
| A-MPR (dB) | ≤ 8.5 |  |  |  | ≤ 2.5 dB |  |  |  | ≤ 8.5 |  |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 5.5 |  |  |  |  |  | ≤ 5.5 |  |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 2 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2: A-MPR for NS_04N, 3.75 kHz SCS

| Modulation | QPSK |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for 3.75 kHz single tone allocation | 0-3 | 4-6 | 6-9 | 10-38 | 39-41 | 42-44 | 45-47 |
| A-MPR (dB) | ≤ 7.5 | ≤ 3 | ≤ 0.5 | ≤ 0 | ≤ 0.5 | ≤ 3 | ≤ 7.5 |

Table 3: A-MPR for NS_05N, 15 kHz SCS

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 8 | 9 | 10 | 11 |
| A-MPR (dB) | ≤ 14 |  | ≤7 | ≤5 | ≤3.5 | ≤2 | ≤2 | ≤3.5 |  | ≤5 | ≤7 | ≤ 14 |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  | 9-11 |  |  |  |  |
| A-MPR (dB) | ≤ 10 |  |  |  | ≤ 4 |  |  |  | ≤ 10 |  |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 8 |  |  |  |  |  | ≤ 8 |  |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 3.5 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4: A-MPR for NS_05N, 3.75 kHz SCS

| Modulation | QPSK |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for 3.75 kHz single tone allocation | 0-3 | 4-6 | 6-10 | 11-37 | 38-41 | 42-44 | 45-47 |
| A-MPR (dB) | 10.5 | 5.5 | 2.5 | 0 | 2.5 | 5.5 | 10.5 |

It is proposed to use the A-MPR specified in Table 1 to Table 4 for NS_04N and NS_05N for NB-IoT. However, there is also an alternative approach, called nominated bandwidth, which can help reduce or remove the need for A-MPR.

Proposal 1: A-MPR in Table 1 to Table 4 is used for NS_04N and NS_05N for PC3, if large enough nominated bandwidth is not specified.

The requirements for NS_04N and NS_05N are originating from ETSI EN 301 441. Specifically, the frequency offsets in Table 4 and Table 5 of EN 301 441 are defined from “the nearest edge of the nominated bandwidth of the nominal carrier closest to the MSS system operating in another operational band within the band 1 610 MHz to 1 626,5 MHz. The frequency offset is measured in the direction of the adjacent MSS system”.

EN 301 441 also gives guidance for definition of nominated bandwidth:

Annex B gives further guidance on the maximum bandwidth

This means that 3GPP can define a nominated bandwidth potentially up to 1 MHz, and the emission requirements would start to apply outside this nominated bandwidth.

Observation 1: ETSI EN 301 441 allows a nominated bandwidth which for NB-IoT could be up to 1 MHz.

The existence of nominated bandwidth concept may make 3GPP emission requirements and A-MPR requirements meaningless: If UE is in reality type approved with different emission requirements (different nominated bandwidth) the value of the 3GPP requirements is limited.

Therefore, we propose to take the nominated bandwidth into use also in 3GPP to more closely align the 3GPP requirements with the conditions used in type approval. There is a need to choose a single value to set minimum requirements, so that all UEs are subject to same requirements and cannot freely declare any nominated bandwidth in 3GPP.

Proposal 2: Nominated bandwidth is taken into use in 3GPP for NS_04N and NS_05N requirements to align 3GPP requirements with bandwidth definitions used for certification. A single value is chosen to set minimum requirements.

Any extension of channel bandwidth will have impact on the adjacent channel position. However, a few 10s of kHz extension impact is not any less than full 100 kHz extension. Therefore, we suggest to use 400 kHz nominated BW.

Proposal 3: Use nominated bandwidth of 400 kHz.

We did a second set of A-MPR evaluations using nominated bandwidth of 400 kHz, i.e. applying the NS_04N and NS_05N emission requirements at 100 kHz larger frequency offset. With the added offset no A-MPR was required.

Proposal 4: No A-MPR is defined for NS_04N and NS_05N with nominated bandwidth of 400 kHz applied to wanted signal.

The emission requirements itself should be adjusted to be valid outside nominated bandwidth of 400 kHz.

Proposal 5: Adjust NS_04N and NS_05N additional in-band emission masks to start to apply +/- 200 kHz from carrier frequency, i.e. outside nominated bandwidth of 400 kHz.

Finally, to ensure that the nominated bandwidth is well understood, it would be useful to add an Annex to TS 36.102. It should be noted that the NTN IoT TDD work item is also discussing about this. [6]

Proposal 6: Add a new Annex in TS 36.102 defining and describing the nominated bandwidth concept, similarly as in 301 441.

## 3	Conclusion

In this contribution completing the requirements for band 254 were discussed. Following observations and proposals were made.

Observation 1: Leaving TBDs for the NS_04N and NS_05N has been intentional.

Observation 2: A-MPR proposals in [2] and [3] do not consider results provided in [4]

Observation 3: Evaluations for 3.75 kHz single-tone transmission have not been presented earlier.

Proposal 1: A-MPR in Table 1 to Table 4 is used for NS_04N and NS_05N for PC3, if large enough nominated bandwidth is not specified.

Observation 1: ETSI EN 301 441 allows a nominated bandwidth which for NB-IoT could be up to 1 MHz.

Proposal 2: Nominated bandwidth is taken into use in 3GPP for NS_04N and NS_05N requirements to align 3GPP requirements with bandwidth definitions used for certification. A single value is chosen to set minimum requirements.

Proposal 3: Use nominated bandwidth of 400 kHz.

Proposal 4: No A-MPR is defined for NS_04N and NS_05N with nominated bandwidth of 400 kHz applied to wanted signal.

Proposal 5: Adjust NS_04N and NS_05N additional in-band emission masks to start to apply +/- 200 kHz from carrier frequency.

Proposal 6: Add a new Annex in TS 36.102 defining and describing the nominated bandwidth concept, similarly as in 301 441.

## 4	References

- R4-2321817, WF on new FDD band (L+S band) for IoT NTN, MediaTek Inc.

- R4-2411543, (IoT_NTN_FDD_LS_band) CR to 36.102 for IoT-NTN UE RF Multi-tones A-MPR requirements (Rel-18), MediaTek Inc.

- R4-2411544, (IoT_NTN_FDD_LS_band) CR to 36.102 for IoT-NTN UE RF 1-tone A-MPR requirements (Rel-18)

- R4-2321798, Analysis on ETSI emission requirement for NB-IoT in IoT NTN band b254, Sony

- R4-2501153, AMPR value for NB-IoT in IoT NTN band b254, Sony

- R4-2506164, Nominated BW use for 249 and freq error, Qualcomm Incorporated