# 3GPP TSG RAN WG4 Meeting#115	R4-2506176

# St. Julians, Malta, May 19 – May 23, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for RedCap and eRedCap UEs over NR-NTN

Agenda item:	7.31.7

Document for:	Discussion

# 1. Introduction

The requirements for RedCap and eRedCap devices were introduced in Rel-17 and Rel-18 for terrestrial networks, respectively. Compared to IoT-NTN, supporting RedCap devices in the FR1 band while operating over NR-NTN networks may offer enhanced service capabilities. This ensures wideband/broadband coverage while maintaining low device complexity. To this end, a WI was agreed to be studied in RAN4 [1]. Furthermore, RAN4 will begin discussing the demodulation requirements starting from RAN#115. This paper presents our perspectives on how to approach the demodulation requirements for this WI.

2. Demodulation Requirements for RedCap UEs

In Rel-17, both FD-FDD and HD-FDD RedCap UEs were considered. It was agreed to apply the same demodulation requirements to both FD-FDD and HD-FDD RedCap UEs, with the only difference being the PDSCH allocation pattern in the test setup. Additionally, for RedCap UEs in Rel-17 for terrestrial networks, RAN4 defined tests for PDSCH, PDCCH, SDR, PMI, and CQI. It is important to note that the RTT in NTN transmission could be quite large, which may result in CSI feedback not accurately representing the channel condition due to the significant propagation delay by the time it is applied for DL transmission.

Proposal 1: Do not define CSI reporting requirements for RedCap UEs over NR-NTN.

Proposal 2: Do not define PBCH and PDCCH requirements for RedCap UEs over NR-NTN.

Proposal 3: Do not define SDR requirements for RedCap UEs over NR-NTN.

Proposal 4: Don’t introduce separate requirements for HD-FDD RedCap UEs.

Additionally, RedCap UE in Rel-17 can be equipped with either 1Rx or 2Rx chains, with 1Rx support being optional. Conversely, NR NTN requirements were defined with 2Rx only. To maintain alignment with NR-NTN, we propose introducing PDSCH requirements only for 2Rx.

Proposal 5: Discuss the feasibility of defining PDSCH demodulation requirements for RedCap UEs over NR-NTN with 2Rx only.

3. Demodulation Requirements for eRedCap UEs

For eRedCap UEs in Rel-18, PDSCH requirements were introduced for both 1Rx and 2Rx UEs. The CSI reporting requirements from Rel-17 RedCap were reused for Rel-18 eRedCap UEs, meaning only CQI and PMI reporting requirements were introduced with new FRCs. Additionally, there were no PDCCH and PBCH requirements introduced for eRedCap UEs.

As with RedCap UEs, we believe that CSI reporting requirements may not be useful for eRedCap UEs over NR-NTN. Furthermore, given that there were no PDCCH and PBCH requirements for eRedCap UEs in Rel-18, we think it is also not essential to have these requirements for eRedCap UEs over NR-NTN in Rel-19. Similar to RedCap UEs, to maintain alignment with NR-NTN, we propose introducing PDSCH requirements only for 2Rx.

Proposal 6: Do not define CSI reporting requirements for eRedCap UEs over NR-NTN.

Proposal 7: Do not define PBCH and PDCCH requirements for eRedCap UEs over NR-NTN.

Proposal 8: Discuss the feasibility of defining PDSCH requirements for eRedCap UEs over NR-NTN with 2Rx only.

4. Conclusions

In this paper, we present our perspectives on the demodulation performance requirements for RedCap and eRedCap UEs over NR-NTN. The following proposals have been made.

Proposal 1: Do not define CSI reporting requirements for RedCap UEs over NR-NTN.

Proposal 2: Do not define PBCH and PDCCH requirements for RedCap UEs over NR-NTN.

Proposal 3: Do not define SDR requirements for RedCap UEs over NR-NTN.

Proposal 4: Don’t introduce separate requirements for HD-FDD RedCap UEs.

Proposal 5: Discuss the feasibility of defining PDSCH demodulation requirements for RedCap UEs over NR-NTN with 2Rx only.

Proposal 6: Do not define CSI reporting requirements for eRedCap UEs over NR-NTN.

Proposal 7: Do not define PBCH and PDCCH requirements for eRedCap UEs over NR-NTN.

Proposal 8: Discuss the feasibility of defining PDSCH requirements for eRedCap UEs over NR-NTN with 2Rx only.

5. References

[1] RP-240775, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3,” 3GPP TSG RAN WG Meeting #103, Maastricht, NL, Mar 18 – 22, 2025.