3GPP TSG-RAN WG4 Meeting#115	R4-2506197

# Malta, MT, 19 – 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101.1: time masks for DL-only CA via switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LBCA_Sw-Core |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce DL and UL masks for low-band DL CA via switching. The switching gap is configured by RRC (number of symbols blanked) in terms of number of symbols blanked by the network or dropped by the UE, while the actual location of the switch is specified by time masks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.3.3.2: a time mask for UL slots with configured switching gaps is introduced (for other slots the general mask applies) Clause 7.11 (new): time masks for the DL introduced with the actual swtiching gaps with timing advance and receive time difference introduced. Two alternatives for the SDL to FDD switch introduced pending a decision on whether symbols (as indicated by RRC) may be dropped by the UE or blanked by the network (TBD). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The location of actual switch in the DL is unspecified. The position of the transient periods for UL slots configured with switching gaps unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.2, 7.11 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

#### 6.3.3.2	General ON/OFF time mask

The general ON/OFF time mask defines the observation period between transmit OFF and ON power and between transmit ON and OFF power for each SCS. ON/OFF scenarios include: contiguous, and non-contiguous transmission, etc

The OFF power measurement period is defined in a duration of at least one slot excluding any transient periods. The ON power is defined as the mean power over one slot excluding any transient period.

Figure 6.3.3.2-1: General ON/OFF time mask for NR UL transmission in FR1

For DL-only inter-band CA configurations supported via switching subject to [capability], the time mask for non-CA NR UL transmission in slots configured with switching gaps by RRC is specified in Figure 6.3.3.2.2-2. For other UL slots the general time mask in Figure 6.3.3.2-1 applies.

Figure 6.3.3.2-2: ON/OFF time mask for NR UL transmission for DL CA via switching with non-CA in the UL

#### 6.3.3.3	Transmit power time mask for slot and short or long subslot boundaries

< text omitted >

### 7.10A.2	Minimum requirement

For the test parameters in Table 7.10A.2-1, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1).

Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. NOTE 5: Void. |  |  |  |  |

For a UE capable of intraBandNR-CA-non-collocated-r18 for the following CA band combinations in Table 7.10A.2-2, the Power imbalance requirements are applicable with 2Rx antenna ports for each component carrier if it is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2.

Table 7.10A.2-2: NR CA combinations

CA combination

CA_n77(2A)1

CA_n78(2A)1

NOTE: All the requirements for intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by UL-DL-configuration-common in the PCell and SCells for NR SA

## 7.11		Switching of downlink carriers

## 7.11A		DL CA via switching

## 7.11A.1	General

In this sub-clause additional requirements are specified for DL-only inter-band CA configurations supported via switching subject to [capability]. The uplink is configured in one band.

## 7.11A.2	Time mask for switching between two downlink carriers

For an inter-band combination of two bands with one DL/UL carrier 1 configured in a paired band and a DL-only carrier 2 configured in another band and supported via switching between the two DL carriers, the time masks for switching between the DL carriers are specified in Figure 7.11A.2-1 and Figure 7.11A.2-2. For the DL/UL carrier 1 full-duplex communication (FDD) is supported, whereas for the DL-only carrier 2 in the other band neither full-duplex communication nor overlapping UL transmissions are supported (non-simultaneous Rx-Tx). Simultaneous reception on any overlapping symbols of DL carrier 1 and carrier 2 is not supported and the [MRTD equals the CP length].

For switching between DL carrier 1 and DL carrier 2 shown in Figure 7.11A.2-1, the switching period of a duration indicated by [capability] is fully contained within the time period between the end of the reception on the switch-from DL carrier 1 and the first symbol of the slot on the switch-to DL carrier 2. There is no transmission scheduled or configured in any direction in the blanked symbols, which fully contain a switching gap of a duration greater than the switching period.

For switching between DL carrier 2 and DL carrier 1 shown in Figure 7.11A.2-2,

< Alt 1: blanked symbols >

the switching period of a duration indicated by [capability] is fully contained within the time period between the end of the transmission on the switch-from DL carrier 2 and the first symbol of the slot on the switch-to UL carrier 1. There is no transmission scheduled or configured in any direction in the blanked symbols, which fully contain a switching gap of a duration greater than the switching period.

< end Alt 1 >

< Alt 2: dropped symbols >

the switching period of a duration indicated by [capability] occurs before the first symbol of the slot on the switch-to UL carrier 1. The UE may drop symbols transmitted on the switch-from DL carrier 2 to accommodate a switching gap of a duration greater than the switching period as specified in [38.214].

< end Alt 2 >

[There is no transmission on any carrier during the switching gaps.] Switching between the DL carriers is triggered as specified in [38.214].

Figure 7.11A.2-1: Time mask for switching between DL carrier 1 and DL Carrier 2

Figure 7.11A.2-2: Time mask for switching between DL carrier 2 and DL Carrier 1

< end of changes >