3GPP TSG-RAN4 Meeting #115	R4-2506201

Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	UE RF requirements of LB carrier aggregation

Agenda item:	7.7.3

Document for:	Discussion

Introduction

In this paper, we provide our views on UE RF requirement for low band CA based on WD [1].

Discussion

The followings were approved in WF [1].

Issue 1: Switching period capability - The switching period capability can be defined per band pair per band combination. Issue 2: Switching period location - Await outcome of RAN1 design Issue 3: Time mask for switching - Define the Tx on-off time mask requirements - FFS on how to define the Rx switching requirements to reflect where the switching period occurs - RAN4 has understanding that RAN1 will take switching period and timing advance into account for RAN1 scheduling pattern for low band CA via switching Issue 4: Considerations related to the Tx on-off time mask - Option 1: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL) - Option 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier - Other options are not precluded Issue 5: Test methodology - Companies are encouraged to contribute proposals related to the testability of the requirement on the time mask for switching

Figure 6.3.3.2-1 shows the existing general On/Off time mask for NR UL transmission.

Figure 6.3.3.2-1: General ON/OFF time mask for NR UL transmission in FR1

In the requirement, Start of On power and End of On power are aligned with slot boundary.

However, for LB CA switching, Start of On power and End of On power may not be aligned with slot boundary. Figure 2-1 shows one example of UE RF architecture supporting the LB CA switching.

Figure 2-1: One example of UE RF architecture supporting LB CA switching

In the example of figure 2-1, we can some switches for switching from FDD(n5)  to SDL(n29) or from SDL(n29) to FDD(n5).

Table 2-1 summaries relationship between SWs and Tx/Rx switching.

Table 2-1: Relationship between SWs vs Tx/Rx for LB CW

|  | SW1 | SW2 | SW3 | SW4 |
| --- | --- | --- | --- | --- |
| Tx On/Off | O | X | X | X |
| Rx Switching | O | O | O | O |

As seen in Table 2-1, SW1 is related to both Tx On/Off and Rx switching simultaneously.  It means Tx On/Off time mask needs to be specified considering Rx switching together.

RAN1 agreement is as follows.

For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact

Figure 2-2 shows Tx On/Off time mask for LB CA switching between FDD and SDL considering the RAN1 agreement.

- (a) : Tx OffOn time mask for switching from SDL to FDD

- (b) : Tx OnOff time mask for switching from FDD to SDL when switching period location is configured in FDD slot

- (c) : Tx OnOff time mask for switching from FDD to SDL when switching period location is configured in SDL slot

Here, carrier switching period is 35us and 140us which were agreed in RAN4#114 meeting.

Figure 2-2: Tx On/Off time mask for LB CA switching between FDD and SDL

Figure 2-3 shows Tx On/Off time mask for LB CA switching between FDD and SDL without considering TA offset from figure 2-2.

Figure 2-3: Tx On/Off time mask for LB CA switching between FDD and SDL

Frome the analysis, we propose as follows.

Proposal 1: Define Tx On/Off time mask for LB CA switching between FDD and SDL considering carrier switching period together.

Proposal 2: Define Tx On/Off time mask for LB CA switching between FDD and SDL as Figure2-2 or Figure2-3.

For Rx switching requirements to reflect where the switching period of 35us and 140us occurs, we think that RAN4 does not need to define it because no time mask for Rx switching between two DL carriers has been specified. The related requirements can be defined as RAN1 scheduling pattern.

Proposal 3: RAN4 does not define Rx switching requirements to reflect where the switching period occurs. It can be defined as RAN1 scheduling pattern.

Conclusion

In this contribution, we proposed our view on UE RF requirement for low band CA.

Proposal 1: Define Tx On/Off time mask for LB CA switching between FDD and SDL considering carrier switching period together.

Proposal 2: Define Tx On/Off time mask for LB CA switching between FDD and SDL as Figure2-2 or Figure2-3.

Proposal 3: RAN4 does not define Rx switching requirements to reflect where the switching period occurs. It can be defined as RAN1 scheduling pattern.

- References

[1] R4-2505223, “WF on UE RF requirements for LB CA via switching”, Apple

[2] RP-243315, “New WID on low band carrier aggregation via switching”, Apple, TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE