3GPP TSG-RAN WG4 Meeting #115	R4-2506207

Malta, MT, 19 May 2025 - 23 May 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM performance requirements for network energy saving

Agenda item:	7.27.4

Document for:	Discussion

- Introduction

According to work plan, performance aspects will be discussed from this meeting. In this contribution we will discuss the test cases for the respective features for Rel-19 network energy saving [1] based on latest agreements in [2] [3].

- OD-SSB SCell operation

Scenarios

For OD-SSB SCell operation, RAN1 has identified different cases and scenarios as below, and made further down scoping or restriction on the details. Up to now, we understood the following cases and scenarios are agreed by RAN1 in this release:

- Case #1: No always-on SSB on the cell

  - Scenario #2: OD-SSB is activated “before UE receives SCell activation command”

  - Scenario #2A: OD-SSB is activated “when UE receives SCell activation command”

  - Scenario #3B: OD-SSB is activated “when SCell activation is completed and SCell is activated” or “after SCell activation is completed and SCell is activated”

- Case #2: Always-on SSB is periodically transmitted on the cell

  - Scenario #2: OD-SSB is activated “before UE receives SCell activation command”

  - Scenario #2A: OD-SSB is activated “when UE receives SCell activation command”

  - Scenario #3B: OD-SSB is activated “when SCell activation is completed and SCell is activated” or“After SCell activation is completed and SCell is activated”

In Case #1, since there is no always-on SSB on the cell, OD-SSB is at least needed in order to activate the SCell. Hence the first OD-SSB activation shall happen no later than when UE receives SCell activation command. Based on the latest RAN1 agreements, scenario #3B was also agreed but we understood this does not refer to the first OD-SSB activation as RAN1 agreed the OD-SSB transmission will not be deactivated after SCell is activated. This means we can assume OD-SSB is always activated after UE receives SCell activation command. In summary, for Case 1 test cases design, we should consider scenario #2 and scenario #2A while assuming OD-SSB is always activated after UE receives SCell activation command.

Observation 1: In Case 1, the first OD-SSB activation shall happen no later than when UE receives SCell activation command in order to activate the SCell.

Proposal #1: For Case 1, RAN4 should define the test cases considering scenario #2 and scenario #2A while assuming OD-SSB is always activated after UE receives SCell activation command.

In Case #2, there is always-on SSB on the cell hence OD-SSB is not a must for activating the SCell. Then all the scenarios listed above shall be considered.

Proposal #2: For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B regarding to when the OD-SSB is activated.

RRM procedures to be tested

On OD-SSB operation, RAN4 has been focusing on the discussion on deactivated SCell measurement and SCell activation. Besides, more impact on interruption, L1 measurement etc. are also discussed. In our view, we should at least define the test cases for the following procedures:

  - OD-SSB based intra-frequency measurement (deactivated SCell)

  - OD-SSB based SCell activation delay, including all the agreed SCell activation cases

  - Interruption at OD-SSB activation/deactivation

Proposal #3: RAN4 to define the test cases at least for the following procedures:

  - OD-SSB based intra-frequency measurement (deactivated SCell)

  - OD-SSB based SCell activation delay, including all the agreed SCell activation cases

  - Interruption at OD-SSB activation/deactivation

- OD-SIB1

While the impact on RRM requirements due to OD-SIB1 is still ongoing, we believe the test cases for cell reselection shall be defined due to the additional delay caused by UL WUS transmission.

In TS38.133 the test cases are specified in TS 38.133 A6.1.1 to verify the UE’s cell reselection behavior. In the test cases, the cell reselection delay is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 2.

TS 38.133:

A.6.1.1.1.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 2.

If UE is performing cell reselection to an OD-SIB1 cell, the cell reselection delay must additionally consider the time for requesting OD-SIB1. Hence the required time for cell reselection in test requirement needs to be extended. A new test case is needed to verify the cell reselection behaviour to an OD-SIB1 cell.

Observation 2: If UE is performing cell reselection to an OD-SIB1 cell, the cell reselection delay must additionally consider the time for requesting OD-SIB1.

Proposal #4: RAN4 to specify a new test case to verify the cell reselection behaviour to an OD-SIB1 cell.

- SSB adaptation

Scenarios and RRM procedures to be tested

The scenario for SSB adaptation for activated SCell has been defined by RAN1 based on the group common DCI indication 2_9.

Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Further clarification on the scope of RRM impact for SSB adaption: - Existing SSB burst periodicity (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - SSB adaptation in SCell for NCD-SSB. - Time domain positions of SSBs within a burst is same before and after SSB adaptation - Note: Above could be further updated based on further RAN1 progress if any.

Hence test cases for activated SCell should be defined for DCI based signalling for L1 measurement and L3 measurement for serving SCell and the SSB activation delay should be considered, i.e. the time UE is ready to receive the adjusted SSB occasion / SMTC occasion, whereby change to lower SSB periodicity as well as to higher SSB periodicity can be performed in one test case.

Proposal #5: 	RAN4 to discuss test cases for activated SCell for DCI based signalling for L1 measurement and L3 measurement for serving SCell and to consider the SSB activation delay, i.e. the time UE is ready to receive the adjusted SSB occasion / SMTC occasion. whereby change to lower SSB periodicity as well as to higher SSB periodicity can be performed in one test case.

In addition, RRC signalling is foreseen to be used if SCell activation and SSB adaptation command are received in the same message. The discussion on SCell activation requirements is still ongoing based on WF from RAN4 #114bis, see extract below.

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements Agreement: - Normal SCell activation - Direct SCell activation - PUCCH SCell activation - FFS Rel-18 SCell activation with L3 reporting - FFS Multiple SCells activation

In this case test cases for the above SCell activation requirements, ensuring that the UE follows the expected timeline for receiving the adjusting SSB transmission, should be defined. In our view, at least normal and direct SCell activation should be considered.

Proposal #6: 	RAN4 to discuss test cases for agreed SCell activation scenarios with RRC based signalling to ensure that UE follows the expected timeline for receiving the adjusted SSB transmission.

Proposal #7: 	RAN4 to define test cases at least for normal and direct SCell activation.

Whether test cases are needed for deactivated SCell measurements, depends on the UE behaviour which is under discussion based on WF from RAN4 #114bis, see extract below.

Issue 1-3-1: L3 requirements for SSB adaptation Agreement: - RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation. - For neighbour cell L3 measurement, legacy requirements apply. Open issue for further discuss: - For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects: - Whether to define L3 measurement requirement at transition. - Any other RRM requirements impacts due to additional SMTC configurations.

If legacy requirements apply, i.e. using measCycleSCell (from Rel-15) or NCSG (from Rel-17), then no new requirements are needed, otherwise this may be FFS.

Proposal #8: 	RAN4 to not define test cases for deactivated SCell measurements, unless a new UE measurement behaviour is introduced in Rel-19.

- Conclusion

In this contribution, we discussed the RRM performance requirements for Rel-19 network energy saving.

The following observations and proposals are made.

OD-SSB SCell operation

Observation 1: In Case 1, the first OD-SSB activation shall happen no later than when UE receives SCell activation command in order to activate the SCell.

Proposal #1: For Case 1, RAN4 should define the test cases considering scenario #2 and scenario #2A while assuming OD-SSB is always activated after UE receives SCell activation command.

Proposal #2: For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B regarding to when the OD-SSB is activated.

Proposal #3: RAN4 to define the test cases at least for the following procedures:

  - OD-SSB based intra-frequency measurement (deactivated SCell)

  - OD-SSB based SCell activation delay, including all the agreed SCell activation cases

  - Interruption at OD-SSB activation/deactivation

OD-SIB1

Observation 2: If UE is performing cell reselection to an OD-SIB1 cell, the cell reselection delay must additionally consider the time for requesting OD-SIB1.

Proposal #4: RAN4 to specify a new test case to verify the cell reselection behaviour to an OD-SIB1 cell.

SSB adaptation

Proposal #5: 	RAN4 to discuss test cases for activated SCell for DCI based signalling for L1 measurement and L3 measurement for serving SCell and to consider the SSB activation delay, i.e. the time UE is ready to receive the adjusted SSB occasion / SMTC occasion. whereby change to lower SSB periodicity as well as to higher SSB periodicity can be performed in one test case.

Proposal #6: 	RAN4 to discuss test cases for agreed SCell activation scenarios with RRC based signalling to ensure that UE follows the expected timeline for receiving the adjusted SSB transmission.

Proposal #7: 	RAN4 to define test cases at last for normal and direct SCell activation.

Proposal #8: 	RAN4 to not define test cases for deactivated SCell measurements, unless a new UE measurement behaviour is introduced in Rel-19.

- References

[1] 	RP-250343, “Revised WID: Enhancements of network energy savings for NR”, Ericsson

[2]	R4-2504906, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson

[3]	R4-2504907, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei