3GPP TSG-RAN WG4 Meeting #115	R4-2506208

# Malta, MT, 19 May 2025 - 23 May 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for UE to UE CLI handling

Agenda item:	7.25.4.1

Document for:	Discussion

1. Introduction

In the last RAN4 meeting, many agreements were made in the WF [1]. We will continue discussing the open issues on L1 UE-to-UE CLI measurement in this contribution.

2. L1-SRS-RSRP measurement

  - Scheduling restriction

In the last RAN4 meeting, the following agreement was reached regarding the scheduling restrictions for L1-SRS-RSRP measurement:

- Agreement (online session on Monday with update) - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for SRS resource and Y symbols before, where Y is re-using same number as R16 requirements. - For the symbols after the SRS resource, not define scheduling restriction on symbols after symbols for SRS resource in TS 38.133, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211. - For DL, - if UE is able to support FDM-ed DL reception and SRS measurement, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, otherwise (if UE does not support the capability), scheduling restriction applies. - The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements. - FFS whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband.

It is left open if restricted symbols are needed after L1-SRS-RSRP measurement resource for DL scheduling due to the switching between UL subband and DL subband. In our opinion, we do not foresee any need for introducing additional scheduling restrictions. One could think about switching from UL subband to DL subband as a UL-to-DL switch in TDD, which has been covered by RAN1 specification and doesn’t require any scheduling restrictions. Thus, we propose the following:

No need to introduce DL scheduling restrictions to cover the switching between UL subband to DL subband after measuring L1-SRS-RSRP.

  - Measurement Restriction

On measurement restriction, the following agreements were reached. It is open how to handle the CLI measurement if the measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping.

- Agreements - Measurement requirements do not apply for L1-SRS-RSRP when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options - Option 1: Prioritize L1-SRS-RSRP measurement - Option 2: Prioritize L1-CLI-RSSI measurement - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI

In our view, the L1-SRS-RSRP measurement should be prioritized over L1-CLI-RSSI measurement if they are partly or fully overlapping. The reason is that we expect L1-SRS-RSRP to provide more knowledge about the CLI conditions of the victim UE than the L1-CLI-RSSI measurement. Moreover, the configuration of a L1-SRS-RSRP requires the transmission of SRS from an aggressor UE, which might end up wasted if the L1-SRS-RSRP is not measured. We prefer not to support Option 3 since it creates ambiguity in the NW about what the content of the received measurement report. Therefore, we prefer to support Option 1.

When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partly or fully overlapping, the UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI and shall prioritize L1-SRS-RSRP measurement.

3. L1-CLI-RSSI measurement

  - Scheduling restriction

The following agreements/conclusions related to measurement resources were reached by RAN4:

#### Issue 1-3-2: Scheduling restriction #### Agreements (adhoc session on Tuesday with addition) #### Method #1: #### For UL, #### Scheduling restrictions always apply, for both FR1 and FR2. #### The restricted symbols include symbols for RSSI resource. #### Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 #### For DL, #### Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource #### Method #3: #### For UL, #### Scheduling restrictions always apply, for both FR1 and FR2. #### The restricted symbols include symbols for RSSI resource. #### Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 #### For DL, #### Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource. #### Note: the common understanding in RAN4 if the RSSI in UL suband is too strong compare to the wanted signals in DL suband(s), UE may fail to decode the PDCCH or PDSCH. #### FFS whether restricted symbol(s) is needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband. #### Generic assumption for defining RRM requirements, not intend to add it in the core requirement. #### For all Method #1, Method #2 and Method #3, #### Note: The RF BW is wide enough to cover DL active BWP, including PRBs from both DL subband(s) and UL subband that fall within the DL active BWP.

The open issue for the scheduling restrictions is whether restricted symbol(s) are needed after the L1-CLI-RSSI measurement resource to cover the switching between UL subband and DL subband when the CLI-RSSI measurements are performed according to Method#3. As already discussed for the L1-SRS-RSRP scheduling restrictions, we do not see the need to introducing any restricted symbols for the switching between UL subband to DL subband since it could be seen as a transition from UL to DL on a legacy TDD system, which doesn’t require any additional guard period for switching.

No need to introduce DL scheduling restrictions to cover the switching between UL subband to DL subband after measuring L1-CLI-RSSI according to Method#3.

  - Measurement restrictions

#### Issue 1-3-3: Measurement restriction - Agreements (adhoc session on Tuesday) - Collision between L1-CLI-RSSI measurements with different methods - Capture in RAN4 specs that UE is not expected to perform CLI measurement using Method #1 and Methods#3 in the same OFDM symbol. - Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements - For Method #1& Method 3: - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR2, FFS for FR1.

There is a pending FFS on whether we should define measurement restrictions for Method#1 and Method#3 L1-CLI-RSSI measurements when colliding with L1/L3 RRM measurements for FR1. In our view, we should define the same measurement restrictions as for FR2, i.e., measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements party or fully overlap in time.

Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements partly or fully overlap in time for both FR1 and FR2.

  - Side condition

Regarding the SCS assumptions for the L1-CLI-RSSI measurements in Method#1 and Method#3, we would like to discuss RAN1 agreements and confirm our understanding in RAN4.

In RAN1#120, it has been agreed that:

Agreement - UE determines the subcarrier spacing of the CLI-RSSI measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - UE determines the subcarrier spacing of the SRS-RSRP measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - Note: No RAN1 spec impact is expected. Agreement For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’ - carrier in CSI-ReportConfig is used to indicate in which serving cell the CLI-RSSI measurement resources or SRS-RSRP measurement resources in CSI-ResourceConfig are to be found. If the field is absent, the resources are on the same serving cell as this report configuration. - bwp-Id in the associated CSI-ResourceConfig is used to indicate the DL BWP where the CLI-RSSI measurement resources or SRS-RSRP measurement resources are located in. Note: No RAN1 specification impact Agreement - UE determines the subcarrier spacing of the CLI-RSSI measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - UE determines the subcarrier spacing of the SRS-RSRP measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - Note: No RAN1 spec impact is expected.

From the RAN1 agreements we make the following observations:

- L1-CLI-RSSI and L1-SRS-RSRP measurements can be configured to be performed in resources from:

- In the same carrier as the serving cell, within the active BWP or in another configured BWP

- In a different carrier than the serving cell.

- The subcarrier spacing to be used for the L1-CLI-RSSI and L1-SRS-RSRP measurements is based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig which can be the same or different from the subcarrier spacing of active BWP. From RAN4 point of view, our preference is that the UE shall perform L1-CLI-RSSI and L1-SRS-RSRP measurement assuming the SCS of the active BWP. Note that this assumption is the same as the Rel-16 L3 CLI measurement.

From RAN4 perspective, the UE shall perform L1-CLI-RSSI and L1-SRS-RSRP measurements assuming the SCS of the active DL BWP.

  - L1-CLI-RSSI report mapping

The reporting range of L1-CLI-RSSI is defined from -100 dBm to -25 dBm with 1 dB resolution. The mapping of measured quantity is defined in Table 10.1.22.2.2-1. One aspect to highlight is the fact that, for current L3 CLI-RSSI measurements, the UE is expected to scale the measurement CLI-RSSI to report a nominal RSSI equivalent to 6 RB measurement with 15 kHz SCS. We would like RAN4 to confirm if the same UE behaviour is expected now for L1 CLI-RSSI measurements. In our view, the scaling of the measured CLI-RSSI is needed to ensure that measurements are comparable and consistent across different measurement bandwidths.

RAN4 to discuss whether the UE shall report the measured L1-CLI-RSSI or a nominal L1-CLI-RSSI equivalent to a 6 RB measurement with 15 kHz SCS.

4. General

  - Requirement applicability

In R16 CLI measurements, the measurement requirements apply only when the measurement resource is fully confined within active BWP. While SBFD operation is considered in R19, the SRS-RSRP and CLI-RSSI in Method#3 will be configured in UL subband. However, the UL subband is also considered within DL active BWP. Hence the existing applicability conditions in R16 are still applicable to R19 L1 CLI measurement.

There was a proposal to clarify the L1-SRS-RSRP measurement resource is confined within UL subband of active BWP. This is true for SBFD operation, however RAN1 is still discussing whether L1 CLI measurements are also applicable for dynamic TDD operation. We don’t see any showstopper to configure L1 CLI measurement in dynamic TDD hence a general wording on the applicability is preferred. The existing applicability conditions in R16 CLI measurement can be reused for R19 L1 CLI measurement.

TS 38.133 clause 9.7.1:

CLI measurements are only applicable for RRC_CONNECTED intra-frequency:

-	when SRS-RSRP measurement resource is fully confined within BW of DL active BWP

-	when CLI-RSSI measurement resource is configured within active BWP

The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

-	when L1-SRS-RSRP measurement resource is fully confined within BW of DL active BWP

-	when L1-CLI-RSSI measurement resource is configured within active BWP

  - L1 UE-to-UE CLI measurements in non-SBFD symbols

There is an on-going discussion in RAN1 about whether a UE can be configured to measure L1 UE-to-UE CLI in non-SBFD symbols, as captured in the following text.

#### Proposal 2-5 Alt.1 The valid symbol type for CSI derivation configured in CSI-ReportConfig is not applicable for L1 UE-to-UE CLI measurement and reporting. - UE shall perform L1 UE-to-UE CLI measurement only in SBFD symbols Alt.2 The valid symbol type for CSI derivation configured in CSI-ReportConfig is not applicable for L1 UE-to-UE CLI measurement and reporting. - UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig Note: L1 CLI measurement is supported in both SBFD symbols and non-SBFD symbols

RAN4 to wait, and potentially take into consideration, RAN1 decision on whether a UE can be configured to perform L1 UE-to-UE CLI measurements in non-SBFD symbols in SBFD operation.

  - Coexistence of L1 and L3 UE-to-UE CLI measurements

As part of the measurement restrictions, it has been already agreed that L1-SRS-RSRP measurement requirements do not apply when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. Similar is expected to be agreed for L1-CLI-RSSI for both FR1 and FR2. However, it is still open what should be the behaviour when L1 UE-to-UE CLI measurements partly or fully overlap with L3 UE-to-UE CLI. One possible way to handle this issue is to prevent a UE from being configured with partly or fully overlapping L1 and L3 UE-to-UE CLI measurement resources. Alternatively, some prioritization should be specified, e.g., the UE prioritizes L1 UE-to-UE CLI over L3 UE-to-UE CLI if measurement resources are partly or fully overlapping.

RAN4 to discuss the case where L1 UE-to-UE CLI and L3-UE-to-UE CLI measurements are partly or fully overlapping.

5. Conclusion

The following Observations and Proposals were made:

- No need to introduce DL scheduling restrictions to cover the switching between UL subband to DL subband after measuring L1-SRS-RSRP.

- When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partly or fully overlapping, the UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI and shall prioritize L1-SRS-RSRP measurement.

- No need to introduce DL scheduling restrictions to cover the switching between UL subband to DL subband after measuring L1-CLI-RSSI according to Method#3.

- Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements partly or fully overlap in time for both FR1 and FR2.

- From RAN4 perspective, the UE shall perform L1-CLI-RSSI and L1-SRS-RSRP measurements assuming the SCS of the active DL BWP.

- RAN4 to discuss whether the UE shall report the measured L1-CLI-RSSI or a nominal L1-CLI-RSSI equivalent to a 6 RB measurement with 15 kHz SCS.

- The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

-	when L1-SRS-RSRP measurement resource is fully confined within BW of DL active BWP

-	when L1-CLI-RSSI measurement resource is configured within active BWP

RAN4 to wait, and potentially take into consideration, RAN1 decision on whether a UE can be configured to perform L1 UE-to-UE CLI measurements in non-SBFD symbols in SBFD operation.

RAN4 to discuss the case where L1 UE-to-UE CLI and L3-UE-to-UE CLI measurements are partly or fully overlapping.

References

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei