3GPP TSG-RAN4 Meeting #115 	R4-2506212

# Malta, MT, 19 May 2025 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR Introducting L1 CLI measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR introduces L1 CLI measurement performance requirements in TS 38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the accuracy requirements for L1-SRS-RSRP and L1-CLI-RSSI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are no performance requirements for L1 CLI measurements in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.X new clause, with title: L1 Cross Link Interference measurements accuracy requirements |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------------- start of change ------------------------------------------------

### 10.1.X L1 CLI measurement accuracy requirements

#### 10.1.X.1	L1-SRS-RSRP

##### 10.1.X.1.1	L1-SRS-RSRP Accuracy

The L1-SRS-RSRP measurement reported by the UE shall fulfil the accuracy requirements defined in table 10.1.X.1.1-1 for FR1 and table 10.1.X.1.1-2 for FR2, provided that the following conditions are met. The accuracy requirements in this clause are derived based on AWGN radio propagation conditions.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SRS-RSRP measurements are fulfilled according to annex [B.2.z] for a corresponding Band for each relevant SRS resource configured for measurement.

-	The time difference between UE’s DL reference timing in the serving cell and SRS arrival time is no larger than Terror_SRS_RSRP, where

-	Terror_SRS_RSRP = TC × NTA_offset + 1.67µs for FR1

-	Terror_SRS_RSRP = TC × NTA_offset + 0.67µs for FR2

-	NTA_offset is defined in table 7.1.2-2

-	TC is 0.509 ns

-	The number of SRS ports in the SRS resource configured for measurement is 1,

-	The number of symbols in the SRS resource configured for measurement is 1,

-	The number of repetitions in the SRS resource configured for measurement is 1,

-	Frequency hopping, sequence group hopping or sequence hopping is disabled in the SRS resource configured for measurement,

-	The bandwidth of the SRS resource is 24 PRBs.

-	One of the following conditions is met

-	There is no other SRS resource with the same root sequence and on the same symbol and with same comb as the relevant SRS resource.

-	If multiple SRS resources are on the same symbol and with same comb, the distance between cyclic shifts of any two resources is no less than 6 if transmissionComb = n4, and no less than 4 if transmissionComb = n2.

Table 10.1.X.1.1-1:  L1-SRS-RSRP absolute accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | SRS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCSSRS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCSSRS (kHz) |  |  | SCSSRS (kHz) |  |  |  |  |  |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| [3] | [3.5] | [5] | [7.5] | [8] | [9.5] | 1 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| [6.5] | [7] | [8.5 | [9.5 | [10] | [11.5] | 1 | NR_TDD_FR1_A, NR_TDD_FR1_C, NR_TDD_FR1_D, NR_TDD_FR1_E | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 10.1.X.1.1-2: L1-SRS-RSRP absolute accuracy in FR2

| Accuracy |  |  |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  | Extreme condition |  | SRS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB |  |  |  | dB | dBm / SCSSRS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
| SCSSRS (kHz) |  | SCSSRS (kHz) |  |  | SCSSRS = | SCSSRS = |  |  |
| 60 | 120 | 60 | 120 |  | 60 kHz | 120 kHz |  |  |
| [6] | [8.5] | [9] | [11.5] | ≥1 | Same value as SRS_RP in Table B.2.7-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| [9] | [11.5] | [11] | [13.5] | ≥1 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on REFSENS and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |  |

##### 10.1.X.1.2	L1-SRS-RSRP report mapping

The reporting range of L1-SRS-RSRP is defined from -140 dBm to -44 dBm with 1 dB resolution. The mapping of measured quantity is defined in table 10.1.X.1.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

The reporting range of differential SRS-RSRP for L1 reporting is defined from 0 dB to -30 dB with 2 dB resolution. The mapping of measured quantity is defined in table 10.1.X.1.2-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.X.1.2-1: L1-SRS-RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| L1-SRS-RSRP_0 | L1-SRS-RSRP<-140 | dBm |
| L1-SRS-RSRP_1 | -140≤ L1-SRS-RSRP<-139 | dBm |
| L1-SRS-RSRP_2 | -139≤ L1-SRS-RSRP<-138 | dBm |
| L1-SRS-RSRP_3 | -138≤ L1-SRS-RSRP<-137 | dBm |
| L1-SRS-RSRP_4 | -137≤ L1-SRS-RSRP<-136 | dBm |
| .. | .. | … |
| L1-SRS-RSRP_95 | -46≤ L1-SRS-RSRP<-45 | dBm |
| L1-SRS-RSRP_96 | -45≤ L1-SRS-RSRP<-44 | dBm |
| L1-SRS-RSRP_97 | -44≤ L1-SRS-RSRP | dBm |
| L1-SRS-RSRP_98 | Infinity |  |
| NOTE: ‘Infinity’ means that UE cannot detect SRS due to too strong signal to measure. |  |  |  |

Table 10.1.X.1.2-2: Differential SRS-RSRP measurement (for L1 reporting) report mapping

| Reported value | Measured quantity value (difference in measured RSRP from strongest RSRP) | Unit |
| --- | --- | --- |
| DIFFSRSRSRP_0 | 0≥ΔRSRP>-2 | dB |
| DIFFSRSRSRP_1 | -2≥ΔRSRP>-4 | dB |
| DIFFSRSRSRP_2 | -4≥ΔRSRP>-6 | dB |
| DIFFSRSRSRP_3 | -6≥ΔRSRP>-8 | dB |
| DIFFSRSRSRP_4 | -8≥ΔRSRP>-10 | dB |
| DIFFSRSRSRP_5 | -10≥ΔRSRP>-12 | dB |
| DIFFSRSRSRP_6 | -12≥ΔRSRP>-14 | dB |
| DIFFSRSRSRP_7 | -14≥ΔRSRP>-16 | dB |
| DIFFSRSRSRP_8 | -16≥ΔRSRP>-18 | dB |
| DIFFSRSRSRP_9 | -18≥ΔRSRP>-20 | dB |
| DIFFSRSRSRP_10 | -20≥ΔRSRP>-22 | dB |
| DIFFSRSRSRP_11 | -22≥ΔRSRP>-24 | dB |
| DIFFSRSRSRP_12 | -24≥ΔRSRP>-26 | dB |
| DIFFSRSRSRP_13 | -26≥ΔRSRP>-28 | dB |
| DIFFSRSRSRP_14 | -28≥ΔRSRP>-30 | dB |
| DIFFSRSRSRP_15 | -30≥ΔRSRP | dB |

#### 10.1.X.2	L1-CLI-RSSI

##### 10.1.X.2.1	L1-CLI-RSSI Accuracy

The L1-CLI-RSSI measurement reported by the UE shall fulfil the accuracy requirements defined in table 10.1.X.2.1-1 for FR1 and table 10.1.X.2.1-2 for FR2, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

Table 10.1.X.2.1-1: L1-CLI-RSSI absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSRS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| [3.5] | [6.5] | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| 5.5] | [8.5] | Note 3 | Note 3 | Note 3 | Note 3 | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |

Table 10.1.X.2.1-2: L1-CLI-RSSI absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |
|  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dBm / SCSSRS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  | SCSSRS = 60 kHz | SCSSRS = 120 kHz |  |  |
| [5] | [8] | Same value as SRS_RP in Table B.2.7-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| [7] | [10] | Note 4 |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on REFSENS and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |

##### 10.1.X.2.2	L1-CLI-RSSI report mapping

The reporting range of L1-CLI-RSSI is defined from -100 dBm to -25 dBm with 1 dB resolution. The mapping of measured quantity is defined in table 10.1.X.2.2-1. The range in the signalling may be larger than the guaranteed accuracy range. UE shall scale the measured L1-CLI-RSSI to report a nominal RSSI equivalent to 6RB measurement with 15 kHz SCS.

The reporting range of differential L1-CLI-RSSI reporting is defined from 0 dB to -30 dB with 2 dB resolution. The mapping of measured quantity is defined in table 10.1.X.2.2-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.X.2.2-1: L1-CLI-RSSI measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| L1-CLI-RSSI_00 | L1-CLI-RSSI < -100 | dBm |
| L1-CLI-RSSI_01 | -100 L1-CLI-RSSI < -99 | dBm |
| L1-CLI-RSSI_02 | -99 L1-CLI-RSSI < -98 | dBm |
| … | … | … |
| L1-CLI-RSSI_74 | -27 L1-CLI-RSSI < -26 | dBm |
| L1-CLI-RSSI_75 | -26 L1-CLI-RSSI < -25 | dBm |
| L1-CLI-RSSI_76 | -25 L1-CLI-RSSI | dBm |

Table 10.1.X.2.2-2: Differential CLI-RSSI measurement (for L1 reporting) report mapping

| Reported value | Measured quantity value (difference in measured RSSI from strongest RSSI) | Unit |
| --- | --- | --- |
| DIFFCLIRSSI_0 | 0≥ΔCLI-RSSI>-2 | dB |
| DIFFCLIRSSI_1 | -2≥ΔCLI-RSSI>-4 | dB |
| DIFFCLIRSSI_2 | -4≥ΔCLI-RSSI>-6 | dB |
| DIFFCLIRSSI_3 | -6≥ΔCLI-RSSI>-8 | dB |
| DIFFCLIRSSI_4 | -8≥ΔCLI-RSSI>-10 | dB |
| DIFFCLIRSSI_5 | -10≥ΔCLI-RSSI>-12 | dB |
| DIFFCLIRSSI_6 | -12≥ΔCLI-RSSI>-14 | dB |
| DIFFCLIRSSI_7 | -14≥ΔCLI-RSSI>-16 | dB |
| DIFFCLIRSSI_8 | -16≥ΔCLI-RSSI>-18 | dB |
| DIFFCLIRSSI_9 | -18≥ΔCLI-RSSI>-20 | dB |
| DIFFCLIRSSI_10 | -20≥ΔCLI-RSSI>-22 | dB |
| DIFFCLIRSSI_11 | -22≥ΔCLI-RSSI>-24 | dB |
| DIFFCLIRSSI_12 | -24≥ΔCLI-RSSI>-26 | dB |
| DIFFCLIRSSI_13 | -26≥ΔCLI-RSSI>-28 | dB |
| DIFFCLIRSSI_14 | -28≥ΔCLI-RSSI>-30 | dB |
| DIFFCLIRSSI_15 | -30≥ΔCLI-RSSI | dB |

-------------------------------------------------- end of change ------------------------------------------------