3GPP TSG-RAN WG4 Meeting #115	R4-2506213

# Malta, MT, 19 May 2025 - 23 May 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation result for L1-CLI-SRS-RSRP measurement

Agenda item:	7.23.3.1

Document for:	Discussion

- Introduction

In RAN4#112-bis, 3GPP agreed on simulation assumptions for evaluating the L1-SRS-RSRP measurement requirements [1]. In this paper we provide a sub-set of our link-level simulation results. The full dataset can be found in the excel spreadsheet companion to this document.

2. Link-level simulation results

  - Case 1: Inter-cell measurements

    - FR1

Figure 1. Inter-cell delta L1-SRS-RSRP for AWGN channel in FR1 (SRS SNR = 1 dB)

Table 1. Summary of the inter-cell L1-SRS-RSRP measurements for AWGN in FR1

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | AWGN | FR1 | 24 | 15 kHz | 1 | 1 | -0.61 | 0.53 |
| Case 1 | AWGN | FR1 | 48 | 15 kHz | 1 | 1 | -0.42 | 0.41 |
| Case 1 | AWGN | FR1 | 24 | 30 kHz | 1 | 1 | -1.19 | 0.08 |
| Case 1 | AWGN | FR1 | 48 | 30 kHz | 1 | 1 | -0.99 | -0.1 |
| Case 1 | AWGN | FR1 | 24 | 60 kHz | 1 | 1 | -2.41 | -0.97 |
| Case 1 | AWGN | FR1 | 48 | 60 kHz | 1 | 1 | -2.19 | -1.17 |

Figure 2. Inter-cell delta L1-SRS-RSRP for TDL-C channel in FR1 (SRS SNR = 1 dB)

Table 2. Summary of the inter-cell L1-SRS-RSRP measurements for TDL-C in FR1

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | TDL-C | FR1 | 24 | 15 kHz | 1 | 1 | -0.72 | 0.69 |
| Case 1 | TDL-C | FR1 | 48 | 15 kHz | 1 | 1 | -0.5 | 0.44 |
| Case 1 | TDL-C | FR1 | 24 | 30 kHz | 1 | 1 | -1.36 | 0.18 |
| Case 1 | TDL-C | FR1 | 48 | 30 kHz | 1 | 1 | -1.1 | -0.01 |
| Case 1 | TDL-C | FR1 | 24 | 60 kHz | 1 | 1 | -2.59 | -0.86 |
| Case 1 | TDL-C | FR1 | 48 | 60 kHz | 1 | 1 | -2.36 | -1.16 |

    - FR2

Figure 3. Inter-cell delta L1-SRS-RSRP for AWGN channel in FR2 (SRS SNR = 1 dB)

Table 3. Summary of the inter-cell L1-SRS-RSRP measurements for AWGN in FR2

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | AWGN | FR2 | 24 | 60 kHz | 1 | 1 | -1.82 | -0.49 |
| Case 1 | AWGN | FR2 | 48 | 60 kHz | 1 | 1 | -1.64 | -0.68 |
| Case 1 | AWGN | FR2 | 24 | 120 kHz | 1 | 1 | -4.59 | -2.63 |
| Case 1 | AWGN | FR2 | 48 | 120 kHz | 1 | 1 | -4.3 | -2.93 |

Figure 4. Inter-cell delta L1-SRS-RSRP for TDLC channel in FR2 (SRS SNR = 1 dB)

Table 4. Summary of the inter-cell L1-SRS-RSRP measurements for TDL-C in FR2

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | TDL-C | FR2 | 24 | 60 kHz | 1 | 1 | -2.23 | -0.47 |
| Case 1 | TDL-C | FR2 | 48 | 60 kHz | 1 | 1 | -1.95 | -0.82 |
| Case 1 | TDL-C | FR2 | 24 | 120 kHz | 1 | 1 | -5.83 | -2.84 |
| Case 1 | TDL-C | FR2 | 48 | 120 kHz | 1 | 1 | -5.37 | -3.09 |

  - Case 2: Intra-cell measurements

    - FR1

Figure 5. Intra-cell delta L1-SRS-RSRP for AWGN channel in FR1 (SRS SNR = 1 dB)

Table 5. Summary of the intra-cell L1-SRS-RSRP measurements for AWGN in FR1

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 2 | AWGN | FR1 | 24 | 15 kHz | 1 | 1 | -0.59 | 0.54 |
| Case 2 | AWGN | FR1 | 48 | 15 kHz | 1 | 1 | -0.42 | 0.38 |
| Case 2 | AWGN | FR1 | 24 | 30 kHz | 1 | 1 | -0.61 | 0.56 |
| Case 2 | AWGN | FR1 | 48 | 30 kHz | 1 | 1 | -0.41 | 0.39 |
| Case 2 | AWGN | FR1 | 24 | 60 kHz | 1 | 1 | -0.86 | 0.35 |
| Case 2 | AWGN | FR1 | 48 | 60 kHz | 1 | 1 | -0.66 | 0.16 |

Figure 6. Intra-cell delta L1-SRS-RSRP for TDL-C channel in FR1 (SRS SNR = 1 dB)

Table 6. Summary of the intra-cell L1-SRS-RSRP measurements for TDL-C channel in FR1

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 2 | TDL-C | FR1 | 24 | 15 kHz | 1 | 1 | -0.73 | 0.67 |
| Case 2 | TDL-C | FR1 | 48 | 15 kHz | 1 | 1 | -0.5 | 0.43 |
| Case 2 | TDL-C | FR1 | 24 | 30 kHz | 1 | 1 | -0.75 | 0.63 |
| Case 2 | TDL-C | FR1 | 48 | 30 kHz | 1 | 1 | -0.51 | 0.47 |
| Case 2 | TDL-C | FR1 | 24 | 60 kHz | 1 | 1 | -1.03 | 0.41 |
| Case 2 | TDL-C | FR1 | 48 | 60 kHz | 1 | 1 | -0.8 | 0.2 |

    - FR2

Figure 7. Intra-cell delta L1-SRS-RSRP for AWGN channel in FR2 (SRS SNR = 1 dB)

Table 7. Summary of the intra-cell L1-SRS-RSRP measurements for AWGN in FR2

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 2 | AWGN | FR2 | 24 | 60 kHz | 1 | 1 | -0.61 | 0.56 |
| Case 2 | AWGN | FR2 | 48 | 60 kHz | 1 | 1 | -0.42 | 0.41 |
| Case 2 | AWGN | FR2 | 24 | 120 kHz | 1 | 1 | -0.73 | 0.44 |
| Case 2 | AWGN | FR2 | 48 | 120 kHz | 1 | 1 | -0.51 | 0.3 |

Figure 8. Intra-cell delta L1-SRS-RSRP for TDL-C channel in FR2 (SRS SNR = 1 dB)

Table 8. Summary of the intra-cell L1-SRS-RSRP measurements in TDL-C FR2

| Case | Channel | FR | BW [PRB] | SCS [kHz] | SRS samples [#] | SRS SNR [dB] | 5th percentile of delta L1-SRS-RSRP [dB] | 95th percentile of delta L1-SRS-RSRP [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 2 | TDL-C | FR2 | 24 | 60 kHz | 1 | 1 | -0.8 | 0.56 |
| Case 2 | TDL-C | FR2 | 48 | 60 kHz | 1 | 1 | -0.63 | 0.32 |
| Case 2 | TDL-C | FR2 | 24 | 120 kHz | 1 | 1 | -1.3 | 0.13 |
| Case 2 | TDL-C | FR2 | 48 | 120 kHz | 1 | 1 | -1.06 | -0.08 |

References

- R4-2416910, “Simulation assumption for evaluate L1-SRS-RSRP measurement requirements”, Ericsson, RAN4#112-bis