3GPP TSG-RAN WG4 Meeting #114bis	R4-2506228

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.21.4

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM core requirement for beam management

Document for:	Discussion

- Introduction

During the past meetings, requirements for beam management has been discussed and several agreements were obtained. This paper further discusses remaining issues in the WF [1].

- Discussion

  - Beam prediction KPIs

During past meetings, the metrics/KPIs for beam prediction has been extensively discussed and agreement was obtained as follows.

Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

The remaining discussion points are N value and x value. The purpose to introduce beam ID prediction metric is to ensure the usability of predicted beam. The prediction includes error with high probability, and it results in not the best beam selection. Thus, the beam ID prediction metric should be introduced to achieve selecting best beam or “better” beam in high probability. The effect of N (>1) value is to consider the reliability of the prediction. If the UE can only predict the best beam with certain accuracy but the prediction of other beams are totally inaccurate, it is hard to say that the prediction is reliable. Figure 1 shows an example.

Figure1: Successful prediction with N = 1, but is it reliable?

In order to consider both accuracy and reliability, N = 1 is not the best. On the other hand, if N = K, it is too strict. Thus, N = 2 is better.

Observation 1: If N = 1, the prediction reliability is doubtful. If N = K, the requirement is too strict.

Proposal 1: N = 2 is better to ensure both reliability and usability.

In the case of N = 2, x cannot be zero. If N = 2 and x = 0, the successful case is limited to the case in which the ground truth of top 2 beams is the same value. In addition, to guarantee optimality of the selected beam, x should be less than 3.

Observation 2: In the case of N = 2, x cannot be zero. If N = 2 and x = 0, the successful case is limited to the case in which the ground truth of top 2 beams is the same value.

Proposal 2: x should be less than 3.

  - Measurement period for inference

During past meetings, agreement of measurement period for inference was obtained as follows.

Issue 2-4: Measurement period for inference Agreement: - For measurement delay component of the prediction delay: - Use “legacy” L1-RSRP measurement as the baseline for Case 1 - FFS whether same values for M,N,P should be reused - FFS for case 2

For case 2, since UE predicts Set A beam based on the measurement history of Set B beam, UE can predict the best beam in advance. Thus prediction delay is not needed.

Proposal 3: For case 2, prediction delay is not needed because UE predicts Set A beam based on the measurement history of Set B beam, UE can predict the best beam in advance.

- Conclusion

In this contribution, we proposed our views on beam prediction.

Observation 1: If N = 1, the prediction reliability is doubtful. If N = K, the requirement is too strict.

Proposal 1: N = 2 is better to ensure both reliability and usability.

Observation 2: In the case of N = 2, x cannot be zero. If N = 2 and x = 0, the successful case is limited to the case in which the ground truth of top 2 beams is the same value.

Proposal 2: x should be less than 3.

Proposal 3: For case 2, prediction delay is not needed because UE predicts Set A beam based on the measurement history of Set B beam, UE can predict the best beam in advance.

References

[1] 3GPP, R4-2505105, “WF on requirements for AI/ML air interface normative work”, Qualcomm Incorporated